3GPP TSG-RAN WG4 Meeting #111                                                     R4-2407798

# Fokuoka, Japan, May 20 –24, 2024

Source:		Nokia

Title:	Test methodology and radiated performance metric for FR1 NTN devices

Agenda item:		10.4.2.2

Document for:	Discussion

# Introduction

Based on the WF from RAN4 110-bis meeting [1], in this paper, we’d like to share our views of NR NTN OTA test related issues as listed below:

### Sub-topic 3-2 UE performance metric Issue 3-2-1: whether RAN4 should consider different performance metric for different UE types or satellite orbits Way forward: - RAN4 further discuss potential performance metric for different FR1 UE types. - FFS whether different performance metric for different UE type. FFS Handheld UE is prioritized. - FFS whether separate metrics for NR/IoT NTN UEs based on their support for either GSO or NGSO or both - FFS whether different performance metrics should be defined for high earth orbit (such as GEO) and low earth orbit (i.e. LEO) due to they have different requirements on the radiating characteristics Issue 3-2-2: Proper performance metric for NR-NTN Way forward: Consider the following initial input as a starting point for further discussions: - Option 1: consider the following aspects - Option 1: integrated power/sensitivity within declared half sphere - Option 2: peak EIRP/EIS only - Option 3: peak EIRP/EIS + X%-tile spherical coverage within declared half sphere - Option 4: peak EIRP/EIS + Y%-tile spherical coverage from whole sphere - Option 2: consider the following aspects - Consider a general framework for each UE type, to specify performance metric for different use cases / power classes, e.g., a set of metric for Handheld, and other set for FR1 VSAT-like UE. - Consider the assumption: UE elevation angles for NTN are typically >20°, in majority of cases. - Consider TRP, TRS, EIRP, and EIS as starting point. Further discuss other performance metric based on NTN usage scenarios, e.g., directivity requirements, Antenna Gain. - Option 3: consider the following aspects - consider WRP and WRS defined in R4-2404278 as performance metrics for NTN devices - consider EIRP and EIS CDF percentile thresholds as performance metrics for NTN devices - Option 4: Conical TRP/EIRP for Tx and conical TRS/EIS for Rx could be considered as the starting point for the metric of NTN device OTA testing. FFS on the range of q and f. - Option 5: Consider the following aspects for NTN performance metric - Adopting a directional antenna pattern as the starting point for the antenna characteristics of an FR1 NTN device. - Take UIRP and UHIS as the as the starting point for the performance metric of devices using directional antenna. - Further discuss whether a smaller angle of test scan can be used or whether a EIRP/EIS CDF-like performance metric and be considered. - Option 6: other aspects are not precluded

# Discussion

  - Whether RAN4 should consider different performance metric for different UE types or satellite orbits

In the current Rel18, NR NTN are separated into 5 types [2], they support the NTN communication for different satellite orbits.

- Define the requirements based on the following UE types

| UE class | UE type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT supporting GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT supporting GSO and LEO with electronical steering antenna. |
|  | 3 | Fixed VSAT supporting LEO only with electronical steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT supporting GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT supporting GSO with electronical steering antenna. |
| Note: Assuming that UE has single beam towards one single satellite at a given time. |  |  |

Fixed VSAT:

Electronic steering

Class 1: 2.5 dB NF  (LEO and GSO)

Class 2: 6 dB NF  (LEO only)

Mechanical steering

Class 1: 2.5 dB NF  (LEO and GSO)

Mobile VSAT:

Electronic steering

Class 1: 2.5 dB NF  (GSO)

Mechanical steering

Class 1: 2.5 dB NF  (GSO)

Whether to support of the GSO and LEO are clearly decided for each type. Therefore, no need to consider different performance metric for satellite orbits, consider the NTN UE class types instead.

Observation 1: in Rel18, 5 NTN UE types has been agreed and whether to support different satellite orbits are clearly decided.

Proposal 1: Follow the Rel18 5 NTN UE types, different satellite orbits are support by those UE types.

  - Performance metric for NR-NTN

In the current standard, the requirement for each UE types has been defined, such as EIPR, EIS, spherical coverage, and so on. It is reasonable to follow the tradition and include those into the performance metric for each NTN UE types.

Observation 2: consider EIPR, EIS, spherical coverage as performance metric for each NTN UE types based on current Rel18 standardization structure of UE requirements.

The NTN UE only communicate with satellites, the radiation pattern of the NTN UE antenna is supposed to point to the sky. It is not necessary to request the NTN UE antenna radiation pattern to point to all the directions of the 3-dimensional sphere. The half sphere antenna radiation pattern of a NTN device is reasonable.

Observation 3: The NTN UE only communicate with satellites in the sky, the half sphere antenna radiation pattern of a NTN device is reasonable.

The satellites in the sky are far away from the ground. For LEO, the altitude is from 160 km to 1,500 km, and for GEO, the altitude is about 35,768 km. When UE need to communicate to LEO or GEO, UE will need to form a beam to point to the direction of satellite for compensating the pass loss of a long distance. The distance between a UE and a satellite is much longer than the distance between a UE and a base station on the ground. Therefore, it is reasonable to consider the directivity and gain of the antenna radiation pattern in the performance metric of the NTN UE types.

Observation 4: due to the long distance between the UE on the ground and the satellites in the sky, UE will need to use beam to communicate with satellites. It is reasonable to consider the directivity and gain of the antenna radiation pattern in the performance metric of the NTN UE types.

Proposal 2: consider EIPR, EIS, spherical coverage, as well as directivity and gain of the antenna radiation pattern as performance metric for each NTN UE types, and only verify the performance metric for a UE declared half sphere.

# Conclusions

In this contribution, we make the following observations and conclusions:

Observation 1: in Rel18, 5 NTN UE types has been agreed and whether to support different satellite orbits are clearly decided.

Proposal 1: Follow the Rel18 5 NTN UE types, different satellite orbits are support by those UE types.

Observation 2: consider EIPR, EIS, spherical coverage as performance metric for each NTN UE types based on current Rel18 standardization structure of UE requirements.

Observation 3: The NTN UE only communicate with satellites in the sky, the half sphere antenna radiation pattern of a NTN device is reasonable.

Observation 4: due to the long distance between the UE on the ground and the satellites in the sky, UE will need to use beam to communicate with satellites. It is reasonable to consider the directivity and gain of the antenna radiation pattern in the performance metric of the NTN UE types.

Proposal 2: consider EIPR, EIS, spherical coverage, as well as directivity and gain of the antenna radiation pattern as performance metric for each NTN UE types, and only verify the performance metric for a UE declared half sphere.

# References

[1]	R4-2406086, Way Forward for [110bis][338] TRP_TRS_MIMO_OTA

[2] 	R4-2321974, “WF on NTN UE RF requirements”, ZTE, Samsung