# 3GPP TSG-RAN WG4 Meeting # 111                                    R4-2407801

# Fukuoka City, JP, 20 May 2024 – 24 May 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on NR repeater EMC performance criteria

# Agenda item:	4.3

# Document for:	Discussion

- Introduction

In R17 repeater discussion, the performance assessment method of NR repeater has decided to choose power accuracy. However the traditional metric for repeater is gain. This discussion paper will discuss the test level of these two metrics.

- Discussions

2.1 Comparison between LTE repeater and NR repeater

The description of LTE repeater performance criteria in TS36.113[1] has shown below:

## 6.5 Performance criteria for continuous phenomena for repeaters The gain of the EUT shall be measured throughout the period of exposure of the phenomenon. The gain measured during the test shall not change from the gain measured before the test by more than ± 1 dB. At the conclusion of the test the EUT shall operate as intended with no loss of user control functions or stored data. ## 6.6 Performance criteria for transient phenomena for repeaters The gain of the EUT shall be measured before the test and after each exposure. At the conclusion of each exposure the gain of the EUT shall not have changed by more than ± 1 dB. At the conclusion of the total test comprising the series of individual exposures, the EUT shall operate as intended with no loss of user control functions or stored data, as specified by the manufacturer, and the gain of the EUT shall not have changed by more than ± 1 dB.

Observation 1: The performance criteria for LTE repeater is ± 1 dB of gain.

The description of NR repeater performance criteria in TS38.114[2] has shown below:

## 6.1 Performance criteria for continuous phenomena for NR repeaters The power accuracy of the EUT shall be measured throughout the period of exposure of the phenomenon. For repeater type 1-C, the measured output power Pmax,p,AC during the test shall not change from the rated passband output power Prated,p,AC measured before the test by more than ± 2 dB. For repeater type 2-O, the maximum passband TRP output power Pmax,p,TRP during the test shall not change from the rated passband TRP output power Prated,p,TRP measured before the test by more than ± 3 dB. At the conclusion of the test the EUT shall operate as intended with no loss of user control functions or stored data. ## 6.2 Performance criteria for transient phenomena for NR repeaters The power accuracy of the EUT shall be measured before the test and after each exposure. For repeater type 1-C, the measured output power Pmax,p,AC after each exposure and after the total test shall not change from the rated passband output power Prated,p,AC measured before the test by more than ± 2 dB. For repeater type 2-O, the maximum passband TRP output power Pmax,p,TRP after each exposure and after the total test shall not change from the rated passband TRP output power Prated,p,TRP measured before the test by more than ± 3 dB. At the conclusion of the total test comprising the series of individual exposures, the EUT shall operate as intended with no loss of user control functions or stored data.

Observation 2: The performance criteria for NR repeater is ± 2 dB for repeater type 1-C and ± 3 dB for repeater type 2-O.

However the test level of 1dB gain and 2/3 dB power accuracy does not share the same. 1dB of change in gain will correspond to 1dB change of power accuracy. This is shown in the equation below:

∆Gain=Gain2 − Gain1=10log10P2Pin − 10log10P1Pin=10log10P2P1

∆P(dB)=P2(dB)−P1(dB)=10log10P2−10log10P1=10log10P2P1

Where:

P1 = Output power before EMC test.

P2 = Output power after EMC test.

Gain1 = Gain before EMC test.

Gain2 = Gain after EMC test.

Observation 3: X dB change in gain will equal to X dB change in power accuracy.

Therefore according to the equations above, the X dB change in gain will equal to X dB change in power accuracy. In order to share the same test level as LTE repeater, we recommend to improve the power accuracy test level of NR repeater from ± 2dB/± 3dB to ± 1dB, which is shown in CR R4-2407855 and R4-2407856.

Proposal 1: Edit the performance criteria of NR repeater, by improving the power accuracy test level from ± 2dB/± 3dB to ± 1dB.

- Conclusion

In this paper, we the performance criteria of NR repeater and compared it with LTE repeater. The summary is listed below:

Observation 1: The performance criteria for LTE repeater is ± 1 dB of gain.

Observation 2: The performance criteria for NR repeater is ± 2 dB for repeater type 1-C and ± 3 dB for repeater type 2-O.

Observation 3: X dB change in gain will equal to X dB change in power accuracy.

Proposal 1: Edit the performance criteria of NR repeater, by improving the power accuracy test level from ± 2dB/± 3dB to ± 1dB.

- Reference

- TS36.113

- TS38.114

- R4-2407855

- R4-2407856