3GPP TSG-RAN WG4 Meeting # 111	R4-2407847

# Fukuoka, Japan, May 20 – May 24, 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for CSI compression

Agenda Item:	10.11.4

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability and interoperability issues for CSI compression.

- Discussion

      - 2.1 Reference decoder definition

Issue 4-2: Reference and test encoder/decoder Test decoder definition: Test decoder(for UE side test): the decoder to be used in RAN4 tests and implemented in TE. it will be captured in the specifications if necessary (for example, for Option 3 it would be explicitly captured in the specifications) test decoder definition covers both Option 3 and Option 4 Companies are invited to bring proposals to clarify the meaning/definition of the reference encoder/decoder used in the RAN4 discussions.

For option 3, the test decoder will be specified in terms of model structure and parameter. While for option 4, it agrees that reference model will be partially specified. There is no clear definition what is partially specified for a model. A model can consist of input/out dataset and model structure/parameter itself. From our understanding, input/output of model, i.e. dataset, can also be part of reference model. Furthermore, it’s also possible to only specify dataset format and dataset can be exchanged. If reference model of option 4 is specified in terms of input/output dataset or dataset format while reference model of option 3 is specified in terms of model structure/parameter, then the reference decoder definition is different from option 3 and option 4. Therefore, RAN4 needs to discuss what is partially specified for a model in option 4. Then we can conclude whether the definition is same or not between option 3 and option 4.

Observation 1: For option 4, RAN4 agrees that reference model will be partially specified. However, there is no clear definition what is partially specified for a model.

Observation 2: Input/output of model, i.e. dataset or dataset format, can also be part of reference model.

Proposal 1: Before discussing whether the reference model definition is the same for both option 3 and option 4, RAN4 needs to discuss what is partially specified for a reference model for option 4 first.

      - 2.2 RAN4 – RAN1 Coordination

Issue 4-4: RAN4 – RAN1 Coordination RAN4 continues to discuss Option 3 and 4 and can review/discuss RAN1 agreements on interoperability if there will be any - Proposals - Option 1: Further alignment is needed between RAN4 and RAN1 - which parameters to align? - Option 2: Discussion in RAN4 should continue, RAN4 should determine testing feasibility of Option 3/4 - Option 3: Option 1 and Option 2 - Option 4: others

Currently, both RAN1 and RAN4 are discussing interoperability about CSI compression. The candidate options in RAN1 is as below:

RAN1 #116 Agreement To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options: - Option 1: Fully standardized reference model (structure + parameters) - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc. Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

Option 1 in RAN1 is quite similar as Option 3 in RAN4, i.e. detail model structure and parameter will be specified.

Option 2~5 in RAN1 can be classified into option 4 in RAN4. There is no clear definition what is partially specified for a model in RN4. From our understanding, a model can be described by model structure/parameter itself. Besides, model can also be described by the input/output dataset of model.

Proposal 2: RAN1 is also discussing interoperation issue, options in RAN1 can be classified into options in RAN4:

- Option 1 in RAN1 is quite similar as Option 3 in RAN4

- Option 2~5 in RAN1 can be classified into option 4 in RAN4

In the next, we will discuss how to define reference model in option 3/4 in RAN4 respectively.

      - 2.3 Reference model for option 3 in RAN4

Issue 4-3: Option 3 split Common understanding: RAN4 is currently discussing the feasibility of deriving a test decoder, discussion/decision on whether multiple decoders would be needed for different tests can happen later if needed. Issue 4-3: Option 3 split - Proposals for parameters that should be agreed upon: - Option 1: - Option 2: Decision to split should be made later (in an eventual WI phase) based on feasibility - it will depend on whether it is feasible to have a “one fits all” decoder or whether it has to be scenario specific - Option 3: others - Option 3a: The test decoder is determined for each test case, using a specific dataset collecting from the configuration/scenario of the considered test case. - Option 3b: The test decoder is determined for more than one test cases, using a mixed dataset collecting from different configurations/scenarios of the considered test cases.

For option 3, the target is to fully standardize the inference encoder (including architecture and parameters).

Since there are many models and parameters from different UE and NW vendors, it’s challenge to align all the parameters. Besides, if several reference models need to be specified for different scenarios, the specification effort is quite high.

Observation 3: For option 3, it’s challenge to align all the parameters. Besides, if several reference models need to be aligned for different scenarios, the specification effort is quite high.

RAN1 also has some discussions about fully specified reference model(option 1 in RAN1) with the following conclusion:

RAN1 #116bis Conclusion: - Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors). - It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4. Observation - Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed - Option 1 and 2 may require high specification effort from RAN1 perspective. Conclusion - Deprioritize Option 2 for inter-vendor training collaboration. Note: This deprioritization shall not affect the ongoing discussion in RAN4 on RAN4-Option3 and RAN4-Option4.

The above conclusion shows that RAN1 also identified the high specification effort and limited performance in the field compared with other options.

Observation 4: From RAN1 conclusion, fully specified model has limited performance in the field compared to other options. It also requires high specification effort from RAN1 perspective.

We suggest to take RAN1’s observation into consideration. If RAN4 really would like to specify a reference model, RAN4 also needs to discuss how to define a rule to choose a model structure and decide the detail model parameter. There are many aspects needs to be considered:

- Performance

- Complexity

- Interoperability and compatibility

- other

here, we would like to emphasize the importance of interoperability and compatibility. The CSI reference decoder needs to be compatible with different CSI encoders from UE vendor to guarantee the performance. Otherwise, the reference decoder will only work for dedicated encoder structure, which will have great limitation on UE implementation.

Proposal 3: For option 3, RAN4 to take RAN1’s observation into consideration.

      - 2.4 Reference model for option 4 in RAN4

For option 4 in RAN4, it’s more flexible compared with option 3 and has less implementation limitation. According to current RAN1 discussion, there are several options for interoperation:

Options in RAN1: - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side - The above options may not be mutually exclusive and may be used together.

Since option 2 has already been deprioritized in RAN1, the candidate will be option 3,4,5 or combination.

For option 3 in RAN1, we still have some concerns about the specification effort. If parameter is exchanged, it means that the parameter number is also fixed, e.g. model layer, how many neurons in each layer. Company still needs to align the detail of the model. Compared with option 1, the main difference is that parameter can change between different scenarios. The performance gain will be better. However, the model alignment work is still heavy.

For option 5 in RAN1, it’s also similar as option 1/2 in RAN4. Reference model will be exchanged between NW and UE. Due to privacy issue, RAN4 has only limited discussion about the option.

Option 4 requires less effort in reaching consensus when compared with other options and also provides implementation flexibility. We suggest RAN4 to discuss test feasibility of option 4 in RAN1.

Proposal 4: For option 4, RAN4 to discuss test feasibility issue for:

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: For option 4, RAN4 agrees that reference model will be partially specified. However, there is no clear definition what is partially specified for a model.

Observation 2: Input/output of model, i.e. dataset or dataset format, can also be part of reference model.

Proposal 1: Before discussing whether the reference model definition is the same for both option 3 and option 4, RAN4 needs to discuss what is partially specified for a reference model for option 4 first.

Proposal 2: RAN1 is also discussing interoperation issue, options in RAN1 can be classified into options in RAN4:

- Option 1 in RAN1 is quite similar as Option 3 in RAN4

- Option 5 in RAN1 is some kind of option 1/2 in RAN4

- Option 2~4 in RAN1 can be classified into option 4 in RAN4

Observation 3: For option 3, it’s challenge to align all the parameters. Besides, if several reference models need to be aligned for different scenarios, the specification effort is quite high.

Observation 4: From RAN1 conclusion, fully specified model has limited performance in the field compared to other options. It also requires high specification effort from RAN1 perspective.

Proposal 3: For option 3, RAN4 to take RAN1’s observation into consideration.

Proposal 4: For option 4, RAN4 to discuss test feasibility issue for:

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface