3GPP TSG-RAN WG4 Meeting #111	R4-2407871

Fukuoka City, Japan, 20th – 24th May, 2024

Agenda Item:	7.24.1

Source:	OPPO

Title:	Discussion on SSB-less SCell operation for intra-band non-contiguous CA

Document for:	Decision

Introduction

This paper will discuss the remaining issue related to intra-band non-contiguous CA with SSB-less operation.

Discussion

Issue 1-1-5: Intra-band non-contiguous CA

Background

RAN4#110 R4-2403526 Issue 1-3-1: Intra-band NCCA Agreement: Regarding FR1 intra-band NCCA with SSB-less operation, further study whether we can reuse or not the SSB-less activation delay requirement specified for FR1 inter-band CA. The study does not impact the completion timeline of the WI.

- Proposals - Option 1 For FR1 intra-band non-contiguous CA, reuse the SSB-less SCell activation delay requirement defined for FR1 collocated inter-band CA with the same RTD side condition i.e. RTD within CP (Nokia, Intel) - Option 1a: Introduce the optional with capability signalling with per FS granularity for UE supporting intra-band NCCA SSB-less SCell operation in Rel-18. (Intel) - Option 2: The delay requirements of SSB-less SCell activation for intra-band non-contiguous CA can be same as that for legacy intra-band contiguous CA provided the UE impelmentation architecture are same for the two scenarios. The exact side conditions for intra-band non-contiguous CA can be further discussed. (ZTE) - Option 3: RAN4 will not consider SSB-less SCell operation for intra-band NCCA in R18. (QC) - Option 4: (Apple, Huawei) - For FR1 intra-band NCCA with SSB-less operation, EPRE difference at UE side shall be NOT larger than 6dB - For FR1 intra-band NCCA with SSB-less operation, RTD between the target SCell and the intra-band NCCA collocated reference serving cell can be within CP - Option 5: (CMCC) - Define two sets of requirement and side condition, reuse the intra-band contiguous CA case and inter-band CA case respectively. - Define two UE capabilities for intra-band NCCA scenario, which corresponding to two set of requirements respectively, the granularity could be per FS indication - Recommended WF: - Discuss above issue in this meeting.

Per the reasons mentioned below, we feel reluctant to extend SSB-less SCell activation requirements of inter-band CA to the case of intra-band non-contiguous CA (NCCA), and it is also not applicable to all UE supporting NCCA to reuse the requirements of intra-band contiguous CA case. Accordingly, further discussion in RAN4 and RAN2 would be avoided (e.g. on how to capture the related UE capability for intra-band SSB-less SCell activation).

- The discussion is clearly out of the scope of WID.

In the WID, the SSB-less operation focuses on the case of inter-band CA.

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

- The assumption at the UE side would be different between inter-band and intra-band cases.

For SSB-less SCell inter-band CA, the basic assumption is the UE has two separate RX chains. Therefore, independent AGC, T/F tracking was possible for the inter-band CA scenario. While, for the intra-band CA, two separate RX chains may not always apply based on the current discussion status in RAN4. Thus, it may be hard to directly reuse the conclusions for inter-band SSB-less SCell activation to intra-band SSB-less SCell activation.

Thus, we propose not to consider to support SSB-less SCell operation for intra-band NCCA in R18.

Not consider SSB-less SCell operation for intra-band NCCA in R18.

Conclusion

We have the following proposals:

- Not consider SSB-less SCell operation for intra-band NCCA in R18.

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR

[3] R4-2406301 WF on RRM requirements for NR network energy saving