3GPP TSG-RAN WG4 Meeting #111	R4-2407879

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	7.12.2.2

Source: 	OPPO

Title: 	Discussion on performance requirements for SL positioning

Document for:	Approval

1	Introduction

As per the conclusions reached in previous meeting [1], this contribution will provide our further considerations on the performance requirements for SL positioning.

2	Discussion

#### Issue 3-1-1: SINR side conditions Agreements: - SL RSTD: - Option 1: (0, -6) dB for reference and another anchor UE - Option 2: (-3, -6) dB for reference and another anchor UE - SL UE Rx-Tx and SL RSRP(P): - FFS -6 dB, - FFS -3 dB - SL AoA and SL RTOA (for core requirements only): - -6 dB

Generally, SL coverage is smaller than Uu coverage and so a better SINR side condition (>-6dB or >0dB) is considered for defining PSBCH-RSRP or L1 SL-RSRP accuracy requirements. For SL RSTD, two SINR side conditions can be used for reference UE and target UE respectively, and option 1 [0, -6]dB is slightly preferred by us. For the other types of SL PRS measurements, a single SINR side condition [-6Db] is preferred considering the quite close simulation results.

Proposal 1: The side condition is [0, -6]dB for SL RSTD and [-6]dB for the other SL PRS measurements.

#### Issue 3-1-4: Measurement accuracy requirements Agreements: - RAN4 to define accuracy requirement for SL RSTD and SL Rx-Tx by considering the framework of ±(X+Y[+Z]) Tc and ±(X[+]) Tc, respectively. - X is the simulated measurement accuracy for a given propagation condition and number of measurement samples, - Y is the frequency/clock drift margin, - FFS: Z and are the RF calibration margins.

When defining accuracy requirements for SL RSTD and SL Rx-Tx, additional margins due to frequency drift and RF calibration are considered similar as DL PRS measurements. Based on TS 38.101, the same frequency drift is assumed for DL and SL receivers. For RF calibration, there is no much difference between DL UE and SL UE. Therefore, the existing margin for DL PRS can be reused for SL PRS.

Proposal 2: The existing margins due to frequency drift and RF calibration for DL PRS can be reused for SL PRS.

The SL Rx – Tx time difference at a UE is defined as TUE-RX – TUE-TX: Where: - TUE-RX is the UE received timing of sidelink subframe #i from a transmitting UE, defined by the first detected path in time. - If the UE reports the transmission timestamp of a SL PRS, TUE-TX is the transmit timing of the sidelink subframe #j of the SL PRS of the UE. Otherwise, TUE-TX is the transmit timing of the UE of sidelink subframe #j that is closest in time to the subframe #i received from the transmitting UE.

In the above SL PRS based measurement, - RSTD error = estimated RSTD – ideal RSTD (based on perfect channel knowledge). - RSRP error = estimated RSRP – ideal RSRP (based on perfect channel knowledge). - RSRPP error = estimated RSRPP – ideal RSRPP (refer to 10.1.38.2.1 of TS 38.133). - Rx-Tx error = abs(estimated TSL-UE-RX – ideal TSL-UE-RX ) (based on perfect channel and UE location knowledge).

Besides the Rel-16/17 Rx-Tx definition with the Tx timing of the closest subframe #j, SL PRS Rx-Tx also supports a new Rx-Tx definition with the actually Tx timing of SL PRS. In this case, the subframes to derive Tx timing and Rx timing may be quite far in the time domain, e.g. 160ms. Then, the Rx-Tx accuracy requirements should be further relaxed consider the frequency drift between Tx and Rx, which is similar as RSTD. However, only SL PRS Rx timing error is simulated based on the assumptions reached before. Time margins due to frequency drift should considered, and the same values for SL RSTD can be used. Alternatively, a tighter time proximity condition for SL PRS transmission and reception, e.g. 10ms, can be considered.

Proposal 3: Consider time margins due to frequency drift to Rx-Tx time difference accuracy requirement if the UE reports the transmission timestamp of a SL PRS, and the time margins for RSTD could be reused.

#### Issue 3-1-6: Test case list Agreements: - No test cases are defined to specifically verify measurement delay requirements for SL PRS-RSRP/RSRPP, regardless of whether the measurement delay requirements are specified for SL PRS-RSRP/RSRPP. - Accuracy requirements for SL PRS-RSRP/RSRPP are verified when configured together with RSTD/RX-TX, respectively. - Option 1a: separate section for testing SL PRS-RSRP/PRS-RSRPP, without verifying the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively - Option 1b: separate section for testing SL PRS-RSRP/PRS-RSRPP, with verifying also the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively - Option 2: same section for SL PRS-RSRP/RSRPP and SL RSTD/Rx-Tx, but different test cases - with and without SL PRS-RSRP/RSRPP.

RSRP and RSRPP are generally used as a supplement and are requested along with other types of measurement like RSTD and Rx-Tx time difference measurement. And as mentioned in proposal 1 that the same side conditions are used for SL PRS measurement except RSTD measurement, we can verify both Rx-Tx and SL RSRP accuracy with a single test case. However, a dedicated test case is needed for SL RSRPP considering the different propagation channel types. The two-tap channel model is used for defining SL RSRPP accuracy while AWGN/TDL channel is considered for RSTD/RSRP/Rx-Tx time difference measurement.

Proposal 4: Verify accuracy requirements for SL PRS-RSRP together with SL Rx-Tx in the same section, but different test cases with and without SL PRS-RSRP.

Proposal 5: Define test case to verify accuracy requirements for SL PRS-RSRPP in a separate section.

3	Conclusion

Based on latest progress, RRM performance requirements for SL positioning are discussed and the following proposals are given:

Proposal 1: The side condition is [0, -6]dB for SL RSTD and [-6]dB for the other SL PRS measurements.

Proposal 2: The existing margins due to frequency drift and RF calibration for DL PRS can be reused for SL PRS.

Proposal 3: Consider time margins due to frequency drift to Rx-Tx time difference accuracy requirement if the UE reports the transmission timestamp of a SL PRS, and the time margins for RSTD could be reused.

Proposal 4: Verify accuracy requirements for SL PRS-RSRP together with SL Rx-Tx in the same section, but different test cases with and without SL PRS-RSRP.

Proposal 5: Define test case to verify accuracy requirements for SL PRS-RSRPP in a separate section.

4	Reference

- R4-2406384, WF on SL positioning and carrier phase positioning, CATT