3GPP TSG-RAN WG4 Meeting #111	R4-2407904

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on gerneral description for ue-PowerClassPerBandPerBC-r17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Power_Limit_CA_DC |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the R17 discussion for tx diversity, it was found that UE can support higher power capability for single band based on tx diversity with two Tx chain, and when working on the CA mode the power class capability of the corresponding band may be lower due to the two tx chains are distributed for each of two bands. Thus, the capability of ue-PowerClassPerBandPerBC-r17 is introduced to indicate the power capability for individual component bands in CA band combinations. According to the WF R4-2406586 approved in RAN4#110bis meeting, the PowerClassPerBandPerBC-r17 may be used to indicate higher power of the corresponding power class for downlink CA with single-carrier uplink and downlink inter-band CA with intra-band CA uplink. Thus, the scenarios of the ue-PowerClassPerBandPerBC-r17 need to be clarified in the specificatioin at least in 38101-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draft CR aims to add the general description for ue-PowerClassPerBandPerBC-r17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The capability of ue-PowerClassPerBandPerBC-r17 may cause ambiguity on the applicable scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sub clause 6.2A.1.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2A	Transmitter power for CA

6.2A.1	UE maximum output power for CA

6.2A.1.0	General

For uplink inter-band CA, the power class applied for a component band within the CA band combination shall not exceed the power class of the band combination itself.  UE may use the capability field of ue-PowerClassPerBandPerBC-r17 in the BandCombination IE to indicate the power class of each band in a band combination if the maximum output power for a component band is lower than that indicated by the capability field of ue-PowerClass. If the field is absent, the applied power class is the lower of the following two values: the power class of the band combination and the power class indicated by the capability field of ue-PowerClass in the BandNR IE if the band has only one uplink carrier, otherwise the default power class if the band has more than one uplink carriers.

For downlink CA with single-carrier uplink and downlink inter-band CA with intra-band CA uplink, a UE may use either the capability field of ue-PowerClassPerBandPerBC-r17 or the capability field of powerClass to indicate the power class capability of the uplink band if it is higher than its parent band combinations. If both are absent, the applied power class is derived from its parent band combination.

<End of Changes>