3GPP TSG-RAN WG4 Meeting # 111		R4-2407935

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda Item:	7.24.2.1

Source: 	CMCC

Title: 			 (Netw_Energy_NR-Perf) Discussion on RRM test setup for SSB-less SCell

Document for:	Discussion

- Introduction

In last meeting, RAN4 agreed the test case list and a few test configurations for FR1 inter-band SSB-less activation, the WF has been approved [1]. In this contribution, we provide our views on the FFS specific test configuration.

- Discussion

Issue 2-1-1: Test configurations for SSB-less

Agreement on Test configurations for SSB-less

- Single P-TRS is configured in SSB-less SCell in the SSB-less SCell activation TCs

- Don’t configure the parameter SCell measurement cycle (measCycleSCell), SSB configuration and SMTC configuration for SSB-less SCell

- Set EPRE difference as [9 dB] + [ΔPL] in the test cases.

  - the [9] can be updated to the same number as in core requirement if any updates on the core requirements.

  - Pcell has [higher or lower] EPRE than Scell.

  - Further discuss whether and how [ΔPL] can be reflected in the TC.

    - ΔPL is the power difference caused by BW difference and/or frequency difference between two Cells.

    - Encourage concrete proposals on how ΔPL due to BW difference and PL difference can be reflected in the test.

- The TCI.State.0 could be used for the TRS of the SSB-less SCell with an additional note that the reference signal is SSB0 from spCell.

Considering the EPRE difference may be larger than 9dB in real network, we think it is rather important to test the pre-compensation behavior of the UE.

Since RAN4 discussed the co-located inter-band CA case in this WI, the main factors which causing EPRE difference are carrier frequency difference and BW difference between cells. In test case, the BW is same between cells, therefore, we only need to care the pathloss difference caused by carrier frequency. For this part, we think we can use 20log(fc) in our test case to approximatively simulate most of the channel models, as in TR 38.901. Thus, the ΔPL =  20 log(fc1/fc2).

Proposal 1:  Set EPRE difference as [9 dB] + ΔPL. ΔPL is the pathloss difference caused by frequency difference between two Cells, which can be set as 20 log(fc1/fc2) in the test case.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1:  Set EPRE difference as [9 dB] + ΔPL. ΔPL is the pathloss difference caused by frequency difference between two Cells, which can be set as 20 log(fc1/fc2) in the test case.

- Reference

- R4-2406301, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon.