3GPP TSG-RAN WG4 Meeting #111	R4-2408049

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	System parameters for LP-WUR

Agenda item:	10.14.2.1

Document for:	Discussion

Introduction

In this paper, we present our views on RF architecture, RF impairments and common simulation parameters.

Following agreements were made in RAN4#110bis [1].

Issue 2-1-1: Operation bands for LP-WUR Agreement: - Focus on FR1 licensed bands - FR2 is not precluded - FFS on which licensed bands will be chosen as example bands for band specific requirements Issue 2-1-3: one or multi-Sets of requirements Way forward: - Discussions could be separated for OOK-based receiver and OFDM-based receiver. And latter stage to discuss whether a generic requirement could be defined, or not. - Further discuss the possibility of a dedicated set of requirements for OOK LP-WUR - FFS whether there is any other factors should be considered for LP-WUR requirements discussion - SNR and NF may be different for these two types. Issue 2-1-4: Performance metric for Rx requirements Way forward: - Agreements have been reached in issue 2-2-1. Issue 2-1-5: Rx antenna assumption for LP-WUR Way forward: - 1Rx is supported in FR1. FFS RX diversity Issue 2-1-6: CBW and RB number for LP-WUR Way forward: - This can be discussed in simulation assumption for ACS/ASCS. Issue 2-1-7: SCS for LP-WUR Way forward: - The SCS of LP-WUS is considered same as in-band NR signals Issue 2-1-8: sync raster for LP-WUR Way forward: - No sync raster is needed for LP-WUR based on RAN1 design ### Sub-topic 2-2 REFSENS requirements Issue 2-2-1: Performance metric for REFSENS Agreement: - Use X% missed detection rate as the starting point for performance metric for LP-WUS RF requirements - FFS on X values - FFS on whether to have false alarm rate Issue 2-2-2: How to specify REFSENS requirements Agreement: - Reuse legacy approach to derive REFSENS, further discuss SNR, NF, IM - FFS whether to design REFSENS requirements or other requirements to ensure LP-WUR meet the coverage target - Side condition for REFSENS test: DL test signal will only have LP-WUS signal. Issue 2-2-3: How to decide SNR value (not requirement) for REFSENS Way forward: - After concluding WUS design in RAN1, the SNR to specify REFSENS requirements should be decided in RAN4 Issue 2-2-4: How to decide NF value (not requirement) for REFSENS Way forward: - Encourage companies input on NF analysis for different LP-WUR types Issue 2-2-5: RF/Antenna Architecture considerations for LP-WUS receiver Way forward: - Detailed antenna/RF architecture can be considered in issue 2-2-4 analysis - FFS other RAN4 impacts than REFSENS ### Sub-topic 2-3 ASCS requirements Issue 2-3-1: Simulation work for ASCS Agreement: - LLS simulation for ASCS is sufficient - The same level PSD for LP-WUS and NR signals is assumed Issue 2-3-2: Detailed Methodology for simulation to evaluate ASCS value and guard RB Way forward: - Use SI assumption as a starting point, further confirm and align the simulation parameters in the group next meeting - Metric for link-level simulation should be aligned - Alignment required on RF impairments before starting the work. Issue 2-3-3: ASCS requirements value Way forward: - FFS ASCS requirements value Issue 2-3-4: Required number of guard RB Way forward: - FFS required number of guard RB for ASCS Issue 2-3-5: RF impairment considerations for ASCS Way forward: - RF impairments can be considered and aligned for ASCS simulation Issue 2-3-6: Side conditions for ASCS test Way forward: - LP-WUS along with required guard RBs is packed with NR legacy DL signal on both sides. ### Sub-topic 2-4 ACS requirements Issue 2-4-1: coexistence System-level simulation to evaluate ACS Agreement: - The same interference level as for main radio is assumed for LP-WUR - Guard RB number needs be evaluated by link level simulation for ACS requirements Issue 2-4-2: Link-level simulation to evaluate ACS Way forward: - Use SI assumption as a starting point, further confirm and align the simulation parameters next meeting - Necessary update and alignment on parameters, e.g., number of WUS RB, RF impairment, ADC bit, and performance metric is required before starting the work. Issue 2-4-3: ACS requirements value Way forward: - FFS ACS requirements value Issue 2-4-4: How to evaluate required guard RB for ACS case Way forward: - FFS required number of guard RB for ACS Issue 2-4-5: Detailed coexistence System-level simulation assumptions Way forward: - System-level simulation is not needed for ACS evaluation ### Sub-topic 2-5 other UE RF requirements Issue 2-5-1: Any other Rx requirements should be specified Agreement: - Apart from REFSENS, ACS, Rx requirements of IBB, OBB, intermodulation as well as spurious emissions should be specified for LP-WUR. - Other legacy receiver requirements are not precluded ### Sub-topic 2-6 Testabilty for UE RF requirements Issue 2-6-1: Test metric for LP-WUR receiver Way forward: - Test metric should be aligned with performance metric for Rx requirements

Discussion

Receiver assumptions

Zero-IF Receiver architecture

The Zero-IF receiver architecture is aimed for multi band usage. This architecture can have a high degree re-use of the NR radio like frontend, antennas, LNA, mixer, ADC (if it is configurable in frequency and sample depth) can be reused.

Figure 1 Basic UE LP-WUR architecture for utilizing a Zero-IF receiver.

As with any direct conversion receiver, DC offset, and self-image are problems here. For the LP-WUS signal, since there can be data at the DC, so any potential DC offset cancellation loop can cause information loss. LO is used for tuning the receiver to different frequencies. Assumed values of the performance parameters of different components used in this architecture are shown in Table 1. These values are taken from published literature ([2]--[8]). As the direct conversion receiver folds the image spectrum, an image reject mixer is used. However, based on the frequency domain content of the signal, an acceptable solution can be designed without an image reject mixer. This however will work only for an envelope detector based receiver. A sequence based detector will need an image reject mixer.

Table 1 Gain, and NF values for a reference Zero-IF based receiver for a sequence detector LP_WUR.

| Component | Gain (dB) | NF (dB) | Remarks |
| --- | --- | --- | --- |
| RF BPF/Duplex filter | 0 | 1.5 |  |
| LNA | 15 | 7 | Low power LNA with large NF. |
| Mixer | 3 | 10 |  |
| BB amplifier | 40 | 25 | BB amplifier providing the majority of required gain. |
| Total | 54.5 | 11.6 |  |

Zero-IF architecture supports a high degree of reuse of the NR main radio components.

To support more than one band, the receiver could use a wideband LNA or multiple LNAs supporting smaller frequency area.

As the bandwidth of the WUS signal is expected to be scaled according to the sub carrier spacing the LP filter will most likely be required to have different cut off frequencies, e.g., one configuration for each sub-band spacing configuration.

DC offset cancellation loop to attenuate the DC signal though could cause information loss.

Agree to use zero-IF receiver as a baseline architecture for LP_WUR.

Agree to use the estimated NF of 12dB as a baseline for LP_WUR.

RF impairments

During the SI phase, companies proposed to use different RF impairments such as frequency offset, timing offset, phase noise, limited ADC bits etc. However, there was no common understanding on which impairments to consider and what models to use for the same. This led to inconsistency in the results shared by different companies. In the normative phase, we should agree first on all the impairments necessary before we conclude the values of guard RBs required.

Inconsistent assumptions regarding impairments will lead to results which cannot be merged.

Agree to impairments and used models before collating the simulation results.

Simulation parameters

We need to agree to a set of common parameters for doing the simulation to have results which can be collated together easily. Here, we are proposing initial values for the parameter which do align with RAN1 assumptions.

Table 4 Common simulation parameter to be used for LLS.

| Simulation Parameter | Design Value |
| --- | --- |
| LP-WUS BW | 11 PRBs irrespective of SCS |
| UE channel BW | 20MHz (2048 tones, 106 PRBs) |
| SCS | 15KHz, 30KHz |
| Modulation | OOK1 and OOK4, M=1, 2, 4 |
| Sequence used in ON duration | Any CAZAC sequence. |
| Number of TX and RX antennas | 1x1 |
| Manchester encoding format | Always present, 1→{0,1} and 0→{1,0} |
| NR transmission in the same carriers | Active with QPSK modulation. |
| LPF filter used | 5th order Butterworth filter with downsampling |
| LP-WUS information bits | Based on RAN1 agreement, 8 bits (16 with Manchester coding) can be starting point. |
| CRC bits | No CRC bits used |
| Preamble symbols | No preamble used for evaluation. LP-WUS location is assumed to be known. |
| LP_WUS location in the channel | In centre. |
| Data symbols per slot | Symbols are scaled accordingly to fit the payload |
| CP handling (DFT-s-OFDM, i.e., OOK-4) | No special handling. |
| Carrier frequency and doppler frequency (TDL cases) | 2.6GHz, 30Hz (8Kmph) |
| Power boosting | No power pooling considered for evaluation. / Flat PSD |
| RF impairments | To be agreed |
| Time synchronization | Both ideal and time jitter of 200 ppm are evaluated. |
| Frequency offset and ADC bit width | 0ppm, ±10ppm, ±150ppm with Ideal ADC bit width considered. |

Agree to use common simulation parameters defined above in Table 2 for the LLS to determine the guard RBs for ACS and ASCS test cases.

Conclusion

In this paper, we presented our views on RF architecture, RF impairments and common simulation parameters. The following Observations and Proposals were made:

Observation 1: Zero-IF architecture supports a high degree of reuse of the NR main radio components.

Observation 2: To support more than one band, the receiver could use a wideband LNA or multiple LNAs supporting smaller frequency area.

Observation 3: As the bandwidth of the WUS signal is expected to be scaled according to the sub carrier spacing the LP filter will most likely be required to have different cut off frequencies, e.g., one configuration for each sub-band spacing configuration.

Observation 4: DC offset cancellation loop to attenuate the DC signal though could cause information loss.

Proposal 1: Agree to use zero-IF receiver as a baseline architecture for LP_WUR.

Proposal 2: Agree to use the estimated NF of 12dB as a baseline for LP_WUR.

Observation 5: Inconsistent assumptions regarding impairments will lead to results which cannot be merged.

Proposal 3: Agree to impairments and used models before collating the simulation results.

Proposal 4: Agree to use common simulation parameters defined above in Table 2 for the LLS to determine the guard RBs for ACS and ASCS test cases.

- References

- R4-2406619, WF on NR LP-WUS UE requirements, Vivo, RAN4#110bis, Changsha, China, 15 – 19 April 2024.

- Pletcher, N.M., Gambini, S. and Rabaey, J., 2008. A 52uW Wake-Up Receiver with 72 dBm Sensitivity Using an Uncertain-IF Architecture. IEEE Journal of solid-state circuits, 44(1), pp.269-280

- Cook, B.W., Berny, A.D., Molnar, A., Lanzisera, S. and Pister, K.S., 2006, February. An ultra-low power 2.4 GHz RF transceiver for wireless sensor networks in 0.13um CMOS with 400mV supply and an integrated passive Rx front-end. In 2006 IEEE International Solid State Circuits Conference-Digest of Technical Papers (pp. 1460-1469).

- E. -H. V. Yeh, A. -H. Lo, W. -S. Chen, T. -J. Yeh and M. Chen, "A 16 nm FinFET 0.4 V inductor-less cellular receiver front-end with 10 mW ultra-low power and 0.31 mm2 ultra-small area for 5G system in sub-6 GHz band," 2017 Symposium on VLSI Circuits, Kyoto, Japan, 2017, pp. C72-C73.

- Du, Y., Hu, W., Li, G.F. and Liu, S., 2018. Low power hybrid PG_Filter-AGC analog baseband for wireless receivers. IEICE Electronics Express, 15(3), pp.20171197-20171197.

- P. -H. P. Wang et al., "A 400 MHz 4.5 nW −63.8 dBm sensitivity wake-up receiver employing an active pseudo-balun envelope detector," ESSCIRC 2017 - 43rd IEEE European Solid State Circuits Conference, Leuven, Belgium, 2017, pp. 35-38..

- N. M. Pletcher, S. Gambini and J. Rabaey, "A 52 μ W Wake-Up Receiver With − 72 dBm Sensitivity Using an Uncertain-IF Architecture," in IEEE Journal of Solid-State Circuits, vol. 44, no. 1, pp. 269-280, Jan. 2009.

- H. Bialek et al., "A Passive Wideband Noise-Canceling Mixer-First Architecture With Shared Antenna Interface for Interferer-Tolerant Wake-Up Receivers and Low-Noise Primary Receivers," in IEEE Journal of Solid-State Circuits, vol. 57, no. 9, pp. 2611-2625, Sept. 2022.