3GPP TSG-RAN WG4 Meeting # 111							R4-2408057

# Fukuoka, Japan, 20 May – 24 May, 2024

Agenda item:	9

Source: 	Huawei, HiSilicon

Title: 	Consideration on Rel-18 RAN4 UE feature list for NR

Document for:	Discussion

- Introduction

This contribution provides our opinion on the RAN4 UE feature list for Rel-18 NR.

- NR_channel_raster_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_channel_raster_enh | 28-1 | Enhanced channel raster | The UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1, TS 38.101-5 |  | Yes |  | UE may not support requirements for UE specific channel bandwidths located on enhanced channel raster; configuring a narrower UE-specific channel bandwidth inside a wider gNB channel bandwidth may not be possible. | Per Band | No | FR1 only | The feature is supported for applicable bands in FDD-TDD and FR1/FR2 combinations | Applies only for bands with a 100 kHz channel raster for both TN and NTN. Should be early implementable from Rel-16. | Mandatory with capability signaling for all Rel-18 UEs for certain bands as defined in 38.101-1 and 38.101-5 Optional otherwise |

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gaps with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Details in Clause [9.1.x.2] of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-2 | Support for dynamic collisions | Support RRM requirements for handling dynamic collisions between a Pre-MG and another measurement gap or Pre-MG. | 32-1 | Yes | No | UE is not expected to meet RRM requirements for dynamic collisions | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-3 | Concurrent gaps with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Details in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-4 | Inter-RAT EUTRAN measurements without gap and outside active DL BWP | Support inter-RAT EUTRAN measurements outside active DL BWP for nogap-noncsg | 19-1b | Yes | NA | UE does not meet the requirements of inter-RAT EUTRAN measurements without gap in TS 38.133 and the UE behavior is unknown to network | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-5 | Inter-RAT EUTRAN measurement without gap and within active DL BWP | Support of inter-RAT EUTRAN measurements without gap when CRS is contained within UE’s active DL BWP |  | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-6 | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | 32-4 or 32-5 | Yes | No | UE is not allowed to cause scheduling restriction defined in TS 38.133 for 32-4 or 32-5 | Per UE | No | No | N.A | - A bitmap for 6 effective measurement window (EMW) patterns defined in TS 38.133. - #0 and #1 are mandatory, if UE supports EMW feature. Other patterns are optional Note: If UE supports 32-4 or 32-5 and UE requires scheduling restriction, UE should support this FG | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-7 | Simultaneous reception of NR data and EUTRAN CRS with different numerology | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS and PDCCH or PDSCH reception from the serving cell with a different numerology | 32-4 or 32-5 | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |

Table 2: Rel-18 LTE UE features for NR_MG_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | x-y | interRAT-NeedForInterruptionNR-r18 | Support of inter-RAT NR measurements without gap with or without interruption when the interRAT-NeedForGapsNR-r16 is false. Note: This feature already has a defined UE capability: ‘interRAT-NeedForInterruptionNR-r18’. The intention of adding this FG is only keep consistency between 38.822 and 36.306. | interRAT-NeedForGapsNR-r16 | Yes | NA | The UE does not support inter-RAT NR measurements without gap with or without interruption for performing inter-RAT NR measurement without gap | [Per target band per BC] Note: the same granularity as interRAT-NeedForGapsNR-r16 | No | No | NA | # Candidate value: “{no-gap-with-interruption, no-gap-no-interruption}” | Optional with capability signalling |
| 32. NR_MG_enh2 | x-z | Simultaneous reception of E-UTRAN data and NR SSB with different numerology | Support concurrent inter-RAT measurement on NR cell with NR SSB and PDCCH or PDSCH reception from the E-UTRAN serving cell with a different numerology | x-y (interRAT-NeedForInterruptionNR-r18) | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | N/A |  | Optional with capability signalling |