3GPP TSG-RAN WG4 Meeting #111 	R4-2408064

Fukuoka, Japan, May 20 – 24, 2024

Source:	Nokia

Title:	On Finalizing the Big CR of HST FR2 Enhanced: PDSCH with Carrier Aggregation

Agenda item:	7.7.3.2

Document for:	Discussion

Introduction

In RAN4#110bis meeting, the draft Big CR on Rel-18 HST FR2 Enhanced for 38.101-4 [1] was endorsed, in which the contributions from corresponding draft CRs were captured, including the draft CR on Rel-18 HST FR2 PDSCH with Carrier Aggregation (CA) [2]. Both in [1] and [2], the SNR requirements Table 7.2A.2.2-3 and Table 7.2A.2.2-4 are still in square brackets as yellow-highlighted below:

Table 7.2A.2.2-3: Single carrier performance for HST-FR2-DPS CA Configurations with 1 Active TCI State Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; TDD UL-DL pattern; Propagation condition; Number of active PDSCH TCI states; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). R.PDSCH.5-16.1 TDD; 50 / 120; 64QAM, 0.43; FR2.120-1; HST-DPS-FR2-BI-B; 1; 2x2, ULA Low; 70; [13.6]. R.PDSCH.5-16.2 TDD; 100 / 120; 64QAM, 0.43; FR2.120-1; HST-DPS-FR2-BI-B; 1; 2x2, ULA Low; 70; [13.9]. R.PDSCH.5-12.2 TDD; 200 / 120; 64QAM, 0.43; FR2.120-1; HST-DPS-FR2-BI-B; 1; 2x2, ULA Low; 70; [14.0]. R.PDSCH.5-16.3 TDD; 400 / 120; 64QAM, 0.43; FR2.120-1; HST-DPS-FR2-BI-B; 1; 2x2, ULA Low; 70; [13.8]. Table 7.2A.2.2-4: Single carrier performance for HST-FR2-DPS CA Configurations with 2 Active TCI States Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; TDD UL-DL pattern; Propagation condition; Number of active PDSCH TCI states; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). R.PDSCH.5-17.1 TDD; 50 / 120; 64QAM, 0.43; FR2.120-1; HST-DPS-FR2-UNI-A; 2; 2x2, ULA Low; 70; [13.7]. R.PDSCH.5-17.2 TDD; 100 / 120; 64QAM, 0.43; FR2.120-1; HST-DPS-FR2-UNI-A; 2; 2x2, ULA Low; 70; [13.9]. R.PDSCH.5-12.1 TDD; 200 / 120; 64QAM, 0.43; FR2.120-1; HST-DPS-FR2-UNI-A; 2; 2x2, ULA Low; 70; [14.0]. R.PDSCH.5-17.3 TDD; 400 / 120; 64QAM, 0.43; FR2.120-1; HST-DPS-FR2-UNI-A; 2; 2x2, ULA Low; 70; [13.8].

which indicate that they are not (yet) the final values.

Discussion

The simulation results contributed by companies working on Rel-18 HST FR2 Enhanced are documented in [3]. The simulation results from companies are quite aligned shown by the less-than 2 dB of span. As all companies’ simulation results have been considered in defining the SNR requirements as captured in Table 7.2A.2.2-3 and Table 7.2A.2.2-4 in [1] and [2], it is expected that they are stable enough. Hence, the square brackets in Table 7.2A.2.2-3 and Table 7.2A.2.2-4 in [1] and [2] can be removed prior to the inclusion into 38.101-4 specifications.

The simulation results from companies have been collected and they are well aligned; and it is not expected that there will be simulation result updates from any companies.

The SNR requirements of Rel-18 HST FR2 have been adapted to align with the SNR requirements of HST FR2 PDSCH with multi-Rx in terms of the additional margin for 64QAM.

RAN4#111 is the last meeting for RAN4 demodulation performance requirements of Rel-18 HST FR2 Enhanced.

The SNR requirements of HST FR2 PDSCH with Carrier Aggregation shall be finalized in RAN4#111, and the square brackets in the corresponding draft CR and the final formal big CR shall be removed.

The changes as proposed in Proposal 1 above have been applied to the revised draft CR [4] submitted in RAN4#111.

Conclusion

In this paper, we have the following Observations and Proposals:

Observation 1: The simulation results from companies have been collected and they are well aligned; and it is not expected that there will be simulation result updates from any companies.

Observation 2: The SNR requirements of Rel-18 HST FR2 have been adapted to align with the SNR requirements of HST FR2 PDSCH with multi-Rx in terms of the additional margin for 64QAM.

Observation 3: RAN4#111 is the last meeting for RAN4 demodulation performance requirements of Rel-18 HST FR2 Enhanced.

Proposal 1: The SNR requirements of HST FR2 PDSCH with Carrier Aggregation shall be finalized in RAN4#111, and the square brackets in the corresponding draft CR and the final formal big CR shall be removed.

- References

- R4-2405985, Big CR for TS 38.101-4 on Rel-18 FR2 HST demodulation requirements, Samsung, RAN4#110bis, Changsha, China, April 15 - 19, 2024.

- R4-2406005, Draft CR on PDSCH requirements with CA, Nokia, RAN4 #110bis, Changsha, China, April 15 – 19, 2024.

- R4-2405860 Simulation results summary for Rel-18 FR2 HST demodulation requirement, Samsung, RAN4#110bis, Changsha, China, April 15 - 19, 2024.

- R4-2408063, Draft CR on PDSCH requirements with CA, Nokia, RAN4 #111, Fukuoka, Japan, May 20 – 24, 2024.