3GPP TSG-RAN WG4 Meeting #111 	R4-2408065

Fukuoka, Japan, May 20 – 24, 2024

Source:	Nokia

Title:	On Finalizing the Big CR of HST FR2 Enhanced: PDSCH with Multi-Rx

Agenda item:	7.7.3.3

Document for:	Discussion

Introduction

In RAN4#110bis meeting, the draft Big CR on Rel-18 HST FR2 Enhanced for 38.101-4 [1]Error! Reference source not found. was endorsed. As shown below, the SNR requirements in [1] are still in square brackets indicating that they are not yet the final values and can still be updated:

Table 7.2.2.2.6-3: Minimum performance for multi-Rx simultaneous reception under HST scenario Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; TDD UL-DL pattern; Propagation condition; Number of active PDSCH TCI states [Note 3]; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; [RRH#i]; 200 / 120; 64QAM, 0.50; FR2.120-1; HST-DPS-FR2-BI-B1-MR; [2]; 2x2; 70; [15.0]. [R.PDSCH. 5-18.1 TDD]. [RRH#j]; 200 / 120; 16QAM, 0.37; FR2.120-1; HST-DPS-FR2-BI-B1-MR; [2]; 2x2; 70; [8.2]. [R.PDSCH. 5-18.2 TDD]. Note 1 : The RRH#i (i =-1,0,1,2…) and RRH#j (j=1,2,3,…) indicate the RRHs simultaneously transmitting PDSCH to the UE. Note 2: Receive timing difference (RTD) of PDSCH data transmitted from RRH#i (i =-1,0,1,2…) and RRH#j (j=1,2,3,…) at the UE between two Rx chains is 0.88 us (1.5*CP). Note 3: Only 1 available active TCI state per RRH..

Furthermore, the structure of Table 7.2.2.2.6-3 can still be refined.

Discussion

The simulation results of HST FR2 PDSCH with multi-Rx in RAN4#110bis are collected in [2]. The span is only 1.4 dB, showing that the simulation results from the companies are well aligned.

The simulation results from companies have been updated in RAN4#110bis, and they are well aligned. Hence, it is not expected that there will be further simulation result updates from any companies on HST FR2 with multi-Rx.

RAN4#111 is the last meeting for RAN4 demodulation performance requirements of Rel-18 HST FR2 Enhanced.

The SNR requirements for HST FR2 PDSCH with multi-Rx shall be finalized in RAN4#111, and the square brackets in the final formal big CR shall be removed.

In [1], the requirements of HST FR2 PDSCH with multi-Rx are tabulated in Table 7.2.2.2.6-3. In the current structure of Table 7.2.2.2.6-3, The RRH number is located in the same column as the Reference Channel.

It is not common to use the Reference Channel column in the requirements table in the specification for both the intended Reference Channel and other parameters.

RAN4 to add another column for RRH number to avoid other parameters than Reference Channel to be placed in the Reference Channel column as below:

| Test Number | RRH Number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states [Note 3] | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | i | [R.PDSCH. 5-18.1 TDD] | 200 / 120 | 64QAM, 0.50 | FR2.120-1 | HST-DPS-FR2-BI-B1-MR | [2] | 2x2 | 70 | [15.0] |
|  | j | [R.PDSCH. 5-18.2 TDD] | 200 / 120 | 16QAM, 0.37 | FR2.120-1 | HST-DPS-FR2-BI-B1-MR | [2] | 2x2 | 70 | [8.2] |
| Note 1 : The RRH#i (i =-1,0,1,2…) and RRH#j (j=1,2,3,…) indicate the RRHs simultaneously transmitting PDSCH to the UE. Note 2: Receive timing difference (RTD) of PDSCH data transmitted from RRH#i (i =-1,0,1,2…) and RRH#j (j=1,2,3,…) at the UE between two Rx chains is 0.88 us (1.5*CP). Note 3: Only 1 available active TCI state per RRH. |  |  |  |  |  |  |  |  |  |  |

The Reference channels in the requirements table are still in square brackets although they are already fixed and stable.

The number of active PDSCH TCI states is still in square brackets although it is already fixed.

The square brackets in Table 7.2.2.2.6-3 for the Reference channel and the Number of active PDSCH TCI states shall be removed.

Conclusion

In this paper, we provide our views and responses to the remaining open issues in HST FR2 PDSCH requirements with multi-RX. Our Observations and Proposals are summarized as follows:

Observation 1: The simulation results from companies have been updated in RAN4#110bis, and they are well aligned. Hence, it is not expected that there will be further simulation result updates from any companies on HST FR2 with multi-Rx.

Observation 2: RAN4#111 is the last meeting for RAN4 demodulation performance requirements of Rel-18 HST FR2 Enhanced.

Proposal 1: The SNR requirements for HST FR2 PDSCH with multi-Rx shall be finalized in RAN4#111, and the square brackets in the final formal big CR shall be removed.

Observation 3: It is not common to use the Reference Channel column in the requirements table in the specification for both the intended Reference Channel and other parameters.

Proposal 2: RAN4 to add another column for RRH number to avoid other parameters than Reference Channel to be placed in the Reference Channel column as below:

| Test Number | RRH Number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states [Note 3] | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | i | [R.PDSCH. 5-18.1 TDD] | 200 / 120 | 64QAM, 0.50 | FR2.120-1 | HST-DPS-FR2-BI-B1-MR | [2] | 2x2 | 70 | [15.0] |
|  | j | [R.PDSCH. 5-18.2 TDD] | 200 / 120 | 16QAM, 0.37 | FR2.120-1 | HST-DPS-FR2-BI-B1-MR | [2] | 2x2 | 70 | [8.2] |
| Note 1 : The RRH#i (i =-1,0,1,2…) and RRH#j (j=1,2,3,…) indicate the RRHs simultaneously transmitting PDSCH to the UE. Note 2: Receive timing difference (RTD) of PDSCH data transmitted from RRH#i (i =-1,0,1,2…) and RRH#j (j=1,2,3,…) at the UE between two Rx chains is 0.88 us (1.5*CP). Note 3: Only 1 available active TCI state per RRH. |  |  |  |  |  |  |  |  |  |  |

Observation 4: The Reference channels in the requirements table are still in square brackets although they are already fixed and stable.

Observation 5: The number of active PDSCH TCI states is still in square brackets although it is already fixed.

Proposal 3: The square brackets in Table 7.2.2.2.6-3 for the Reference channel and the Number of active PDSCH TCI states shall be removed.

- References

- R4-2405985, Big CR for TS 38.101-4 on Rel-18 FR2 HST demodulation requirements, Samsung, RAN4#110bis, Changsha, China, April 15 - 19, 2024.

- R4-2405860 Simulation results summary for Rel-18 FR2 HST demodulation requirement, Samsung, RAN4#110bis, Changsha, China, April 15 - 19, 2024.