# 3GPP TSG-RAN WG4 Meeting #111	R4-2408078

# Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	7.24.1

Source: 	Samsung

Title: 	Views on intra-band NC CA scenario

Document for:	Discussion

1		Introduction

For Rel-18 NES, there is one remaning open issue for RRM core maintenance on SSB-less feature supporting under intra-band NC CA scenario.

RAN4#110 R4-2403526 Issue 1-3-1: Intra-band NCCA Agreement: Regarding FR1 intra-band NCCA with SSB-less operation, further study whether we can reuse or not the SSB-less activation delay requirement specified for FR1 inter-band CA. The study does not impact the completion timeline of the WI. RAN4#110bis R4-2406301 Issue 1-1-5: Intra-band non-contiguous CA - Proposals - Option 1 For FR1 intra-band non-contiguous CA, reuse the SSB-less SCell activation delay requirement defined for FR1 collocated inter-band CA with the same RTD side condition i.e. RTD within CP (Nokia, Intel) - Option 1a: Introduce the optional with capability signalling with per FS granularity for UE supporting intra-band NCCA SSB-less SCell operation in Rel-18. (Intel) - Option 2: The delay requirements of SSB-less SCell activation for intra-band non-contiguous CA can be same as that for legacy intra-band contiguous CA provided the UE impelmentation architecture are same for the two scenarios. The exact side conditions for intra-band non-contiguous CA can be further discussed. (ZTE) - Option 3: RAN4 will not consider SSBless SCell operation for intra-band NCCA in R18. (QC) - Option 4: (Apple, Huawei) - For FR1 intra-band NCCA with SSB-less operation, EPRE difference at UE side shall be NOT larger than 6dB - For FR1 intra-band NCCA with SSB-less operation, RTD between the target SCell and the intra-band NCCA collocated reference serving cell can be within CP - Option 5: (CMCC) - Define two sets of requirement and side condition, reuse the intra-band contiguous CA case and inter-band CA case respectively. - Define two UE capabilities for intra-band NCCA scenario, which corresponding to two set of requirements respectively, the granularity could be per FS indication - Option 6: - Specify only assuming separate chains and reuse largely the requirements specified for inter-band cases; - Specify UE optional capability signalling for intra-band NCCA SSB-less SCell operations in a similar way as for inter-band cases; - UE with single chain implementation does not indicate support for intra-band NCCA SSB-less SCell operations and does not need to meet the requirements. - Option 7: - Specify two sets of requirements respectively assuming single and separate chains, where one of the sets reuse largely the requirements specified for inter-band cases; - Specify UE optional capability signalling for two different implementations separately; - Different UE implementations are required to meet different sets of requirements correspondingly. - Other options are not precluded

In this contribution, our view for intra-band NC scenario supporting with SSB-less operation were provided.

2		Discussion

The main debate point in previous meeting is the default assumption on UE architecture for intra-band NC CA i.e., intra-band continuous CA like (single RF chain and single baseband/FFT) or inter-band CA like (multi-RF chains and separate baseband/FFT operation).

The side condition including RTD and power difference for SSB-less operation under intra-band NC CA scenario is depending on UE architecture assumption.

In our understanding, UE implementation on supporting intra-band NC CA can be varied pending on frequency separation distance across different CCs. At least partial RF components can be shared by different carriers in the same band i.e. RF frond-end including antenna and LNA. With this consideration, same power imbalance assumption from intra-band contiguous CA, and EN-DC can be applied here as 6 dB difference from receiver side as power density difference.

Observation 1: At least partial RF components can be shared across CCs i.e., antenna, LNA for Intra-band NC CA.

Observation 2: For power difference side condition: 6dB can be assumed for intra-band NC CA.

Pending on pass-band bandwidth/ frequency separation distance, other RF components e.g., mixer, Analog filter can be shared or dedicated for each cc. Regarding baseband processing, single FFT operation is still possible if aggregated BW less than 100MHz for FR1. On other hand, additional BB filtering is still required to meet ACS and emission requirements.

Observation 3: It’s not precluded single Rx chain (RF and baseband) can be implemented to support intra-band NC CA which pending on frequency separation distance and pass-band bandwidth on operating bands.

Considering the RTD is the receiving timing difference between SSB-less SCell and the reference cell, and RTD is mainly composed of three factors: the TAE (time alignment error) at the BS side, the relative propagation delay difference over the air interface and the receive time delay at the UE side. Thus, RTD value may be impacted by if the different carrier components on UE side use the same RF chain or not, and the relative propagation delay difference over the air interface between the SSB-less SCell and the reference cell. Due to the different channel characteristic over the frequency of SSB-less SCell and the reference cell, this relative propagation delay difference between them will change. Inherently, the definition of different RTD sets could depend partly on frequency separation range between SSB-less SCell and the reference cell.

From UE implementation perspective:

- If UE implemented separate RF chain and BB processing for intra-band NC CA on certain bands, then UE can tolerate larger RTD value i.e., same as inter-band CA with value less than CP.

- If UE implemented single RF chain and single FFT operation, then RTD value shall be small enough to avoid inter-symbol interference i.e. 260ns or value between 260ns ~ 3us.

Observation 4: The side condition of RTD can be different pending on UE architecture and frequency separation range across CCs.

Observation 5: In order to optimize performance under intra-band NC CA scenario, multiple sets of requirements maybe required pending on UE capability and operating bands.

Observation 6: It’s hard to conclude in Rel-18 during maintenance phase for intra-band NC CA scenario supporting.

We also aware that RAN4 will have a new SI in Rel-19 for fragmented carriers’ usage in June RAN-P (scope endorsed in RAN#103 RP-240498). This SI targets to reduce number of Rx chains on supporting multiple fragmented spectrum on the band . To efficient usage of these fragmented carriers, SSB-less feature can be considered.

Proposal: Further discuss the supporting of SSB-less operation feature with intra-band NC CA scenario in Rel-19 e.g., under Rel-19 Fragmented Carriers SI or Rel-19 NES WI.

3		Conclusion

In this contributions, our view for intra-band NC scenario supporting with SSB-less operation were provided:

- Observation 1: At least partial RF components can be shared across CCs i.e., antenna, LNA for Intra-band NC CA.

- Observation 2: For power difference side condition: 6dB can be assumed for intra-band NC CA.

- Observation 3: It’s not precluded single Rx chain (RF and baseband) can be implemented to support intra-band NC CA which pending on frequency separation distance and pass-band bandwidth on operating bands.

- Observation 4: The side condition of RTD can be different pending on UE architecture and frequency separation range across CCs.

- Observation 5: In order to optimize performance under intra-band NC CA scenario, multiple sets of requirements maybe required pending on UE capability and operating bands.

- Observation 6: It’s hard to conclude in Rel-18 during maintenance phase for intra-band NC CA scenario supporting.

Proposal: Further discuss the supporting of SSB-less operation feature with intra-band NC CA scenario in Rel-19 e.g., under Rel-19 Fragmented Carriers SI or Rel-19 NES WI.

Reference

[1] RP-240678, “Way Forward on Fragmented Carriers in Rel-19”, TELUS

[2] R4-2406301, “WF on RRM requirements for NR network energy saving”, Huawei