TSG-RAN Working Group 4 (Radio) meeting #111	R4-2408083

Fukuoka, Japan, 20th to 24th May 2024

Source:	Ericsson

Title:	TP to TR 38.922: Addition of array antenna model description in clause 7

Agenda item:	10.3.5

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#110-bis in Changsha) the work to describe AAS BS antennas operating within 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz started within the scope of the SI (FS_NR_IMT_4400_7125_14800MHz) initiated to provide ITU-R WP 5D with information.

In this contribution we have collected technical background and descriptions for the array antenna model used by 3GPP and ITU-R. Currently, the background is fragmented into multiple technical reports which often cause confusion and is source to misunderstandings. It would be good to collect information relevant for the antenna model in the technical report TR 38.922. At the end of this contribution a text proposal based on information available in TR 37.840, TR 38.803, TR 38.820 and TR 38.921 to be included as technical background in TR 38.922 is attached for approval.

# Discussion

In TR 37.840 when the array antenna model for AAS BS first was defined in Rel-13 modelling aspects of non-perfect array excitation was considered. With non-perfect array excitation means situations where random excitation is considered for situations like modelling of array calibration errors for the wanted carrier or modelling of the array response outside the carrier in the unwanted emission domain.

Based on the array antenna model overview in previous contribution [1] we now focus on modelling aspects related to array antenna characteristics related to modelling non-perfect array excitations, like in the case of modelling the array response moving away from the carrier centre frequency.

Information of the antenna gain outside the carrier is of great interest evaluating adjacent service spectrum sharing situations. In these situations, both the antenna response and filter response will determine the interference within the victim receiver bandwidth. Group delay distortion of the RF filters in the BS will have a de-correlation effect which further whiten the spatial properties of emission for adjacent systems.

Considering a AAS BS as an aggressor the produced EIRP towards the victim can be expressed as:

EIRPθ,φ=Ptx+AAθ,φ

, where Ptx is the total conducted power fed to the array antenna and AA(q,j) is the array antenna gain. It is worth to notice that Ptx will reduce as function of frequency offset due to RF filter suppression, ACLR suppression, transmission line mismatch effects and AA(q,j) will reduce due to array decorrelation and antenna element detuning effects. Hence, to model out-of-carrier and out-of-band spatial emission requires detailed information relevant for both the transmitter and antenna implementation.

Observation 1: To model the interference level at the non-mobile adjacent system victim receiver, reasonable knowledge about the transmitter out-of-carrier and out-of-band characteristics could be on top of the antenna characteristics. Typically, the transmitter characteristics is gathered from information in the BS RF specification.

In reality, the array antenna gain at a specific carrier frequency offset will depend on many aspects. The array decorrelation is just one technical aspect that will occur. The array antenna model as it is defined does not captured impact on the antenna element due to frequency offset. If the frequency offset considered is large the antenna element will be distorted and for very large offset collapse to an omni-directional pattern. The effect of RF filters in this context is not negligible.

The array antenna model originally captured in TR 37.840 have support to model array factor decorrelation effects only. An array excitation signal correlation factor is defined to model array patterns considering full array factor to the other extreme considering no gain due to the array factor. The correlation factor is applied to the array factor as described in equation below. The average radiation pattern is modelled as the sum of the sub-array pattern and the array factor pattern in logarithmical scale as:

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1

, where r is the correlation factor defined in the interval 0≤ρ≤1.

For r=1, full array factor is applied which is the typical value for wanted carrier. For r=0, the array factor reduces to 1 with the consequence that only the element factor provides array antenna gain.

Assuming an array antenna configuration with 4x8 array of 3x1 sub-arrays the impact of signal correlation impact on the radiation pattern can be evaluated for the case with 10 degrees down-tilt angle applied. The vertical average radiation pattern is plotted in Figure 2-1. It can be seen that the array factor reduces gradually for r<1.

# Figure 2-1: Vertical average radiation pattern

It can be noticed that significant array factor beamforming still remains for r=0.5. For r=0, only gain produced by the sub-array remains.

With vertical sub-array structures the separation between vertical sub-arrays will typically be much larger than 0.5l, hence “grating lobes” will be generated when the beam is steered along the vertical axis. But also, the sub-array itself will help to suppress the grating lobes, as show in Figure 2-1. The relation between directivity and correlation is plotted in Figure 2-2.

# Figure 2-2: Relation between directivity and correlation

The challenge for sharing studies is that guidance for how to model r including the RF filter effects for adjacent systems is currently captured in 3GPP or in other forums.

For a AAS BS the correlation factor outside the carrier would typically depend on the intermodulation response, wide band noise from power transmitters, decorrelation effects due to detuning effects of antenna elements and transmission lines in the Radio Distribution Network (RDN) and transmitters and filters having different out-of-band characteristics.

Observation 2: Information on relevant correlation factor as function of frequency offset is currently not captured in 3GPP technical reports.

To model the correlation factor response as function of frequency offset an approach using a piece-wise linear roll-off or multi-step profile can be considered. In Figure 2-2, an example of a multi-step profile is visualized including the wanted signal and adjacent channels. The first adjacent channel contents will be dominated by intermodulation distortion. Further away it will be a mix of intermodulation distortion and wide band noise.

In the example, the correlation factor r is equal to 0.5 for the first adjacent channel, equal to 0.25 for the second adjacent channel and equal to 0.12 for the third adjacent channel. At some frequency offset the array factor will collapse to 0 (not shown in figure).

# Figure 2-2: Example of correlation roll-off model

Observation 3: RAN4 need to find appropriate method to determine the correlation roll-off profile. Potential methods are establishing profile based on measured characteristics from multiple vendors and/or simulation of PA characteristics.

It is reasonable to assume that the transmitter correlation behaviour outside the wanted carrier will depend on implementation details related to considered PA technology and linearization algorithm. In addition, the correlation profile will depend on the carrier configuration (configured power, number of carriers, contiguous/non-contiguous carrier aggregation and carrier bandwidth). It will be difficult but not impossible to find relevant profiles that can cover all cases of configurations and implementation also taking to account RF filter effects. This is the reason why it was not done in Rel-13. Now have more experience from building and using AAS BS, which would make it possible to continue the work to establish a correlation roll-off profile requested by ITU-R.

Proposal 1: Capture array antenna model and corresponding description presented in [1] in TR 38.922, clause 7. A text proposal to [2] is attached at the end of this contribution.

Proposal 2: Within the scope of the SI continue to discuss how to model the correlation factor roll-off with the goal to find a representable model.

# Conclusion

In previous contribution we presented an overview of the array antenna model together with some description and background. It would be beneficial to capture the description of the array antenna model in the technical report TR 38.922.

With the array antenna model in mind, missing information related to the correlation factor roll-off needs to be established to model out-of-carrier characteristics. RAN4 need to find a concept for how to model the correlation factor properly.

Based on the discussion in this contribution we have observed following:

Observation 1: To model the interference level at the non-mobile adjacent system victim receiver, reasonable knowledge about the transmitter out-of-carrier and out-of-band characteristics could be on top of the antenna characteristics. Typically, the transmitter characteristics is gathered from information in the BS RF specification.

Observation 2: Information on relevant correlation factor as function of frequency offset is currently not captured in 3GPP technical reports.

Observation 3: RAN4 need to find appropriate method to determine the correlation roll-off profile. Potential methods are establishing profile based on measured characteristics from multiple vendors and/or simulation of PA characteristics.

To progress the work, we propose following:

Proposal 1: Capture array antenna model and corresponding description presented in [1] in TR 38.922, clause 7. A text proposal to [2] is attached at the end of this contribution.

Proposal 2: Within the scope of the SI continue to discuss how to model the correlation factor roll-off with the goal to find a representable model.

# References

[1]	R4-2404760, “An overview of the array antenna model”, Ericsson

[2]	R4-2406614, “TR skeleton for the SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson

TEXT PROPOSAL:

# 7	Additional information on AAS

<Editor’s note: We could capture in this sub-clause the background information used to answer ITU-R additional questions.>

## 7.1	Array antenna model

### 7.1.1	Overview

A parameterized array antenna model has been developed over time in 3GPP. The technical background and modelling aspects relevant for AAS base stations was originally described in TR 37.840, subclause 5.4.4. The model has been used for numerous studies in RAN4, including AAS, NR, HST, IAB, etc. The model has been adopted in other forums outside RAN4 and is also described in RAN1 in e.g., TR 36.897 and in ITU-R in recommendation M.2101. The extended model including sub-array structures is documented in TR 38.803, subclause 5.2.3.2.4.

The model is defined around a set of equations, which rely on a set of input parameters to describe the array antenna. At a high level the model can be described as shown in Figure 7.1.1-1.

# Figure 7.1.1-1: Array antenna model overview

Parameters can be divided into different categories:

- General parameters, which are parameters that will be required for the simulator (spatial angles, considered wavelength).

- Element parameters used to model the radiating elements.

- Sub-array parameters used to model the sub-array.

- Array parameters used to model the array.

The model will produce gain normalized radiation pattern for given q, j angles defined in the range 0≤θ≤180 degrees and -180≤φ≤180 degrees.

The wavelength, ld is related to the design of the array and is fixed by design, while the excitation wavelength, le may vary as function of considered frequency within a specific operating band.

### 7.1.2	Parameters

The input parameters required to describe the antenna is summarized and described in Table 7.1.2-1.

Table 7.1.2-1: Model input parameters

| Category | Parameter | Description | Note |
| --- | --- | --- | --- |
| General | ld | Design wavelength for array antenna in meters | The wavelength is fixed and selected for a given design frequency. The wavelength will not vary within a given operating band for a given design. |
|  | le | Array excitation wavelength in meters | This wavelength varies within the considered band. |
|  |  | Vertical angle in degrees |  |
|  |  | Horizontal angle in degrees |  |
|  | Ptx | Total conducted power in dBm | For a dual polarized antenna, the total conducted power is calculated over MxNx2 ports |
|  | mech | BS mechanical down-tilt angle in degrees | The mechanical down-tilt angle is a deployment parameter selected to maximize coverage within a given coverage area. The mechanical tilt can be applied as a coordinate transformation as described in TR 36.814, subclause A.2.1.6.2. |
| Element | Am | Element front-to-back ratio in dB |  |
|  | SLAv | Element side-lobe suppression in dB |  |
|  | 3dB | Element horizontal half power beamwidth in degrees |  |
|  | 3dB | Element vertical half power beamwidth in degrees |  |
|  | GE,max | Element peak gain in dBi | This parameter is related to the selection of 3dB, 3dB and LE, where GE,max=DE,max-LE and DE,max is related to 3dB, 3dB and array lattice unit area for the element. |
|  | LE | Element loss in dB |  |
| Sub-array | Msub | Number of element rows in sub-array |  |
|  | dv,sub | Vertical element separation in sub-array in meters |  |
|  | subtilt | Electrical pre-set sub-array down-tilt angle in degrees | The pre-set sub array down-tilt is a fixed design parameter for a base station. It is envisaged as a passive fixed (non-varying) electrical tilt within the sub-array elements. The deployment configuration (including mechanical tilt) for a base station is dependent on the environment. Thus, a same base station with fixed pre-set subarray tilt could also be used across different environments. The value of the pre-set tilt is determined based on the intended deployment scenarios and targeted user distribution and coverage range. |
| Array | M | Number of sub-array rows in array |  |
|  | N | Number of sub-array columns in array |  |
|  | dh | Horizontal sub-array separation in array in meters |  |
|  | dv | Vertical sub-array separation in array in meters | This parameter is related to Msub and dv,sub. |
|  | etilt | Electrical beam direction down-tilt angle in degrees |  |
|  | escan | Electrical beam direction scan angle in degrees |  |
|  | r | Array excitation correlation factor | For wanted signal ρ=1, while for unwanted emission the correlation falls of as function of frequency offset from carrier within the interval 0≤ρ<1. |

The total conducted power, Ptx at the Transceiver Array Boundary (TAB) is related to Total Radiated Power (TRP) in logarithmical scale as:

Ptx=10log102MN+Pn

TRP=Ptx-LE

, where Pn is the power per transmitter branch in dBm. Total conducted power is defined as the total power for all ports, including two orthogonal polarizations.

The equivalent isotropic radiated power (EIRP) is calculated in dBm using the model as:

EIRPθ,φ=Ptx+AAθ,φ

The peak EIRP can also be derived from parameters as:

EIRP0=Ptx+GE,max+10log10Msub+10log10MN

When co-existence is evaluated in RAN4, the focus is on the main beam pointing towards the UE, but for other compatibility scenarios, e.g., with other services, the focus is not necessarily just the main beam. Spatial regions outside the main beam may also be highly relevant. Since the sub-array topology will affect the radiating characteristics in the sidelobe region, the model needs to be extended to provide the ability to model the sidelobe region characteristics correctly with reasonable complexity.

For the case where single element array geometries is considered (Msub=1), the extended array model collapses to the original model described in TR 37.840.

The antenna element separation is an important parameter that must be selected with care. Obviously, the value is static for a specific base station design, whereas the compatibility analysis may need to cover an entire frequency band. Typically, the antenna is designed to support given element separations for the highest frequency to avoid grating lobes for lower frequencies, but other design principles can be used for base stations supporting multiple bands or very wide operating bands.

Some parameters, such as the element beamwidths are related to the peak element gain via the element loss. This means that these parameters must be selected carefully.

Considering base stations are optimized for various factors including performance, cost, and coverage, it is expected that sub array configurations are relevant for these bands as well as a set of physical antenna elements are combined to form a logical element. The model comprises of a basic element pattern which is then combined appropriately based on the equations to form the sub array pattern and the composite pattern. Since dual polarized elements are used in typical base stations, it is sufficient to model each polarization separately as considered in the specific model. The models are selected so that they are simple and representative to model base station performance with sufficient confidence. The element pattern is based on a simple gaussian beam which has a flat sidelobe level. The Gaussian pattern is sufficiently wide and cover most of the regions of interest, especially in the elevation domain. At high elevation angles, the flat sidelobe level is sufficient to model the side lobes of the antenna element which are significantly lower than the main beam. Thus, the proposed extended antenna model to model sub arrays is sufficient to model the beamforming capability of IMT base stations in considered frequency ranges.

Another aspect also to consider is the performance and coexistence simulator complexity. The antenna model provides is reasonable complex and produce a gain normalized radiation pattern. If parameters are selected properly, no additional directory normalization is required which will reduce complexity and save processing capacity and simulation time.

The antenna model has support to model the array response outside the wanted carrier bandwidth, within adjacent channels using the correlation factor, r. When the wanted signal is considered r is equal to 1. For unwanted emissions outside the wanted signal bandwidth values within the range 0≤ρ<1can be considered. Further details on the frequency response of r require further investigations.

### 7.1.3	Model equations

The array antenna model is built around pattern multiplication between the element radiation pattern, sub-array array factor radiation pattern and array factor radiation pattern, as described by equations listed in Table 7.1.3-1.

Table 7.1.3-1: Array antenna model equations

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλdsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλdcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλesinθetilt-n-1dhλecosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1 , where vm,n=expj2πm-1dvλdcosθ+n-1dhλdsinθsinφ |