3GPP TSG-RAN WG4 Meeting # 111 	R4-2408094

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement for AIoT device

Agenda Item:	10.13.3.2

Document for:	Approval

- Introduction

In this contribution, we provide our views on the RF requirement for AIoT device.

- Discussion

  - General

For the Tag, there are different device type, e.g., device1, device 2a, device 2b. The device 2b can transmit signal by itself which is similar to the NB-IoT, but the device 1/2a can only reflect the CW which is a new type UE in RAN4, so in the SI stage, we prefer focus on the requirement for such new UE.

Proposal 1: In SI, the RF requirement for device 1/2a can be prioritized.

Currently, the RF requirements for sub-6GHz are defined at antenna connector which is conducted power, but for the AIoT device, RAN4 should check whether it is still feasible to perform conducted conformance testing. For device 1/2a, the output power of Tag is based on retuning the load impedance. If the impedance of antenna mismatch with the load impedance, the Tag can reflect the power, so the antenna is a part of the procedure to show the performance of Tag. If we still use conducted testing, we are not for sure whether the testing still can get the meaningful results.

Observation 1: The conducted conformance testing may not feasible for AIoT device.

Proposal 2: RAN4 to discuss whether conducted conformance testing is still feasible for AIoT device. If not, it is suggested to define radiated requirement for device.

  - System parameters

- Operating band

This depends on operator request, but in SI stage, maybe we can take band n8 as the example band.

- Channel bandwidth

Similar to NR signal, the D2R can also define with channel bandwidth, transmission bandwidth configuration and guard band. Different channel bandwidth can be defined to match with different data rate.

- Minimum receiver bandwidth

This requirement is also mentioned in [1]. In RAN1 discussion, it is also assumed the receiver bandwidth is larger than signal bandwidth since the Tag may hard to be implemented with RF filter and it is hard to provide enough interference supersession, so in RAN4, it is better to discuss whether this requirement need to be defined.

  - Tx requirements

- Transmit output power

In [1], the MOP (maximum output power) is described as follow the regulation, and in [2], the MOP is defined as below:

The lower band is the frequency range from 865 MHz to 868 MHz which is based on the 320kHz bandwidth and upper band is from 915 MHz to 921 MHz which is based on the 640kHz bandwidth. To avoid define MOP for different bandwidth, it is better to define the MOP based on PSD limit.

As for the MPR/AMPR, since the Tag mainly work by changing the load impedance to reflect the CW and the ouput power is quite low, even though the device 2a is capable to have amplifier, we think the power level of non-linear product should not be significant, so such requirement may not be needed. As for the Pcmax, since the Tag power depends on CW, so this requirement seems also useless.

- Output power dynamic

The minimum output power is related to the power control to ensure UE does not cause significant interference when UE is near the BS, but the Tag output power depends on CW so this requirement seems to be not needed. Similarly, the requirement for power control also can be skipped. As for the transmit OFF power, the NR requirement can be the starting point for Tag which is -40 dBm.

For the time mask, in[1]，3 different type of time requirements are defined which can be the starting point

- Transmit signal quality

The frequency accuracy is defined in [1]:

The tolerance depends on BLF and DR which is different from the traditional frequency error in RAN4. The EVM, IBE are also needed for device, but the details need more discussion.

- Output RF spectrum emissions

For the SEM and SE, the unwanted emission for RFID in [2] can be starting point.

The definition of occupied bandwidth can be same as NR and the ACLR depends on co-existence study.

  - Rx requirements

For the Rx requirement, the critical issue is which metric can replace throughput for device. In co-existence discussion, BLER is an option to represent the performance of Tag, which may also can be used for Rx requirement definition. In [2], for the Reader, 50% success rate is used as metric for the sensitivity design, we prefer use same metric for the whole AIoT system.

Proposal 3: It is suggested that success rate is used as performance metric for Rx requirement definition for AIoT device.

Based on the analysis above, we have following proposal:

Proposal 4: The following table can be the starting point for AIoT device RF requirement discussion:

| RF Requirement for AIoT device |  |  |  |
| --- | --- | --- | --- |
| System parameter | Operating band |  | Band n8 as example band in SI |
|  | Channel bandwidth | Transmission bandwidth configuration | 180 kHz, FFS larger bandwidth for higher data rate |
|  |  | Guard band | FFS |
|  | Minimum receiver bandwidth |  | RAN4 need to discuss whether this requirement is need considering the feasibility of RF filter |
| Tx requirement | Transmit output power | Maximum output power | -25 dBm/100kHz for frequency <900MHz; -18 dBm/100kHz for frequency >900MHz; FFS whether/how to convert radiated requirement to conducted requirement |
|  | Output power dynamic | Transmit OFF power | Same as NR, -40 dBm |
|  |  | Transmit time mask | FFS following requirement: Transmit-to-Receive Turn-Around Time; Receive-to-Transmit Turn-Around Time; Transmit Power-On Ramp; |
|  | Transmit signal quality | Frequency error | FFS |
|  |  | EVM | FFS, ASK/PSK are considered |
|  |  | IBE | FFS |
|  | Output RF spectrum emissions | Occupied bandwidth | Same as NR |
|  |  | SEM | Unwanted emission in [2] as starting point |
|  |  | ACLR | Depends on co-existence study |
|  |  | SE | Unwanted emission in [2] as starting point |
| Rx requirement | RAN4 should first discuss and decide what metric will be used to represent the Rx performance, e.g., BLER. |  |  |

- Conclusion

In this contribution, we provide our views on the RF requirement for AIoT device.

Proposal 1: In SI, the RF requirement for device 1/2a can be prioritized.

Observation 1: The conducted conformance testing may not feasible for AIoT device.

Proposal 2: RAN4 to discuss whether conducted conformance testing is still feasible for AIoT device. If not, it is suggested to define radiated requirement for device.

Proposal 3: It is suggested that success rate is used as performance metric for Rx requirement definition for AIoT device.

Proposal 4: The following table can be the starting point for AIoT device RF requirement discussion:

| RF Requirement for AIoT device |  |  |  |
| --- | --- | --- | --- |
| System parameter | Operating band |  | Band n8 as example band in SI |
|  | Channel bandwidth | Transmission bandwidth configuration | 180 kHz, FFS larger bandwidth for higher data rate |
|  |  | Guard band | FFS |
|  | Minimum receiver bandwidth |  | RAN4 need to discuss whether this requirement is need considering the feasibility of RF filter |
| Tx requirement | Transmit output power | Maximum output power | -25 dBm/100kHz for frequency <900MHz; -18 dBm/100kHz for frequency >900MHz; FFS whether/how to convert radiated requirement to conducted requirement |
|  | Output power dynamic | Transmit OFF power | Same as NR, -40 dBm |
|  |  | Transmit time mask | FFS following requirement: Transmit-to-Receive Turn-Around Time; Receive-to-Transmit Turn-Around Time; Transmit Power-On Ramp; |
|  | Transmit signal quality | Frequency error | FFS |
|  |  | EVM | FFS, ASK/PSK are considered |
|  |  | IBE | FFS |
|  | Output RF spectrum emissions | Occupied bandwidth | Same as NR |
|  |  | SEM | Unwanted emission in [2] as starting point |
|  |  | ACLR | Depends on co-existence study |
|  |  | SE | Unwanted emission in [2] as starting point |
| Rx requirement | RAN4 should first discuss and decide what metric will be used to represent the Rx performance, e.g., BLER, success rate, etc. |  |  |

- References

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard

- ETSI EN 302 208 V3.4.1