# 3GPP TSG-RAN WG4 Meeting #111	R4-2408125

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	vivo

Title: 	Simulation results of MIMO layer evaluation for 6Rx UE

Agenda Item:	10.1.1.4.2

Document for:	Approval

Introduction

In the last meeting, we started the discussion on requirements for 6Rx and the WFs were captured in [1]. For 6Rx, 4 MIMO layer is confirmed, while it needs further study whether or not to support 6MIMO layer. In this contribution, we give some initial simulation results for the comparisons between 4 and 6 MIMO layers, based on that, our views on 6 MIMO layer support are provided.

Discussion

For MIMO layer support for 6Rx, we had following WF in the last meeting:

Way forward: Further discuss number of MIMO layers support for FWA devices and handhelds considering the following aspects:

-	antenna correlation analysis based on form factor constraints

-	performance simulation in comparison with 4 MIMO layer considering possible implementation constraints

We conducted performance simulation for 4&6 MIMO layer and the detailed simulation assumptions were listed in the Annex.

Simulation assumptions

TDL & CDL

TDL (Tap Delay Line) and CDL (Clustered Delay Line) are two typical types of channel models used to simulate the behaviour of wireless communication channels. Since RAN4 did not have unified simulation assumptions for MIMO layer support, both TDL and CDL channel models are used in our simulations. In general, TDL is the model of taps with different delays (therefore the name Tapped Delay Line), e.g., based on Jakes model, where each tap is modelled as a random variable. CDL is a generalization that taps are replaced by clusters for the interest of spatial characteristics. This is the reason why CDL is more commonly used for MIMO evolution in RAN1. The TDL models are simpler and it was used in MIMO layer evolution by some company in RAN4. In the annex part, we give the simulation assumptions for both TDL and CDL.

UE antenna correlation

Antenna correlation is an important aspect for designing MIMO antenna systems. With lower antenna correlation, it indicates better MIMO performance that can be achieved with the underlying antenna system. The antenna correlation coefficient is given as the range of 0.1~0.5 between any two antennas, which is reasonable considering the actual antenna performances. The UE antenna parameter is given in Table 1 in the annex. Both antenna correlation coefficient 0.1 and 0.5 are simulated for TDL channel model.

Simulation results

### Simulation results based on TDL channel model

#### Rx correlation=0.1

| a) Fixed rank with MCS13 | b) Fixed rank with MCS18 |
| --- | --- |
| c) Fixed rank with MCS22 | d) MCS and RI adaption |

#### Rx correlation=0.5

| a) Fixed rank with MCS13 | b) Fixed rank with MCS18 |
| --- | --- |
| c) Fixed rank with MCS18 | d) MCS and RI adaption |

From the simulation results, based on TDL channel model, it’s observed that the throughput gain can be obtained by 6 MIMO layer under high SNR condition with fixed MCS and Rank. However, when MCS and Rank adaption is enabled, there is no gain of 6 MIMO layers compared with 4 MIMO layer, since higher MCS could be scheduled and correctly demodulated in the case of 4 MIMO layer, while may not be demodulated correctly in the case of 6 MIMO layers. Moreover, for different Rx correlations, i.e., 0.1 and 0.5 as assumed in simulation, the simulation results are similar. The main difference between them is that higher Rx correlation requires higher SNR to achieve the same throughput.

Observation 1: Under high SNR condition, 6 MIMO layer has gain over 4 MIMO layer with fixed MCS and Rank.

Observation 2: When enabling MCS and Rank adaption, 6 MIMO layer has no gain over 4 MIMO layer.

Observation 3: For different Rx correlation, it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers. With poor Rx correlation, it requires better SNR conditions when 6 MIMO layer outperforms 4MIMO layer.

### Simulation results based on CDL channel model

| a) Fixed rank with MCS13 | b) Fixed rank with MCS18 |
| --- | --- |
| c) Fixed rank with MCS18 | d) MCS and RI adaption |

As shown in the above simulation results based on CDL channel model, 6 MIMO layer could provide throughput gain under high SNR condition when MCS is relatively small, e.g., MCS13 and MCS18. However, 6 MIMO layer can’t provide throughput gain with relatively higher MCS, e.g., MCS22. Besides, when enabling MCS and Rank adaption, 4 MIMO layer even outperforms 6 MIMO layer. Compared the results of CDL channel model, we can observe that it is more difficult to achieve the gain of 6 MIMO layer under CDL channel model based on the same other conditions. This is because CDL channel model is based on several clusters with specific angle modelled including AOD, AOA, ZOD and ZOA, which would lead to limited spatial DoF (degrees of freedom), compared with TDL channel model with more spatial DoF to support 6 MIMO layers.

Observation 4: Compared with TDL, it is more challenging to achieve gain for 6 MIMO layer under CDL channel model.

Observation 5: Under high SNR and relatively small MCS (e.g., MCS 13/18), it shows gain for 6 MIMO layer with fixed MCS and Rank.

Observation 6: When enabling MCS and Rank adaption, 4 MIMO layer outperforms 6 MIMO layer.

In the practical deployment, to adapt the dynamically changed traffic and channel state, it is more feasible to enable MCS and Rank adaption. Additionally, CDL channel model is a more appropriate to evaluate the support of 6 MIMO layers, which is highly correlated with spatial characteristic and precoding performance. From these perspectives, no matter based on TDL or CDL channel model, there is no benefit to support 6 MIMO layers according to our simulation results.

Observation 7: In the practical deployment, it is more feasible to enable MCS and Rank adaption to adapt the dynamically changed traffic and channel state.

Observation 8: It is more appropriate to evaluate the support of 6 MIMO layers based on the CDL channel model with its specific spatial DoF.

Proposal 1: At least for handheld UE, do not support 6 MIMO layer for 6Rx.

Conclusion

This contribution discusses 6 MIMO-layer support for 6Rx. The following observations and proposal are made:

Observation 1: Under high SNR condition, 6 MIMO layer has gain over 4 MIMO layer with fixed MCS and Rank.

Observation 2: When enabling MCS and Rank adaption, 6 MIMO layer has no gain over 4 MIMO layer.

Observation 3: For different Rx correlation, it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers. With poor Rx correlation, it requires better SNR conditions when 6 MIMO layer outperforms 4MIMO layer.

Observation 4: Compared with TDL, it is more challenging to achieve gain for 6 MIMO layer under CDL channel model.

Observation 5: Under high SNR and relatively small MCS (e.g., MCS 13/18), it shows gain for 6 MIMO layer with fixed MCS and Rank.

Observation 6: When enabling MCS and Rank adaption, 4 MIMO layer outperforms 6 MIMO layer.

Observation 7: In the practical deployment, it is more feasible to enable MCS and Rank adaption to adapt the dynamically changed traffic and channel state.

Observation 8: It is more appropriate to evaluate the support of 6 MIMO layer based on the CDL channel model with its specific spatial DoF.

Proposal 1: At least for handheld UE, do not support 6 MIMO layer for 6Rx.

- References

[1] R4-2406585, WF on requirements for 6Rx, AT&T, RAN4#110-bis.

# Annex

Table 1 Simulation Assumption for TDL

| Parameters | Value |
| --- | --- |
| Duplexing | TDD |
| Carrier frequency | 3.5GHz |
| Subcarrier spacing | 30kHz |
| Propagation condition | TDL-A with DS=100 in TR 38.901 |
| gNB antenna parameter | 8Tx with ideal non-correlation RBS RBS=11111111 |
| UE antenna parameter | 6Rx with correlation RUE with β=0.1/0.5, RUE=1ββββββ1ββββββ1ββββββ1ββββββ1ββββββ1 -2dB power offset for last two Rx |
| Digital precoding | Type I codebook, PRG=2 |
| TRS periodicity | 40ms, 2 slot TRS |
| CSI-RS periodicity | 40ms |
| DMRS type | Type 2 |
| Number of DMRS symbols | [3,11] |
| PDSCH mapping | Type A, Starting symbol 2, Duration 12 |
| Bandwidth | 52RB |
| MCS | MCS13/18/22, or MCS adaptation (up to 256QAM) |
| Rank | Fixed rank 4/6, or RI adaptation |
| UE speed | 3km/h |
| Receiving equalizer | MMSE |

Table 2 Simulation Assumption for CDL

| Parameters | Value |
| --- | --- |
| Duplexing | TDD |
| Carrier frequency | 3.5GHz |
| Subcarrier spacing | 30kHz |
| Propagation condition | CDL-C with DS=100 in TR 38.901 |
| gNB antenna parameter | 32Tx: (M, N, P, Mg, Ng, Mp, Np) = (4,4,2,1,1,4,4) |
| UE antenna parameter | 6Rx: (M, N, P, Mg, Ng, Mp, Np) = (1,3,2,1,1,1,3), -2dB power offset for last two Rx |
| Digital precoding | Type I codebook, PRG=2 |
| TRS periodicity | 40ms, 2 slot TRS |
| CSI-RS periodicity | 40ms |
| DMRS type | Type 2 |
| Number of DMRS symbols | [3,11] |
| PDSCH mapping | Type A, Starting symbol 2, Duration 12 |
| Bandwidth | 52RB |
| MCS | MCS13/18/22, or MCS adaptation (up to 256QAM) |
| Rank | Fixed rank 4/6, or RI adaptation |
| UE speed | 3km/h |
| Receiving equalizer | MMSE |