3GPP TSG-RAN WG4 Meeting #111 	R4-2408153

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	[LTE_NR_DC_enh2-Core] Alignment of RAN4 requirements with RAN2 procedures

Agenda item:	4.4

Document for:	Approval

Introduction

This topic was also discussed shortly in RAN4#109 and RAN4#110 meetings. However, as there was not much time to discuss legacy maintenance no conclusion was reached. This discussion paper is mostly a resubmission of earlier papers. Notice, that we’re proposing that RAN4 solves the misalignment between RAN4 and RAN2 by either of the following two options:

- Proposal 1: Update the RAN4 UE requirements capturing that tci-ActivatedConfig can be configured for a deactivated SCell and direct activated SCell.

- Proposal 2: If proposal 1 is not agreeable, send LS to RAN2 clarifying the RAN2 understanding of the applicability of tci-ActivatedConfig.

RAN4 has been discussing the requirements related to SCell activation using tci-ActivatedConfig for a couple of meetings.

The issue at hand is that tci-ActivatedConfig has not been accounted fully into the RAN4 requirements. Exactly how to capture such requirements and when the tci-ActivatedConfig is applicable was left open.

In this paper we discuss the applicability issue of tci-ActivatedConfig and in our CR [9] we have a proposal how to capture the requirements.

As alternative we have also provided a drat LS to RAN2 in the appendix.

Main reason for the clarification is to align RAN2 and RAN4 and for the network being aware of the expected UE delay for the activation discussed.

Discussion

tci-ActivatedConfig

tci-ActivatedConfig is introduced in Rel-17 by RAN2. tci-ActivatedConfig may be configured together with ServingCellConfig [2]:

#### –	ServingCellConfig

The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add.

The detailed parameter description [2]:

where

Where:

More specifically, looking at the parts that would be of interest for defining the UE requirements:

tci-ActivatedConfig

- If configured for an SCell, or

- if configured for the PSCell when the SCG is being activated upon the reception of the containing message,

  - the UE shall consider the TCI states provided in this field as the activated TCI states for PDCCH/PDSCH reception on this serving cell.

- If configured for the PSCell when the SCG is indicated as deactivated in the containing message:

- the UE shall consider the TCI states provided in this field as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the TCI states provided in this field for PDCCH as RS for RLM, respectively for BFD.

When this field is absent for the PSCell and the SCG is being deactivated:

- the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.

tci-ActivatedConfig applicability

As we see from the RAN2 description, the tci-ActvatedConfig can be configured for:

- an SCell, or

- a PSCell when the SCG is activated upon configuration.

From the RAN2 description it is apparent that the tci-ActvatedConfig can be configured for an SCell unconditionally while for a PSCell tci-ActvatedConfig can be configured only when the PSCell is activated upon configuration. Based on the current RAN2 specification we conclude following:

tci-ActivatedConfig can be configured for any SCell unconditionally.

tci-ActvatedConfig ActvatedConfig can only be configured for a PSCell which is activated when configured.

Hence, from the current RAN2 wording, tci-ActivatedConfig can be configured for an SCell no matter if the SCell is activated not activated when configured. The tci-ActivatedConfig can be configured for a deactivated SCell as well as for an SCell being configured as activated upon configuration (direct activated).

tci-ActivatedConfig can be configured for a deactivated SCell and a direct activated SCell.

Currently, the RAN4 specifications are not clear about the UE requirements when tci-ActivatedConfig is used. In [7] we have provided a CR capturing the UE requirements accounting tci-ActivatedConfig.

In earlier RAN4#108 meeting views were diverse on how the RAN2 specification was to be interpreted and the proposed changes in the CR was not agreed. Hence, our other proposal was to ask RAN2 to clarify the relevant part of the specification quoted above and clarify RAN4 specification accordingly – if needed.

In the appendix we have provided a draft LS TP as follows:

RAN4 has been discussing the requirements related to PSCell and SCell activation when the PSCell or SCell configuration includes tci-ActivatedConfig.

RAN4 would like to ask RAN2 if the tci-ActivatedConfig can be configured for an SCell which is not activated upon configuration (hence, the SCell is configured in deactivated state)?

Update the RAN4 UE requirements capturing that tci-ActivatedConfig can be configured for a deactivated SCell and a direct activated SCell.

If proposal 1 is not agreeable, send LS to RAN2 clarifying the RAN2 understanding of the applicability of tci-ActivatedConfig.

Conclusion

In this paper we discussed the applicability issue of tci-ActivatdConfig and observe:

- tci-ActivatedConfig can be configured for any SCell unconditionally.

tci-ActvatedConfig can only be configured for a PSCell which is activated when configured.

tci-ActivatedConfig can be configured for a deactivated SCell and a direct activated SCell.

We propose either of:

- Update the RAN4 UE requirements capturing that tci-ActivatedConfig can be configured for a deactivated SCell and a direct activated SCell.

If proposal 1 is not agreeable, send LS to RAN2 clarifying the RAN2 understanding of the applicability of tci-ActivatedConfig.

In CR [9] we have a proposal how to capture the requirements and we have provided a Draft LS in the appendix.

- References

- R4-2313357, LTE_NR_DC_enh2-Core CR correcting TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell.

- 38.331.

- R4-2313356, LTE_NR_DC_enh2-Core Alignment of RAN4 requirements with RAN2 procedures, Nokia, Nokia Shanghai Bell.

- R4-2320284, LTE_NR_DC_enh2-Core CR correcting TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell.

- R4-2320286, Draft LS on TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell

- R4-2320283, LTE_NR_DC_enh2-Core Alignment of RAN4 requirements with RAN2 procedures, Nokia, Nokia Shanghai Bell

- R4-2401727, LTE_NR_DC_enh2-Core CR correcting TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell.

- R4-2401726, (LTE_NR_DC_enh2-Core) Discussion on correcting TCI state activation command at SCell activation, Nokia

- R4-2408154, [LTE_NR_DC_enh2-Core] CR on alignment of RAN4 requirements with RAN2 procedures, Nokia

Appendix

3GPP TSG RAN WG 4 Meeting #111		TDoc R4-24xxxxx

Fukuoka, Japan, May 20th – May  24th, 2024

Title:	LS on alignment of RAN4 requirements with RAN2 procedures

Response to:

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Lars Dalsgaard

lars.dalsgaard@nokia.com

Attachments:	None

# 1	Overall description

RAN4 has been discussing the requirements related to PSCell and SCell activation when the PSCell or SCell configuration includes tci-ActivatedConfig.

RAN4 would like to ask RAN2 if the tci-ActivatedConfig can be configured for an SCell which is not activated upon configuration (hence, the SCell is configured in deactivated state)?

# 2	Actions

To RAN2

ACTION: 	RAN4 asks RAN2 to provide guidelines on whether the tci-ActivatedConfig can be configured for an SCell which is not activated upon configuration

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN4 Meeting #112	19 August – 23 August, 2024	Maastricht, Netherlands

TSG-RAN4 Meeting #112bis	14 October – 18 October, 2024	TBC, China