3GPP TSG-WG4 Meeting #111	R4-2408195

# Fukuoka, Japan, 20 - 24 May, 2024

Source: 	NEC

Title: 	Discussion on extreme conditions for SAN

Agenda Item:		7.16.4	SAN RF conformance testing requirements 	[NR_NTN_enh-Perf]

Document for:	discussion

Introduction

This contribution discusses the issues of the extreme testing conditions for SAN and provides proposals to solve the issues.

Discussion

In the previous meeting, extreme testing conditions for SAN were discussed and the following Way Forwards were agreed in [1].

Extreme testing conditions for NTN SAN (Ka band)

Way forward 1: Consider the following options (with other not precluded) for SAN extreme testing conditions decision next meeting:

- Option 1: Do not consider extreme conditions testing;

- Option 2: Consider to introduce new dedicated set of optional manufacturer declarations in TS 38.181, table 4.6-1, and Annex B as below (Track Changes):

Way forward 2: If Option 2 in Way Forward 1 is agreed, agree that extreme conditions testing for NTN SAN is a in band-agnostic topic, i.e. not to re-open this discussion for new NTN frequency range or band(s), e.g. Ku, etc.

The fundamental question is whether we need optional manufacturer declarations for extreme testing conditions?

We believe we do not need declarations for extreme testing conditions because of the following reasons.

- There are no core requirements with extreme conditions.

  - We have agreed not to have extreme condition for radiated transmit power in WF [2].

  - We have agreed to remove text on radiated transmit power requirements in the extreme condition in the dart CR to TS 38.108 [3].

- There are no conformance requirements with extreme conditions.

- The meaning of the “optional” declaration is not clear.

  - In our understanding, manufacturers have freedom to declare anything they want. We wander if the proposed table with optional declaration is approved then only listed items could be declared. It may implicitly say items not in the list could not be declared. We do not think that is what we want.

Therefore, we support option 1 in Way Forward 1.

Proposal 1:

Not to consider extreme conditions testing

It should be noted that Way forward 2 is applied only when option 2 in Way Forward 1 is agreed. As we support option 1 in Way Forward 1, Way forward 2 does not make sense.

To avoid confusion on extreme testing conditions and to clarify extreme conditions are not required, we propose to remove text on extreme conditions from TS 38.181.

Proposal 2:

To remove text on extreme conditions from TS 38.181

To remove text on extreme conditions from TS 38.181, accompanying CR has been submitted [4].

Conclusions

We discussed the issue of the requirement set applicability tables in TS 38.106 and provided the following proposals.

Proposal 1:

Not to consider extreme conditions testing

Proposal 2:

To remove text on extreme conditions from TS 38.181

References

[1] R4-2406002, “Way Forward for [110bis][306] NR_NTN_enh_Part2 Extreme conditions and MU”, Huawei, HiSilicon

[2] R4-2302877, “WF for SAN RF requirements on above 10 GHz”, Moderator (THALES)

[3] R4-2320153, “Draft CR on TS 38.108: Radiated transmit power requirements in extreme conditions”, NEC

[4] R4-24xxxxx, “CR to TS 38.181 removal of extreme conditions”, NEC