3GPP TSG-RAN WG4 Meeting #111	R4-2408244

Fukuoka City, Fukuoka , Japan, 20th–24th May, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR

Agenda Item:	7.5.2

Document for:	Discussion

- Introduction

In the #RAN4 110bis meeting, the efforts mainly pay for the UE feature discussion. While for the maintenance core part, no progress was achieved. In this paper, we further share our views on the remaining issues.

- Discussion

- 2.1 DRX

Misalignment between DRX-on duration and SMTC for NFG measurements

DRX is an important technology in NR to achieve power saving, thus DRX based interruption requirement is worthy to be discussed. In last meeting, the following options were proposed:

- Option 1: UE measures only within SMTC occasions once every DRX cycle and no interruption is allowed. - Option 1a: interruption can be allowed during the time extended due to drx-inactivityTimer. - Option 2: UE measures within SMTC occasions when DRX cycle is larger than 320ms and no interruption is allowed; but when DRX cycle is equal or smaller than 320ms interruption is allowed once every DRX cycle according to Tcycle, i. - Option 2a: 160ms instead of 320ms. And interruption can be allowed during the time extended due to drx-inactivityTimer. - Option 3: UE measurements can affect DRX-on duration. Interruption is allowed once every DRX cycle, and it is according to Tcycle,i. - Option 5: Interruptions may be allowed during DRX ON duration under the following conditions: - if DRX ON duration is smaller or equal to [5 or 10] ms and DRX cycle is larger than [40 or 80ms], - Interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer - Otherwise - interruptions are allowed, except for the last slot containing PDCCH in the DRX ON duration, excluding the time extended due to drx-inactivityTimer - Option 6: Interruptions may be allowed during DRX ON duration under the following conditions: - if DRX ON duration is smaller or equal to [5 or 10] ms and DRX cycle is larger than [40 or 80ms], - Interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer - Otherwise - interruptions are allowed

To our understand, If DRX cycle is larger than 320ms, a DRX cycle would have multiple SMTC occasions. The UE can select one SMTC occasion within DRX OFF duration to perform measurement on, which can reduce interruption ratio with the cost of power consumption.

If DRX cycle is smaller than 320ms, since SMTC is cell specific and DRX is UE specific, it is possible that the configured SMTC occasions are completely misaligned with DRX ON duration. In this case, the UE only can perform measurement on the DRX OFF duration, which can not cause any interruption. If the SMTC occasion is partially overlapping with DRX ON duration, the UE performs measurement during DRX OFF duration as much as possible, similar to legacy DRX based measurement requirement.

While considering the UE vendors and NW vendors shared different view, so as to move forward, whether and how to allow the interruption, the options can be categorized into the following:

|  |  | Small DRX cycle | Large DRX cycle |
| --- | --- | --- | --- |
| Option 1 | Small DRX ON | No interruption | No interruption |
|  | Large DRX ON | No interruption | No interruption |
| Option 1a | Small DRX ON | No interruption during DRX ON | No interruption during DRX ON |
|  | Large DRX ON | No interruption during DRX ON | No interruption during DRX ON |
| Option 2 | Small DRX ON | Interruption allowed once per Tcycle | No interruption |
|  | Large DRX ON | Interruption allowed once per Tcycle | No interruption |
| Option 2a | Small DRX ON | Interruption allowed once per Tcycle | No interruption during DRX ON |
|  | Large DRX ON | Interruption allowed once per Tcycle | No interruption during DRX ON |
| Option 3 | Small DRX ON | Interruption allowed once per Tcycle | Interruption allowed once per Tcycle |
|  | Large DRX ON | Interruption allowed once per Tcycle | Interruption allowed once per Tcycle |
| Option 5 | Small DRX ON | No interruption on last PDCCH slot of DRX ON | No interruption during DRX ON |
|  | Large DRX ON | No interruption on last PDCCH slot of DRX ON | No interruption on last PDCCH slot of DRX ON |
| Option 6 | Small DRX ON | Interruption allowed once per Tcycle | No interruption during DRX ON |
|  | Large DRX ON | Interruption allowed once per Tcycle | Interruption allowed once per Tcycle |

We prefer to down-select within all the above options, as a compromise, Option 1a, 2a, 5 and 6 are suggested by us.

Proposal 1: To move forward, Option 1a, 2a, 5 and 6 are preferred.

|  |  | Small DRX cycle | Large DRX cycle |
| --- | --- | --- | --- |
| Option 1a | Small DRX ON | No interruption during DRX ON | No interruption during DRX ON |
|  | Large DRX ON | No interruption during DRX ON | No interruption during DRX ON |
| Option 2a | Small DRX ON | Interruption allowed once per Tcycle | No interruption during DRX ON |
|  | Large DRX ON | Interruption allowed once per Tcycle | No interruption during DRX ON |
| Option 5 | Small DRX ON | No interruption on last PDCCH slot of DRX ON | No interruption during DRX ON |
|  | Large DRX ON | No interruption on last PDCCH slot of DRX ON | No interruption on last PDCCH slot of DRX ON |
| Option 6 | Small DRX ON | Interruption allowed once per Tcycle | No interruption during DRX ON |
|  | Large DRX ON | Interruption allowed once per Tcycle | Interruption allowed once per Tcycle |

- 2.2 Others

- Whether allow to enable R17 and R18 UE capabilities

Regarding this issue, some discussion were held in previous meeting and the following progress achieved in [1]:

Issue 1-3-2: 1-to-1 mapping between NeedForGaps and NCSG capabilities - Agreements - No need to establish the mapping between UE’s indication for NeedForGaps and NCSG.

During online discussion in last meeting,the issue of whether allow to enable both R17 NCSG and R18 NeedForGaps capabilities was also touched. Some company believe only enable one of the two UE capabilities is enough, we have different view.

Firstly from the signalling perspective, R17 NCSG capability includes semi-static UE capability nr-NeedForGapNCSG-Reporting-r17 and dynamic reporting NeedForGapNCSG-InfoNR-r17, whether UE  dynamically reports NeedForGapNCSG-InfoNR-r17, depends on NW request NeedForGapNCSG-ConfigNR. We listed all these R17 NCSG signallings defined in [2] as below:

UE capability(semi-staitic): nr-NeedForGapNCSG-Reporting-r17 nr-NeedForGapNCSG-Reporting-r17 ENUMERATED {supported} OPTIONAL,

UE reporting(dynamic): NeedForGapNCSG-InfoNR-r17 NeedForGapNCSG-InfoNR-r17 ::= SEQUENCE { intraFreq-needForNCSG-r17 NeedForNCSG-IntraFreqList-r17, interFreq-needForNCSG-r17 NeedForNCSG-BandListNR-r17 } NeedForNCSG-IntraFreqList-r17 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForNCSG-IntraFreq-r17 NeedForNCSG-BandListNR-r17 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForNCSG-NR-r17 NeedForNCSG-IntraFreq-r17 ::= SEQUENCE { servCellId-r17 ServCellIndex, gapIndicationIntra-r17 ENUMERATED {gap, ncsg, nogap-noncsg} } NeedForNCSG-NR-r17 ::= SEQUENCE { bandNR-r17 FreqBandIndicatorNR, gapIndication-r17 ENUMERATED {gap, ncsg, nogap-noncsg} }

NW request: needForGapNCSG-ConfigNR-r17 RRCReconfiguration-v1700-IEs ::= SEQUENCE { otherConfig-v1700 OtherConfig-v1700 OPTIONAL, -- Need M sl-L2RelayUE-Config-r17 SetupRelease { SL-L2RelayUE-Config-r17 } OPTIONAL, -- Need M sl-L2RemoteUE-Config-r17 SetupRelease { SL-L2RemoteUE-Config-r17 } OPTIONAL, -- Need M dedicatedPagingDelivery-r17 OCTET STRING (CONTAINING Paging) OPTIONAL, -- Cond PagingRelay needForGapNCSG-ConfigNR-r17 SetupRelease {NeedForGapNCSG-ConfigNR-r17} OPTIONAL, -- Need M needForGapNCSG-ConfigEUTRA-r17 SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17} OPTIONAL, -- Need M musim-GapConfig-r17 SetupRelease {MUSIM-GapConfig-r17} OPTIONAL, -- Need M ul-GapFR2-Config-r17 SetupRelease { UL-GapFR2-Config-r17 } OPTIONAL, -- Need M scg-State-r17 ENUMERATED { deactivated } OPTIONAL, -- Need N appLayerMeasConfig-r17 AppLayerMeasConfig-r17 OPTIONAL, -- Need M ue-TxTEG-RequestUL-TDOA-Config-r17 SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17} OPTIONAL, -- Need M nonCriticalExtension RRCReconfiguration-v1800-IEs OPTIONAL }

The dynamic UE reporting NeedForGapNCSG-InfoNR-r17 can be per band per BC level. So it is possible that UE report different status for different bands.

Regarding the R18 signalling, it is on top of R16 NFG, an additional interruption indication can be dynamic reported for the R16 and R18 capable UE. Both the NeedForGapsInfoNR-r16 and NeedForInterruptionInfoNR-r18 are dynamically reported by the UE if NW requests. The two reporting are both per band per BC level. We listed all the R16 and R18 signallings as below.

R16 NFG:

nr-NeedForGap-Reporting-r16 Indicates whether the UE supports reporting the measurement gap requirement information for NR target in the UE response to a network configuration RRC message. - nr-NeedForGap-Reporting-r16 ENUMERATED {supported} OPTIONAL

needForGapsConfigNR - Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message. RRCReconfiguration-v1610-IEs ::= SEQUENCE { otherConfig-v1610 OtherConfig-v1610 OPTIONAL, -- Need M bap-Config-r16 SetupRelease { BAP-Config-r16 } OPTIONAL, -- Need M iab-IP-AddressConfigurationList-r16 IAB-IP-AddressConfigurationList-r16 OPTIONAL, -- Need M conditionalReconfiguration-r16 ConditionalReconfiguration-r16 OPTIONAL, -- Need M daps-SourceRelease-r16 ENUMERATED{true} OPTIONAL, -- Need N t316-r16 SetupRelease {T316-r16} OPTIONAL, -- Need M needForGapsConfigNR-r16 SetupRelease {NeedForGapsConfigNR-r16} OPTIONAL, -- Need M onDemandSIB-Request-r16 SetupRelease { OnDemandSIB-Request-r16 } OPTIONAL, -- Need M dedicatedPosSysInfoDelivery-r16 OCTET STRING (CONTAINING PosSystemInformation-r16-IEs) OPTIONAL, -- Need N sl-ConfigDedicatedNR-r16 SetupRelease {SL-ConfigDedicatedNR-r16} OPTIONAL, -- Need M sl-ConfigDedicatedEUTRA-Info-r16 SetupRelease {SL-ConfigDedicatedEUTRA-Info-r16} OPTIONAL, -- Need M targetCellSMTC-SCG-r16 SSB-MTC OPTIONAL, -- Need S nonCriticalExtension RRCReconfiguration-v1700-IEs OPTIONAL }

UE dynamic reporting: NeedForGapsInfoNR-r16 ::= SEQUENCE { intraFreq-needForGap-r16 NeedForGapsIntraFreqList-r16, interFreq-needForGap-r16 NeedForGapsBandListNR-r16 } NeedForGapsIntraFreqList-r16 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 NeedForGapsBandListNR-r16 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 NeedForGapsIntraFreq-r16 ::= SEQUENCE { servCellId-r16 ServCellIndex, gapIndicationIntra-r16 ENUMERATED {gap, no-gap} } NeedForGapsNR-r16 ::= SEQUENCE { bandNR-r16 FreqBandIndicatorNR, gapIndication-r16 ENUMERATED {gap, no-gap} }

R18 NFInerruption:

nr-NeedForInterruptionReport-r18 ENUMERATED {supported} OPTIONAL,

needForInterruptionConfigNR Indicates whether the UE shall report interruption requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message. The network sets this field to enabled only if the needForGapsConfigNR is configured. The network sets this field to disabled if the needForGapsConfigNR is released. RRCReconfiguration-v1800-IEs ::= SEQUENCE { needForInterruptionConfigNR-r18 ENUMERATED { enabled, disabled } OPTIONAL, -- Need M uav-Config-r18 SetupRelease { UAV-Config-r18 } OPTIONAL, -- Need M sl-IndirectPathAddChange-r18 SetupRelease { SL-IndirectPathAddChange-r18 } OPTIONAL, -- Need M n3c-IndirectPathAddChange-r18 SetupRelease { N3C-IndirectPathAddChange-r18 } OPTIONAL, -- Need M n3c-IndirectPathConfigRelay-r18 SetupRelease { N3C-IndirectPathConfigRelay-r18 } OPTIONAL, -- Need M otherConfig-v1800 OtherConfig-v1800 OPTIONAL, -- Need M srs-PosResourceSetLinkedForAggBWList-r18 SetupRelease { SRS-PosResourceSetLinkedForAggBWList-r18 } OPTIONAL,-- Need M ltm-Config-r18 SetupRelease {LTM-Config-r18} OPTIONAL, -- Need M nonCriticalExtension SEQUENCE {} OPTIONAL }

UE dynamic reporting: NeedForInterruptionInfoNR-r18 ::= SEQUENCE { intraFreq-needForInterruption-r18 NeedForInterruptionIntraFreqList-r18, interFreq-needForInterruption-r18 NeedForInterruptionBandListNR-r18 } NeedForInterruptionIntraFreqList-r18 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForInterruptionNR-r18 NeedForInterruptionBandListNR-r18 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18 NeedForInterruptionNR-r18 ::= SEQUENCE { interruptionIndication-r18 ENUMERATED {no-gap-with-interruption, no-gap-no-interruption} OPTIONAL }

If UE only support one of the alternatives between R17 capability and R16+R18 capability, then certainly NW can only request the supported reporting. While if UE supports both of R17 and R16+R18, from NW perspective, since not sure what status the UE would report for a certain band in the R17 and R16+R18 reporting, so the most reliable solution is to request both R17 and R16+R18 reporting.

Furthermore, the NW request is semi-statically configured, while the R17 and R16+R18 reporting are dynamic reported, so NW can not adaptively change the request depending on what status the UE reports. E.g. at first NW only request R17 reporting, and the UE reports ‘ncsg’ for a band. After a while, the UE reports ‘gap’ for the same band, then the NW does not know why the UE can not support ‘ncsg’ for the same band, and the NW wants to know whether the UE can support ‘no gap with interruption’, but unfortunately, the NW can not reconfigure the request dynamically. So as to avoid such case, it is better to allow the NW to request both R17 and R16+R18 reporting.

Another case which would be beneficial by requesting both R17 and R16+R18 is the handover case. Since it is possible that the source BS supports R16+R18 while the target BS only supports R17, if the source BS only requests R16+R18 instead of requests both R17 and R16+R18, after the UE handovers to the target BS, the UE still only report R16+R18 reporting to the target BS, but since not support R18 signalling, the target BS can not understand the actual status reported by UE.

Consequently, it is beneficial and flexible to allow the NW to request both R17 and R16+R18 reporting for the capable UE.

It has been apporved that not and mapping between the R17 and R16+R18 reporting would be defined, so the NW could decide whether to configure gap or NCSG or neither of them based on the received reporting.

Proposal 2: Allow to enable both R17 and R18 reporting.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: To move forward, Option 1a, 2a, 5 and 6 are preferred.

|  |  | Small DRX cycle | Large DRX cycle |
| --- | --- | --- | --- |
| Option 1a | Small DRX ON | No interruption during DRX ON | No interruption during DRX ON |
|  | Large DRX ON | No interruption during DRX ON | No interruption during DRX ON |
| Option 2a | Small DRX ON | Interruption allowed once per Tcycle | No interruption during DRX ON |
|  | Large DRX ON | Interruption allowed once per Tcycle | No interruption during DRX ON |
| Option 5 | Small DRX ON | No interruption on last PDCCH slot of DRX ON | No interruption during DRX ON |
|  | Large DRX ON | No interruption on last PDCCH slot of DRX ON | No interruption on last PDCCH slot of DRX ON |
| Option 6 | Small DRX ON | Interruption allowed once per Tcycle | No interruption during DRX ON |
|  | Large DRX ON | Interruption allowed once per Tcycle | Interruption allowed once per Tcycle |

Proposal 2: Allow to enable both R17 and R18 reporting.

- Reference

- R4-2403543 “WF on NR_MG_enh2_part2”, Intel Corporation

- TS 38.331-i00