3GPP TSG RAN WG4 Meeting #111	R4-2408275

Fukuoka City, Fukuoka, Japan, 20 – 24 May, 2024

Agenda item:			7.6.1

Source:	vivo, Vodafone

Title:	On remaining issues for BWP operation without restriction

Document for:	Discussion

# Introduction

In the RAN4#110bis meeting, there were discussions on core maintenance part of BWP operation without restriction. Almost all the issues are addressed except the following one that was captured in the WF [1].

<Core Maintenance> #### Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1 - FFS following options - Option 1: - LS RAN2 and RAN1 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1. - Option 2a: - RAN4 to confirm that SSB outside a BWP can be configured within CSI-ResourceConfig for the BWP if UE supports option B-1-1. - Option 2b: - Configuration of SSB resources outside active BWP is allowed within CSI-ResourceConfig settings for UEs supporting option B-1-1 - No need to send an LS to RAN1/RAN2. - Option 2c: - RAN4 should further clarify what is the ambiguity part of the other WG spec before sending an LS requesting for further details. - Option 3: - Update FG53-1 related clarification in L1 RSRP measurement requirements.

In this contribution, we further provide our views on remaining open issues for BWP operation without restriction.

# Discussion

The issue was discussed for two meetings, with the details being provided in [2]. Based on all the discussions so far, it is fine that RAN4 focuses on RRM requirements only.

The L1-RSRP/L1-SINR requirements for UE supports option B-1-1 were updated in the last meeting as below in [3].

### 9.5.1 Introduction When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP. For UE supporting bwpOperationMeasWithoutInterrupt-r18, the measurements shall also be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements outside the active BWP. For UE supporting ncd-SSB-BWP-Wor-r18, the SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified. If SSB in the active DL BWP of serving cell i is NCD-SSB, for serving cell i the requirements in clause 8.1 apply provided that serving cell i is PCell or activated PSCell. The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, provided that the number of resources, including the number of SSB resources of the cell with PCI different from serving cell configured for L1-RSRP measurements in 9.13, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS. If the UE supports bwpOperationMeasWithoutInterrupt-r18, [UE shall be able to measure the SSB reources of csi-SSB-ResourceSet within the CSI-ResourceConfig configured outside active BWP], provided that the SSB is within the configured UE-specific CBW. The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP. The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured.

The highlighted requirements in bracket are valid only with the assumption that configuring SSB outside active BWP is allowed by RAN1 and RAN2 specifications. It is fine agreeing on the highlighted requirements in brackets without triggering discussions in RAN1/RAN2.

Proposal 1: Remove the bracket in L1-RSRP/L1-SINR requirements for UE supporting option B-1-1.

# Summary

In this contribution, we provided our views on remaining issues for BWP operation without restriction. Following proposals are provided.

Proposal 1: Remove the bracket in L1-RSRP/L1-SINR requirements for UE supporting option B-1-1.

# References

- R4-2406364	WF on core maintenance and performance part for BWP operation without restriction, vivo, Vodafone

- R4-2404953	On remaining issues for BWP operation without restriction, vivo

- R4-2406511	Draft Big CR to TS 38.133 on core requirement maintenance for Completion of specification support for BWP without restriction in NR, Vodafone, vivo