3GPP TSG RAN WG4 Meeting #111	R4-2408298

Fukuoka, JP, 20 May – 24 May, 2024

Agenda item:			7.12.2.5

Source:	vivo

Title:	Discussion on performance requirements for BW aggregation for positioning

Document for:	Discussion

Introduction

In RAN4#110bis meeting, there are discussions on the performance requirement of BW aggregation positioning and outcomes are captured in the WF [1]. In this paper, we provide our views on the issues of RRM performance requirements for BW aggregation for positioning.

Discussion

Accuracy requirements for PRS-RSRP and PRS-RSRPP

In the last meeting, a remaining issue for accuracy requirements for PRS-RSRP and PRS-RSRPP measurements based on aggregated PRS resources is copies as

Agreements: FFS: - Option 1: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement for the largest PRS BW among the aggregated PFLs applies to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation. - Option 2: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement applies for the total aggregated BW to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation.

In RAN1#115 meeting, the agreement of RSRP(P) measurement was reached as

Agreement If the UE/gNB reports aggregated timing measurement, the single reported RSRP/RSRPP (if reported) is based on aggregated PRS/SRS resources across aggregated PFLs/carriers. - Note1: it is up to RAN4 whether to define a corresponding requirement - Note2: for UL, measured SRS signals refer to aggregated SRS resources. For DL, measured PRS signals refer to aggregated PRS resources.

As the content highlighted in yellow, it can be observed that the RSRP and RSRPP is measurement based on the total aggregated BW, which means that option 2 is more applicable for defining performance requirement.

RAN4 has reached the agreement that existing accuracy requirements shall apply for both RSRP and RSRPP. In the current spec, the accuracy requirement is based on PRS BW. Since option 2 is preferred, the PRS BW should be revised to aggregated PRS BW.

Proposal 1: Existing accuracy requirement applies for the total aggregated BW to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation.

Conclusion

In this contribution, we provided our views on the RRM performance requirements for BW aggregation for positioning. Based on analysis following proposal are present.

Proposal 1: Existing accuracy requirement applies for the total aggregated BW to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation.

References

- R4-2406383 WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson