3GPP TSG RAN WG4 Meeting #111	R4-2408301

Fukuoka, JP, 20 May – 24 May, 2024

Agenda item:			10.6.2

Source:	vivo

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction for connected mode

Document for:	Discussion

- Introduction

In the RAN#103 meeting, a new WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode.

- Discussion

- 2.1 L3 measurement delay reduction by optimizing Rx BSF for UE supporting multiple-Rx simultaneous reception

In general, the FR2 L3 measurement delay is much longer than the delay in FR1 due to the Rx beam sweeping factor and the enhancement is needed. There are so many works have been already done in Rel-18 for reducing the FR2 measurement delay, e.g., introducing UE capability for FR2 L1 measurement delay reduction with reduced Rx beam sweeping factor (currently under discussion). For UE supporting multiple-Rx simultaneous reception on single carrier, RAN4 could consider to extend the capability to apply for L3 measurement for reducing the delay.

Proposal 1: RAN4 to consider introduce multi-Rx UE capability of reduced Rx beam sweeping factor for FR2 L3 measurement delay reduction.

In the last meeting, following WF is reached

Issue 2-1-1: applicable scenarios of L3 measurement delay reduction by optimizing Rx BSF WF for next meeting: - FFS: - L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided the target frequency layer to be measured is the only one in the single FR2-1 band and UE is configured [with one FR2-1 band]. - RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

For the WF of the applicable scenarios, it is OK to the reuse the Rel-18 assumptions and the description of the first bullet is not out of the WID scope. For the 2nd bullet, we can treat the PC3 as the 1st priority. However, whether the UE is configured with one FR2-1 band or at least one FR2-1 band is still needs to be discussed. If another FR2-1 band is added, the IBM is needed and multi-Rx is operated in each CC of FR2-1 band. If we still want to reduce the L3 measurement delay by optimizing Rx BSF, at least 3 panel are needed and it is out of WID scope.

Proposal 2: The applicable scenarios are to be revised to:

- L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-Rx simultaneous reception are applicable provided the target frequency layer to be measured is the single carrier and UE is configured with one FR2-1 band.

- RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

Issue 2-1-2: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF [Moderator note]: The condition here means in which case/condition/use-case/mode UE can apply the L3 measurement delay reduction by optimizing Rx BSF. WF for next meeting: FFS：Conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF - FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception features - FFS: UE’s mobility status, e.g., whether HST is precluded or not - FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - FFS: SSB processing delay/time for processing multiple beams received in a SMTC - FFS: Power consumption issue - FFS: UE has prior knowledge on the cell to be measured - FFS: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment - FFS: Other conditions: cell-centre UE or cell-edge UE - FFS: Simultaneous operation between multi-Rx simultaneous reception (L1) and L3 measurement delay reduction by optimizing Rx BSF.

First of all, we think the last bullet that simultaneous operation between multi-Rx simultaneous reception (L1) and L3 measurement delay reduction by optimizing Rx BSF is out of scope and no need to consider for RAN4.

Condition 1: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception features. We are OK with condition 1.

Condition 2: UE’s mobility status, e.g., whether HST is precluded or not. We think RAN4 needs to treat the low mobility status as the baseline to find out the optimization ways and no need to consider the HST in Rel-19.

Condition 3: RRM measurement with two panels activated, two searchers are occupied by this single carrier. We think this condition is make sense and since 2 searchers are occupied by single carrier, it may have impacts to other CCs.

Condition 4: SSB processing delay/time for processing multiple beams received in a SMTC. This condition needs to be studied since whether the delay can be reduced is depending on the way of buffering and processing.

Condition 5: Power consumption issue. It is a very important issue for BSF optimization based RRM measurement since it is a long-term activity and there is no reason to active the multi-Rx all the time which will cause the overheating issues.

Condition 6: UE has prior knowledge on the cell to be measured. Different prior knowledges may have different impacts for BSF optimization based RRM measurements and more study is needed.

Condition 8: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment. Rel-18 discussion is based on mTRP from NW perspective. For Rel-19 BSF optimization, RRM measurements are more important in cell-edge which is different from the typical sscenario of the mTRP.

Condition 9: Other conditions: cell-centre UE or cell-edge UE. As we mentioned before, cell-edge RRM measurement may be more important than the cell-centre RRM measurement in Rel-19, and it needs to be studied.

Proposal 3: RAN4 to discuss the following conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF.

  - Multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception features

  - Low mobility status

  - RRM measurement with two panels activated, two searchers are occupied by this single carrier

  - SSB processing delay/time for processing multiple beams received in a SMTC

  - Power consumption issue

  - UE has prior knowledge on the cell to be measured

  - Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment

  - Other conditions: cell-centre UE or cell-edge UE

Issue 2-1-3: UE measurement procedures to use L3 measurement delay reduction by optimizing Rx BSF WF for next meeting: FFS：Scenarios to use L3 measurement delay reduction by optimizing Rx BSF: - FFS: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - FFS: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - FFS: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - FFS: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - FFS: Handover - FFS: PSCell addition - FFS: RRC Re-establishment/RRC Connection Release with Redirection - FFS: SCell activation - FFS: SCG activation - FFS: CGI identification - FFS: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

From our perspective, we think the following aspects are out of scope:

    - Handover

    - PSCell addition

    - RRC Re-establishment/RRC Connection Release with Redirection

    - SCell activation

    - SCG activation

    - CGI identification

    - CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB

      - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

Proposal 4: Following aspects are out of scope:

    - Handover

    - PSCell addition

    - RRC Re-establishment/RRC Connection Release with Redirection

    - SCell activation

    - SCG activation

    - CGI identification

    - CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB

      - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

- 2.2 L3 measurement delay reduction by optimizing CSSF for UE not in multiple-Rx simultaneous reception mode

For UE not in multiple-Rx simultaneous reception mode in CA/DC scenario, the current delay requirements for measurement outside gap are defined based on 2 searchers assumption for total CCs. For FR2, more resources are needed and another independent searcher may be needed. In the current specification, a UE support 2-searcher is assumed as a PCC use one searcher, and SCC share the other searcher. However, consider the scenario that a UE is measuring a FR1 PCC and a FR2 SCC outside gap (CSSF=1), while a FR2 SCC is within the gap. In this scenario, a UE support 3-searchers measurement is perhaps implied to be considered to meet the requirements. However, changing the number of searchers is a very challenged job for vendors and whether the 3 searcher is a missing piece of current spec needs to be discussed.

Observation 1: It is a very challenged job to change the number of searchers and 3 searcher-implementation may impacts a lot of aspects of UE architecture.

Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement: - Only consider CSSF outside MG case. Proposal for further discussion: - Scenarios to use L3 measurement delay reduction by optimizing CSSF: - SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter and TSSB_measurement_period_inter

It is OK for the scenario 1 that the measurement is based on the SSB based Intra-frequency measurement without MG, For the SSB based inter-frequency measurement without MG scenario 2, we believe that there are also some impacts on scenario 2 if we optimize CSSF for scenario 1, since the 2 scenarios use the common CSSF table.

Proposal 5: RAN4 to consider the SSB based intra and inter frequency measurement without MG as the scenarios to use L3 measurement delay reduction by optimizing CSSF.

- 2.3 Common aspects for L3 measurement delay reduction

Issue 2-3-1: whether and/or which previous release feature shall also be considered in “FR2-1 SSB based L3 measurement delay reduction for connected mode” FFS in next meeting: - Moderator note: - encourage companies to provide view on which features in previous release shall be considered for “FR2-1 SSB based L3 measurement delay reduction for connected mode” by optimizing Rx beam sweeping factor - encourage companies to provide view on which features in previous release shall be considered for “FR2-1 SSB based L3 measurement delay reduction for connected mode” by optimizing CSSF outside gap

According to the previous release of FR1 HST, there were also some discussion for the CSSF enhancement and no conclusions were made. For the BSF optimization based L3 measurement delay reduction, RAN4 could consider to extend the scenario to FR1. Besides, if the CSSF is enhanced, CHO may be also enhanced and could be further discussed.

- Summary

In this contribution, we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode. Based on above analysis, following observations and proposals are present.

Proposal 1: RAN4 to consider introduce multi-Rx UE capability of reduced Rx beam sweeping factor for FR2 L3 measurement delay reduction.

Proposal 2: The applicable scenarios are to be revised to:

- L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-Rx simultaneous reception are applicable provided the target frequency layer to be measured is the single carrier and UE is configured with one FR2-1 band.

- RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

Proposal 3: RAN4 to discuss the following conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF.

  - Multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception features

  - Low mobility status

  - RRM measurement with two panels activated, two searchers are occupied by this single carrier

  - SSB processing delay/time for processing multiple beams received in a SMTC

  - Power consumption issue

  - UE has prior knowledge on the cell to be measured

  - Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment

  - Other conditions: cell-centre UE or cell-edge UE

Proposal 4: Following aspects are out of scope:

    - Handover

    - PSCell addition

    - RRC Re-establishment/RRC Connection Release with Redirection

    - SCell activation

    - SCG activation

    - CGI identification

    - CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB

      - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

Observation 1: It is a very challenged job to change the number of searchers and 3 searcher-implementation may impacts a lot of aspects of UE architecture.

Proposal 5: RAN4 to consider the SSB based intra and inter frequency measurement without MG as the scenarios to use L3 measurement delay reduction by optimizing CSSF.

- References

- R4-240830 New WID: WI resulting from discussion on RRM enhancements

- R4-2406392 WF for [110bis][230] NR_RRM_Ph5 Apple