# 3GPP TSG-RAN WG4 Meeting #111	R4-2408311

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.5.1

Source:	China Telecom

Title:	Discussion on RRM core requirements maintenance for pre-configured MGs, multiple concurrent MGs and NCSG

Document for:	Discussion

- Introduction

In RAN4 #110bis meeting, RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG were discussed and the WF [1] was approved.

In this paper, we present our views about RRM core requirements maintenance for pre-configured MGs, multiple concurrent MGs and NCSG.

- Discussion

Status in the WF [1]:

Issue 3-2-1: [Case 2] When the UE is configured with Concurrent gaps with NCSG, what is the potential changes to UE behaviour for NCSG upon SCell activation (in Rel-18) <Way Forward> - Option 1: - Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule) - Deactivated Scell MO associated with NCSG is measured within NCSG - Deactivated Scell MO not associated with NCSG is measured outside NCSG - Option 1a: - Based on the principle of reusing the gap association rule to determine in which MG the deactivated SCell MO would be performed, when the deactivated SCell switches to be activated, still reuse the R17 conditions to decide whether this SCell can be measured with the NCSG. That is, keep alignment with the understanding of R17 UE behaviours - Option 2: - When the SCell is deactivated, - the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG regardless of gap association. - Option 2a: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within the associated NCSG if NCSG+NCSG is configured. - Option 3: - RAN4 should discuss the following scenarios: - Scenario 1: UE is configured with two NCSGs. Association between SCell MO and one NCSG is provided. - The measurement should be done with the associated NCSG. - Scenario 2: UE is configured with two NCSGs. Association between SCell MO and NCSG is not provided. - Discuss whether to define requirement for this scenario. If so, which NCSG shall be used for measurement? - Scenario 3: UE is configured with one NCSG and one type-2 legacy gap. Association between SCell MO and NCSG or MG is not provided. - The measurement should be done with the associated NCSG. - Scenario 4: UE is configured with one NCSG and one type-2 legacy gap. MO is associated to NCSG. - The measurement should be done with the associated NCSG. - Scenario 5: UE is configured with one NCSG and one type-2 legacy gap. MO is associated to MG. - Discuss whether to define requirement for this scenario. If so, whether the MO shall be moved from MG to NCSG when the Scell becomes deactivated. - Option 4: - No need to distinguish the UE indication for activated status and deactivated status separately for a single serving cell. - Option 5: - The collision case between measured deactivated SCell MO’s outside NCSG, if allowed in Rel-17, and concurrent MG occasions needs to be considered for Case 2 requirements in Rel-18. - Option 6: - RAN4 to further investigate for Rel-18 the case the UE needs a dedicated NCSG pattern for measuring deactivated SCell MO’s from a set of NCSG patterns and to introduce a UE capability for that.

When the SCell is deactivated, the deactivated SCell’s MO being measured within NCSG can avoid interruption, it will be beneficial for network throughput. It’s proposed that the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG regardless of gap association.

Proposal 1: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG regardless of gap association.

- Conclusion

The contribution presents our views about RRM core requirements maintenance for pre-configured MGs, multiple concurrent MGs and NCSG, with the following proposals:

Proposal 1: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG regardless of gap association.

- References

- R4-2406439, WF on NR_MG_enh2_part1, MediaTek inc., RAN WG4 Meeting #110bis, April 2024.