# 3GPP TSG-RAN WG4 Meeting #111	R4-2408312

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.5.3

Source:	China Telecom

Title:	Discussion on RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG

Document for:	Discussion

- Introduction

In RAN4 #110bis meeting, RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG were discussed and the WF [1] was approved.

In this paper, we present our views about RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG issues captured in the WF [1].

- Discussion

Status in the WF [1]:

Issue 4-3-2: [Case 2] Test cases list for Case 2 Agreement: - FFS Con-NCSG TC4, pending on the core part maintenance conclusions. - Agree to add Con-NCSG TC3 No; Test case category; Test purpose; Volunteering companies. Con-NCSG TC1; Event triggered reporting test on inter-frequency in FR1 with Type-2 + NCSG; - Inter-frequency cell search/measurement delay is met for Cell2 in NCSG and Inter-frequency cell search/measurement delay for Cell3 in Type-2 MG, and - UE receives data in Cell1 meeting scheduling restriction requirements, and - Verify the gap collision rule; Ericsson. Con-NCSG TC2; Event triggered reporting test on intra-frequency in FR2 with Type-2 + NCSG; · Same as Con-NCSG TC 1, yet for intra-f in FR2.; CATT. Con-NCSG TC3; Event triggered reporting test on inter-frequency in FR1 with concurrent gap and NCSG of two NCSGs; · Inter-frequency cell search delay is met for Cell2 in NCSG1 and Inter-frequency cell search/measurement delay for Cell3 in NCSG2, and · UE receives data in Cell1 meeting scheduling restriction requirement - Verify the gap collision rule; Ericsson. < Way Forward > FFS the following test cases: No; Test case category; Test purpose; Volunteering companies. Con-NCSG TC4; Event triggered reporting test on deactivated SCell in FR1 with concurrent gap and NCSG; · Intra-frequency cell search/measurement delay for deactivated SCC is met for Cell2 in NCSG, and Inter-frequency cell search/measurement delay for Cell3 in MG · UE receives data in Cell1 meeting scheduling restriction requirements, and UE will not cause any interruption on Cell1 outside VIL windows..

The deactivated SCell measurement within NCSG is discussed in the core part maintenance and we prefer that the deactivated SCell’s MO can be measured within NCSG if the SMTC is partially or fully overlapped with NCSG regardless of gap association. If the deactivated SCell’s MO being measured within NCSG is agreed, it’s necessary to define test case to verify this UE behavior.

Proposal 1: It’s proposed to define Con-NCSG TC4 Event triggered reporting test on deactivated SCell in FR1 with concurrent gap and NCSG.

- Conclusion

The contribution presents our views about RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG, with the following proposals:

Proposal 1: It’s proposed to define Con-NCSG TC4 Event triggered reporting test on deactivated SCell in FR1 with concurrent gap and NCSG.

- References

- R4-2406439, WF on NR_MG_enh2_part1, MediaTek inc., RAN WG4 Meeting #110bis, April 2024.