3GPP TSG-RAN WG4 Meeting # 111									       R4-2408318

Fukuoka City, Fukuoka, Japan, 20th May – 24th May 2024

Title: 	MSD for CA_n71(2A)

Agenda Item:	6.19.2

Source: 	Murata Manufacturing Co. Ltd

Document for:	Approval

- Introduction

This contribution is a follow up to our previous MSD analysis [1] for CA_n71(2A).

- Discussion

2.1 MSD Test Point Selection

In RAN4#110-bis, as discussed in our paper [1], the test point candidates for CA_n71(2A) in the below table maximize the SCC MSD.

Table 1: CA_n71(2A) REFSENS one UL test point candidates, extracted from [2]

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap [MHz] | UL PCC allocation | ΔRIBNC (dB) | Candidate | BCS | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | 10.0 | 5 (RBstart = 2) TC 1.106 | 22.2 | #1: Legacy worst-case MSD (Obs 1.1) | All | near full overlap IMD3 |
|  |  | 20MHz + 5MHz | 10.0 | 20 (RBstart = 0) TC 1.104 | FFS | #2: Alternative to legacy (Obs.1.3) |  | IMD3 full overlap |
|  |  | 25MHz + 5MHz | 5.0 | 20 (RBstart = 8) TC 1.102 | FFS | #3: Rel-18 UEs that support symmetrical UL/DL CBW (Obs. 1.4) | 4/5 only |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band (Obs. 1.2) |  |  |  |  |  |  |  |  |

Figure 1: Configurations of each test point candidate, extracted from [1]

Based on the Release 17 simplifications, it is good to retain the current 15MHz+10MHz test point as the “legacy” test point and remove other “legacy” test points which result in lower MSD. Furthermore, two alternative test points should be added for CA_n71(2A); (1) 20MHz+5MHz test point which causes IMD3 full overlap to SCC DL for all BCS and (2) 25MHz+5MHz test point which is applicable for only BCS 4/5 and falls full overlap IMD3 into SCC DL. It could also be considered that the two alternative test points would be adopted, and the legacy test point would be removed because 20MHz+5MHz test point will cause worse interference to SCC DL for all BCS than the legacy test point.

Observation	1: The two alternative test points would be adopted, and the legacy test point would be removed because 20MHz+5MHz test point will cause worse interference to SCC DL for all BCS than the legacy test point.

Proposal 1: For PC3 and PC2 CA_n71(2A) REFSENS, adopt the test points proposed in table 2.

We are open to discuss whether the legacy worst test point 15MHz+10MHz can be removed and the two alternative test points are introduced.

Table 2: CA_n71(2A) REFSENS one UL test points

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap /[MHz] | UL PCC allocation | Duplex mode |
| --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap=10.0 | 5 (RBstart = 2) | FDD |
|  |  | 20MHz + 5MHz | Wgap=10.0 | 20 (RBstart = 0) | FDD |
|  |  | 25MHz + 5MHz(NOTE 11) | Wgap=5.0 | 20 (RBstart = 8) |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |
| NOTE 11: Bandwidth Combination Set 4/5 |  |  |  |  |  |

2.2 MSD Analysis

We reviewed our MSD analysis for CA_n71(2A) PC3, PC2 1Tx, and PC2 2Tx using the same test point in [1]. MSD analysis for the legacy test point is performed for reference.

2.2.1 Power Amplifier Calibration

For PC3, PA is calibrated to deliver -30 dBc ACLR using full allocation 20 MHz with DFTs-QPSK at MPR 1dB.

For PC2, PA is calibrated to deliver -31dBc ACLR using full allocation 20MHz with DFTs-QPSK at MPR 1dB.

2.2.1 Rx Noise Levels

PC3, 1x26dBm PC2, and 2x23dBm PC2 interference levels affecting the DL SCC are summarized in Table 3.

It should be noted that reversed IMD is also considered when re-evaluating Dual Tx PC2 MSD values.

Table 3: PC3 1x26dBm PC2, and 2x23 dBm PC2 measured noise levels.

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation | 1Tx PC3 [dBm/MBW] | 1Tx PC2 | 2Tx PC3+PC3 PA1 | 2Tx PC3+PC3 PA2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  | [dBm/MBW] | [dBm/MBW] | [dBm/MBW] |
| CA_n71(2A) | 15/15 | 15 MHz + 10 MHz | 5 (RBstart = 2) | -17.2 | -14.3 | -16.2 | -16.2 |
|  |  | 20MHz + 5 MHz | 20 (RBstart = 0) | -15.6 | -13.7 | -14.6 | -14.6 |
|  |  | 25 MHz + 5 MHz | 20 (RBstart = 8) | -15.5 | -13.8 | -14.5 | -14.5 |

2.1.3 MSD Evaluations

We evaluated MSD values for CA_n71(2A) PC3, 1x26dBm PC2, and 2x23dBm PC2 using the following assumptions:

- Tx-Rx duplexer isolation: 55dB

- Tx-antenna duplex rejection in Rx range: 45 dB

- Primary antenna to diversity antenna isolation: 10 dB

- MRC combining and -1 dB Signal to-Noise Ratio(SNR)

Figure 2: CA_n71(2A) PC3, 1x26dBm PC2 and 2x23dBm MSD analysis.

Proposal 2: Consider adopting MSD values highlighted in yellow for CA_n71(2A) proposed in Table 4 and 5.

Table 4: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands for 7.3A.2.2-1

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap /[MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap=10.0 | 5 (RBstart = 2) | 22.2 | FDD |
|  |  | 20MHz + 5MHz | Wgap=10.0 | 20 (RBstart = 0) | [27.0] | FDD |
|  |  | 25MHz + 5MHz(NOTE 11) | Wgap=5.0 | 20 (RBstart = 8) | [27.1] |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |  |
| NOTE 11: Bandwidth Combination Set 4/5 |  |  |  |  |  |  |

Table 5: intra-band non-contiguous CA with one uplink configuration for reference sensitivity for PC2 in FDD bands-Table 7.3A.2.2-1a

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap [MHz] | UL PCC allocation | ΔRIBNCX (dB) | ΔRIBNCY (dB) | BCS | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | 10 | 5 (RBstart = 2) | [25.1] | [27.3] | All | FDD |
|  |  | 20MHz + 5MHz | 10 | 20 (RBstart = 0) | [28.9] | [32.1] |  | FDD |
|  |  | 25MHz + 5MHz | 5 | 20 (RBstart = 8) | [28.8] | [32.2] | 4/5 only |  |
| NOTE X: Applicable to UE supporting PC2 with single Tx. NOTE Y: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |

- Conclusion

Observation 1: The two alternative test points would be adopted, and the legacy test point would be removed because 20MHz+5MHz test point will cause worse interference to SCC DL for all BCS than the legacy test point.

Proposal 1: For PC3 and PC2 CA_n71(2A) REFSENS, adopt the test points proposed in table 2.

We are open to discuss whether the legacy worst test point 15MHz+10MHz can be removed and the two alternative test points are introduced.

Table 2: CA_n71(2A) REFSENS one UL test points

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap /[MHz] | UL PCC allocation | Duplex mode |
| --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap=10.0 | 5 (RBstart = 2) | FDD |
|  |  | 20MHz + 5MHz | Wgap=10.0 | 20 (RBstart = 0) | FDD |
|  |  | 25MHz + 5MHz(NOTE 11) | Wgap=5.0 | 20 (RBstart = 8) |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |
| NOTE 11: Bandwidth Combination Set 4/5 |  |  |  |  |  |

Proposal 2: Consider adopting MSD values highlighted in yellow for CA_n71(2A) proposed in Table 4 and 5.Table 4: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands for 7.3A.2.2-1

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap /[MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap=10.0 | 5 (RBstart = 2) | 22.2 | FDD |
|  |  | 20MHz + 5MHz | Wgap=10.0 | 20 (RBstart = 0) | [27.0] | FDD |
|  |  | 25MHz + 5MHz(NOTE 11) | Wgap=5.0 | 20 (RBstart = 8) | [27.1] |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. |  |  |  |  |  |  |
| NOTE 11: Bandwidth Combination Set 4/5 |  |  |  |  |  |  |

Table 5: intra-band non-contiguous CA with one uplink configuration for reference sensitivity for PC2 in FDD bands-Table 7.3A.2.2-1a

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap [MHz] | UL PCC allocation | ΔRIBNCX (dB) | ΔRIBNCY (dB) | BCS | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | 10 | 5 (RBstart = 2) | [25.1] | [27.3] | All | FDD |
|  |  | 20MHz + 5MHz | 10 | 20 (RBstart = 0) | [28.9] | [32.1] |  | FDD |
|  |  | 25MHz + 5MHz | 5 | 20 (RBstart = 8) | [28.8] | [32.2] | 4/5 only |  |
| NOTE X: Applicable to UE supporting PC2 with single Tx. NOTE Y: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |

- References

- R4-2404762, MSD considerations on CA_n71(2A), 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, Murata Manufacturing Co. Ltd.

- R4-2400365, PC2 CA_n71(2A) MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc.