# 3GPP TSG-RAN WG4 Meeting # 111	R4-2408328

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	4.4

Source:	Ericsson

Title:	(NR_MG_enh-Core)Remaining issues of Rel-17 MG enh.

Document for:	Discussion

# Introduction

The Rel-17 and Rel-18 MG enh. WI are already closed. However, in RAN4 #107 meeting, some remaining Rel-17 MG issues were identified in WF[1].

In this paper, we will discuss the remaining issues of Rel-17 MG enhancements.

# The remaining issues of Rel-17 NCSG

In RAN4 #107 meeting, some remaining issues for NCSG were identified as follow.

Sub-topic 4-4: Other Rel-17 rules to be revisited Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from meeting RAN4#106-bis-e >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped. < Agreements from online session >: - Option 1: - UE measures the deactivated SCell outside of MG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG. - FFS whether a new indication shall be introduced enable support of NCSG for deactivated SCell only. < Agreement >: - Align the understanding of Rel-17 UE behaviours - Only up to 1 NCSG can be configured. All activated Scell MOs are implicitly associated to the NCSG - In the dynamic UE capability signalling, there is no separate indication for activated/deactivated serving cells. This implies UE only indicate the capability if it supports all scenarios, including - deactivated Scell - activated Scell but SSB not in active BWP - Understanding to be clarified: - Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG?

In Rel-15, UE will perform deactivated SCell measurement outside gaps with interruption. In Rel-17, RAN4 agrees to introduce NCSG. One of the use cases is to handle the deactivated SCell measurement. Deactivated SCell measurement is different with other inter-frequency measurement since no additional RF chain is needed. When UE supports NCSG capability which means UE can support the NCSG patterns. The intention of intraFreq-needForNCSG will be used to indicate whether the intra-frequency with SSB outside active BWP can be measured within NCSG other than MGother than deactivated SCell measurement within NCSG. Furthermore, this issue has already captured in TS38.133 as follow:

#### 9.1.5.3 Monitoring of multiple layers within NCSG … The carrier-specific scaling factor CSSFwithin_ncsg,i for a measurement object i derived in this clause is applied to following measurement types: - SSB-based intra-frequency measurement object without measurement gap as defined in clause 9.2.1 corresponding to an activated serving cell, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the NCSG; - SSB-based intra-frequency measurement object with NCSG as defined in clause 9.2.1 corresponding to an activated serving cell (in non-dormancy); - SSB-based intra-frequency measurement object corresponding to a deactivated serving cell or to an activated serving cell in dormancy, when all or part of the SMTC occasions of this intra-frequency measurement object are overlapped by the NCSG;

Therefore, we propose that all deactivated SCells are measured within NCSG regardless of further UE capability reporting of intraFreq-needForNCSG.

Observation 1: Deactivated SCell will be measured within NCSG regardless of FR type has already captured in spec.

Proposal 1: In Rel-17, when UE supports NCSG capability, all deactivated SCell are measured within NCSG provided that NW configures NCSG.

# Summary and Conclusion

In the contribution, we discuss the remaining issues for Rel-17 MG enh with the following proposals:

Observation 1: Deactivated SCell will be measured within NCSG regardless of FR type has already captured in spec.

Proposal 1: In Rel-17, when UE supports NCSG capability, all deactivated SCell are measured within NCSG provided that NW configures NCSG.

# References

- R4-2321604, “WF on further enhancements on measurement gaps and measurements without gaps”, MTK