# 3GPP TSG-RAN WG4 Meeting #111                                R4-2408364

# Fukuoka, Japan, 20th-24th May, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on testability for LP-WUS UE RF requirements

# Agenda Item:   10.14.2.4

# Document for:  Approval

- Introduction

In RAN4#110bis, RAN4 had some preliminary discussion on the testability for LP-WUS but there were no agreements achieved because we need to discuss performance requirements first and then go to testability[1]. In this contribution, we discussed some issues needed to be considered in testability.

- Discussion

Usually legacy test for NR receiver is performed in CONNECTED mode where there is ACK/NACK feedback to TE to calculate throughput. But this test metric is not suitable for LP-WUR because the purpose to test LP-WUS is to check the capability to wake up the main radio (MR). LP-WUS can be used in IDLE mode to wake up MR to receive Paging message and CONNECTED mode to wake up MR to receive PDCCH and PDSCH[2].

Therefore, RNA4 should determine to perform the test of LP-WUR in IDLE mode or CONNECTED mode first.

Proposal 1: RNA4 should determine to perform the test of LP-WUR in IDLE mode or CONNECTED mode first.

## 2.1 Test in IDLE Mode

For test in IDLE mode, in our understanding, the wake-up procedure of LP-WUS mechanism in IDLE mode as follows.

- Step 1: BS transmits WUS to UE, which carries UE group ID. - Step 2: UE receives the signal and check CRC or correlation power. If pass, the MR will be waken up. - Step 3: If MR is waken up, MR will receive corresponding Paging message from BS and feed back PRACH to the BS. Then connection is established.

For Step 1, if there is no WUS transmitted from BS but UE wakes up the MR, it is a false alarm. In this case, even if MR is waken up, there is no corresponding Paging message at all, the connection will not be established. For Step 2, if there is something wrong with the received WUS caused by low SNR for example and CRC check does not pass or correlation power does not exceed the threshold, UE will not wake up the MR correctly. This is a missed detection.

In order to test the performance of LP-WUR, TE will work as BS and should be able to transmit Paging message corresponding to LP-WUS and receive PRACH from the main radio to establish the connection. Based on the discussion above, from TE’s perspective, the number of LP-WUS transmitted to UE is known and the corresponding PRACH received from UE can be counted. Thus, miss detection rate (MDR) can be calculated by TE while UE dose not because UE is not clear about how much LP-WUS is transmitted in total. From UE’s perspective, all the detection number is known and the number of false alarms can be counted. Thus, false alarm rate (FAR) can be calculated by UE while TE is not clear about FAR.

Proposal 2: For IDEL mode test, TE should be designed to transmit Paging message corresponding to LP-WUS and receive PRACH from the main radio after LP-WUS is transmitted.

For test metric, it should be aligned with performance metric, but RAN4 haven’t decided performance metric yet. As we discussed above, if only MDR is chosen as test metric, there is no need to design dedicated feedback to help calculate MDR. If both MDR and FAR are chosen as test metric, then for FAR test, a feedback mechanism should be designed to help TE to know the FAR. Besides, test mode should be designed for MDR and FAR separately. When testing MDR, there are successive LP-WUS transmitted (including Paging message transmitting and PRACH receiving). When testing FAR, no LP-WUS is transmitted and only noise exists.

Observation 1: For IDLE mode test, if only MDR is chosen as test metric, there is no need to design dedicated feedback to help test MDR.

Observation 2: For IDLE mode test, if both MDR and FAR are chosen as test metric, a feedback mechanism should be designed to help TE to know FAR. Besides, test mode should be designed for MDR and FAR separately. When testing MDR, there are successive LP-WUS transmitted (including Paging message transmitting and PRACH receiving). When testing FAR, no LP-WUS is transmitted and only noise exists.

## 2.2 Test in CONNECTED Mode

The procedure of LP-WUS mechanism in CONNECTED mode is that:

- Step 1: BS transmits WUS to UE, which carries UE group ID. - Step 2: UE receives the signal and check CRC or correlation power. If pass, the MR will be waken up. - Step 3: If MR is waken up, MR will receive PDCCH and PDSCH, then feed back ACK/NACK.

In this case, legacy test method can be reused while the test metric is MDR (or with FAR together) rather than throughput. If TE transmits LP-WUS and PDCCH and PDSCH but there is no ACK/NACK fed back, it is regarded as a missed detection. If there is no LP-WUS transmitted but MR is waken up, it is regarded as a false alarm. As discussed in section 2.1, there is no need to design dedicated feedback to help test MDR while a feedback mechanism should be designed to help TE to know FAR.

Proposal 3: For CONNECTED mode test, legacy test method can be reused while the test metric is MDR (or with FAR together) rather than throughput.

Observation 3: For CONNECTED mode test, if only MDR is chosen as test metric, there is no need to design dedicated feedback to help test MDR.

Observation 4: For CONNECTED mode test, if both MDR and FAR are chosen as test metric, a feedback mechanism should be designed to help TE to know FAR. Besides, test mode should be designed for MDR and FAR separately. When testing MDR, there are successive LP-WUS transmitted (including PDCCH and PDSCH). When testing FAR, no LP-WUS is transmitted and only noise exists.

- Conclusion

Proposal 1: RNA4 should determine to perform the test of LP-WUR in IDLE mode or CONNECTED mode first.

Proposal 2: For IDEL mode test, TE should be designed to transmit Paging message corresponding to LP-WUS and receive PRACH from the main radio after LP-WUS is transmitted.

Observation 1: For IDLE mode test, if only MDR is chosen as test metric, there is no need to design dedicated feedback to help test MDR.

Observation 2: For IDLE mode test, if both MDR and FAR are chosen as test metric, a feedback mechanism should be designed to help TE to know FAR. Besides, test mode should be designed for MDR and FAR separately. When testing MDR, there are successive LP-WUS transmitted (including Paging message transmitting and PRACH receiving). When testing FAR, no LP-WUS is transmitted and only noise exists.

Proposal 3: For CONNECTED mode test, legacy test method can be reused while the test metric is MDR (or with FAR together) rather than throughput.

Observation 3: For CONNECTED mode test, if only MDR is chosen as test metric, there is no need to design dedicated feedback to help test MDR.

Observation 4: For CONNECTED mode test, if both MDR and FAR are chosen as test metric, a feedback mechanism should be designed to help TE to know FAR. Besides, test mode should be designed for MDR and FAR separately. When testing MDR, there are successive LP-WUS transmitted (including PDCCH and PDSCH). When testing FAR, no LP-WUS is transmitted and only noise exists.

- Reference

- R4-2406619 WF on NR LP-WUS UE requirements, RAN4#110bis, Changsha, China, Apr. 2024.

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN#103, Maastricht, Netherlands, Mar. 2024.