# 3GPP TSG-RAN WG4 Meeting #111                                R4-2408365

# Fukuoka, Japan, 20th-24th May, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on BS RF requirements for LP-WUS/WUR

# Agenda Item:   10.14.3

# Document for:  Approval

- Introduction

In this contribution, we discussed the dynamic range for LP-WUS and the impact of LP-WUS power boosting.

- Discussion

## 2.1 Definition of Dynamic Range

In last meeting, RAN4 discussed the definition of dynamic range of LP-WUS and achieved a preliminary consensus of the concept of LP-WUS as follows under the assumption that (1) the transmission power is shared between NR OFDM signal and NR LP-WUS for the same carrier, and (2) the rated carrier output power and rated total output power are not changed with LP-WUS power boosting[1].

- Concept of LP-WUS dynamic range/power boosting is considered as starting point - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

In our understanding, before we discuss the dynamic range of LP-WUS, it is important to declare its definition first to avoid different understanding, and these two assumptions are necessary when analyzing dynamic range. We propose to keep current concept of LP-WUS dynamic range/power boosting for later discussion, and it is okay to revise some wording if necessary when we finally finish the whole discussion and add it into current spec.

Proposal 1: Keep current concept of LP-WUS dynamic range/power boosting for later discussion: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

## 2.2 Power Boosting of LP-WUS

As shown in section 2.1, the transmit power is shared between NR OFDM signal and NR LP-WUS for the same carrier and the total transmit power is fixed. Therefore, when the power of LP-WUS is boosted, the power of NR signal will be decreased. In our previous contribution [2], we have shown the power degradation of NR signal with different channel bandwidth when the power of LP-WUS is boosted by 3 dB and 6dB, which is quoted below in Table 1.

Table 1  The power degradation of NR signal with different channel bandwidth

| Declared BS power (dBm) | CBW (MHz) | SCS (kHz) | N_RB | LP-WUS RB | NR power degradation (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | WUS Boosting 3dB | WUS Boosting 6dB |
| 46 | 5 | 15 | 25 | 24 | X | X |
|  | 10 | 15 | 52 | 24 | 10.20 | X |
|  | 15 | 15 | 79 | 24 | 2.60 | X |
|  | 20 | 15 | 106 | 24 | 1.60 | 9.10 |
|  | 25 | 15 | 133 | 24 | 1.10 | 5.20 |
|  | 30 | 15 | 160 | 24 | 0.77 | 3.28 |
|  | 35 | 15 | 188 | 24 | 0.62 | 2.44 |
|  | 40 | 15 | 216 | 24 | 0.58 | 2.10 |
|  | 45 | 15 | 242 | 24 | 0.48 | 1.75 |
|  | 50 | 15 | 270 | 24 | 0.45 | 1.55 |
|  | 60 | 30 | 162 | 12 | 0.39 | 1.33 |
|  | 70 | 30 | 189 | 12 | 0.29 | 0.96 |
|  | 80 | 30 | 217 | 12 | 0.27 | 0.92 |
|  | 90 | 30 | 245 | 12 | 0.21 | 0.72 |
|  | 100 | 30 | 273 | 12 | 0.19 | 0.62 |

Usually, the power degradation of NR signal is unwanted because of the reduction of coverage and deterioration of signal quality. Especially when channel bandwidth is narrow, the power degradation of NR signal is unacceptable. For wider channel bandwidth, although the power degradation of NR signal can be a little lighter, the out-of-band emission deterioration caused by LP-WUS power boosting becomes more obvious, as shown in Fig. 1[2].

Fig. 2  The effect of LP-WUS power boosting on out-of-band emission.

As discussed above, the level of LP-WUS power boosting depends on BS implementation. Therefore, we propose to set the LP-WUS a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the boosting level supported.

Proposal 2: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level.

## 2.3 Applicable BS Types

Currently in TS38.104, only BS Type 1-C supports NR with in-band NB-IoT operation[1]. BS Type 1-H and BS Type 1-O are excluded considering the power consumption issue. When it comes to LP-WUS, more PRBs (for example 11 or 12 PRBs for 30kHz SCS) compared to NB-IoT with single PRB are configured, which means that power boosting on more PRBs should be considered.

As we discussed in the last meeting, it is a consensus that the rated carrier output power and rated total output power are not changed with LP-WUS power boosting and the transmission power is shared between NR OFDM signal and NR LP-WUS, which is similar to NB-IoT in [1] as follows.

NOTE: For NB-IoT operation in NR in-band, the NR carrier and NB-IoT carrier shall be seen as a single carrier occupied NR channel bandwidth, the output power over this carrier is shared between NR and NB-IoT. This note shall apply for Pmax,c,AC and Prated,c,AC.

Just like that in NB-IoT discussion, it is necessary to discuss which type of BS is able to support LP-WUS with some level of power boosting.

Proposal 3: RAN4 needs to discuss which type of BS is able to support LP-WUS with some level of power boosting.

- Conclusion

Proposal 1: Keep current concept of LP-WUS dynamic range/power boosting for later discussion: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

Proposal 2: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level.

Proposal 3: RAN4 needs to discuss which type of BS is able to support LP-WUS with some level of power boosting.

- Reference

- R4-2406140 Way Forward for [110bis][314] NR_LPWUS, RAN4#110bis, Changsha, China, Apr. 2024.

- R4-2405120 Discussion on BS RF requirements for LP-WUS/WUR, RAN4#110bis, ZTE, Changsha, China, Apr. 2024.

- TS38.104, v18.5.0