# 3GPP TSG-RAN WG4 Meeting # 111                                                R4-2408380

# Fukuoka, Japan, May. 20th – 24th, 2024

# Source:	ZTE Corporation, Skyworks Solutions, Inc.

# Title:	TP for TR38.718-02-01_CA_n40A-n41C

# Agenda Item:	6.1.1.1

# Document for:	Approval

- Introduction

For UL CA_n40A-n41C_BCS4 and 5, it was already included in the specification TS38.101-, which is:

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n41C | CA_n41C CA_n40A-n41A CA_n40A-n41C | n40 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |

Due to simultaneous Rx/Tx operation is supported between band n40 and band n41, the IMDx caused by UL CA_n41C would fall into band 40 DL Rx, which means the MSD values should be defined. But unfortunately, the MSD is missing in current specification.

Hence, we provide the MSD analysis for UL CA_n41C which would fall into band 40 DL Rx in this TP based on the MSD test points in WF [2].

- Reference

- [1] TR38.718-02-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.9.0

- [2] R4-2403716	WF on CA_n40A-n41C, ZTE, Skyworks

- Text Proposal

----- Start of TP -----

  - 5.x		CA_n40-n41

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1 Operating bands for CA

Table 5.x.1.1-1: CA band combination CA_n40-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n40-n41 | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Same configurations in the specification.

      - 5.x.1.3	UE Co-existence studies

For the UL/DL harmonic and harmonic mixing caused by single band n40 or single band n41, it was already analyzed in the fallback configurations of DL CA_n40A-n41A with UL single carrier or UL CA_n40A-n41A, and it can be applied for DL CA_n40A-n41C with UL single carrier or UL CA_n40A-n41C.

      - 5.x.1.3.1	UE co-existence studies for 2 Uplink CCs in one Intra-Band CA

Additionally, the co-existence studies caused by UL CA_n41C should be analyzed for CA_n40A-n41C, which can be found in table Table 5.x.1.3.1-1:

Table 5.x.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 190 | 2496 | 2690 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 2496 | 2516 | 2686 | 2690 | 2670 | 2500 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 190 | 5012 | 5360 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2306 | 2880 | 7508 | 8050 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 380 | 4802 | 5570 | 10004 | 10740 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2116 | 3070 | 7298 | 8260 | 12500 | 13430 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 570 | 4612 | 5760 | 9794 | 10950 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1926 | 3260 | 7108 | 8450 | 12290 | 13640 |

It can be seen that 3rd order, 5th order and 7th order IMD caused by CA_n41C impact band n40 DL.

      - 5.x.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n40-n41 are already defined in the spec TS38.101-1.

      - 5.x.1.5	REFSENs requirements

According to the co-existence study, the additional MSD requirements should be defined when simulataneous Rx/Tx operation is supported for UL CA_n40A-n41C.

In terms of the measurement study in R4-2405955, it shows the measured Band n41 2UL IMD3 level at 23dBm at the Band n40 5MHz downlink channel is is +4.7dBm, i.e. ~ 0.7dBm at the antenna port. At that level, the UE fails the -13dBm/MHz emission requirements. Therefore, assuming that the UE uses the MPR allowance to meet the -13dBm/MHz requirements, then the Tx noise caused by CA_n41C on the n40 Rx is -9dBm considering the 4dB IL from LNA to antenna. The corresponding MSD requirement is shown in table 5.x.1.5-1:

Table 5.x.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2358.5 | 42.511,16 | TDD | IMD3 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
| NOTE 11: This band is subject to IMD5 also which MSD is not specified. NOTE 16: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

      - 5.x.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

----- End of TP -----