3GPP TSG-WG4 Meeting # 111	 R4-2408400

# Fukuoka, Japan, 20 – 24 May 2024

Agenda item:	10.12.2.2

Source: 	Qualcomm Incorporated

Title: 	Views on new BS RF requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#110-bis, a WF was agreed in [2]. In this contribution we share our views on existing BS RF requirements for Rel-19 SBFD.

# New BS RF requirements for SBFD operation

As discussed in Section 10.1.4 [3], the following requirements are expected to be added for SBFD BS:

- In-channel adjacent subband leakage ratio

- In-channel adjacent subband Blocking and adjacent subband selectivity

- Transmitter transient period

Within in-band operation, the in-channel adjacent subband leakage will impact the receiving subband of the same BS. Such leakage might contribute to REFSENSE. It was noted during the SI [3] that OTA REFSENSE will be derived considering self-interference, inter-site and inter-sector interference. The in-channel adjacent subband leakage (selectivity) is the same as the ACLR (ACS), but on a subband in-channel granularity level. Consequently, there is a direct implication of the coexistence scenarios and assumptions on the subband leakage and selectivity definition.

Observation 1: The in-channel subband sub-band Tx leakage and selectivity new requirements depend on the self-interference, inter-site and inter-sector interference considerations.

During the SI phase, to progress the adjacent channel coexistence, RAN4 agreed to reuse legacy ACLR and ACS requirement for the in-channel adjacent subband leakage and selectivity. There was lengthy discussions on whether to consider some enhanced performance at the BS side, owing to additional filtering or enhanced interference mitigation capabilities. Accordingly, RAN4 is expected to define scenarios and assumptions upon which the new requirements will be defined for. 

Proposal 1: RAN4 to first define scenarios and assumptions required for the definition of the in-channel adjacent subband leakage and selectivity.

Finally on the transmitter transient period, RAN4 was not able to conclude during the SI phase on the necessity of such requirement. In our view, it is important to define a new requirement for the transient period between non-SBFD slot and SBFD slot. As a starting point, RAN4 can start with the study the adoption of the TDD transmitter transient period requirement given in TS 38.104.

Proposal 2: RAN4 to define a new requirement for the transmitter transient period between SBFD and non-SBFD slot. As a starting point, RAN4 can start with the feasibility of adopting the Tx transient period requirement for TDD BS.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Observation 1: The in-channel subband sub-band Tx leakage and selectivity new requirements depend on the self-interference, inter-site and inter-sector interference considerations.

Proposal 1: RAN4 to first define scenarios and assumptions required for the definition of the in-channel adjacent subband leakage and selectivity.

Proposal 2: RAN4 to define a new requirement for the transmitter transient period between SBFD and non-SBFD slot. As a starting point, RAN4 can start with the feasibility of adopting the Tx transient period requirement for TDD BS.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, RAN4#110-bis, Samsung.

- TR 38.858, “Study on evolution of NR duplex operation”, 3GPP.