3GPP TSG-RAN WG4 Meeting # 111		R4-2408419

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:		7.16.6.1

Source:		Samsung

Title:	Discussion on RRM core maintenance for NTN in above 10GHz

Document for:		Discussion

- Introduction

In RAN4#110-bis meeting, the WF has been approved in [1]. However, there are still some open issues for NTN RRM requirements in above 10GHz band which has been mentioned to be checked in this meeting. In this contribution, we further discuss the open issues for RRM requirements in above 10GHz band.

- Discussion

For UL timing requirements in bands above 10 GHz, it is said that it needs “further check and come back” as below:

### Issue 1-6A: Te_NTN for 60kHz and 120kHz in Case2 Agreement: (ad-hoc) - Further check and come back in next meeting ### Issue 1-6B: Te_NTN for 60kHz and 120kHz in Case3 Agreement: (ad-hoc) - In Case-3, Te_NTN [Ts] for 120kHz UL SCS is: - [10] <Note: No update was made compared RAN4#110 agreement>

For Te_NTN for 60kHz and 120kHz in Case2, it has been agreed in RAN4#110 meeting in [2]:

### Issue 1-6A: Te_NTN for 60kHz and 120kHz in Case2 Agreement: (ad-hoc agreement) - For 120kHz of UL SCS in case2, the requirements are applicable only if the ephemeris information be refreshed (i.e. update rate of ephemeris information in SIB19) at least every X seconds. - X= [7] s

In RAN4#110-bis meeting, there was one proposal of removing the side condition above.

According to the methodology in Rel-17 NTN and previous discussion on uplink timing in Rel-18 NTN, Te_NTN consists legacy Te and Tp = Tp,ue + Tp,sat. Tp, ue and Tp,sat are defined as a round trip propagation delay estimation error due to UE position and satellite position estimation errors.

Case 1: it is Fixed VSAT+GSO.

Case 2: it is Fixed VSAT+NGSO.

For the component of Tp, ue, the same Fixed VSAT has the same GNSS accuracy.

If the Te_NTN in Case 2 is expected be the same as that in Case 1. The only possibility is to enhance the accuracy of Tp, sat. The satellite position estimation error is from UE estimation of satellite position. In Rel-17 NTN discussion, the ephemeris information is important for the accuracy. Update ephemeris information more often or reduce the validity time of the ephemeris information can decrease the satellite position estimation error. That is why such side condition has been introduced to enhance the satellite position estimation error. If the NW configures the update of SIB19 in a longer time, the timing requirements are not applicable. The performance degradation is expected.

Proposal 1: Support to keep the current side condition for case 2.

Companies to further check the following options taking into account RAN2 spec: - For Type 2 UE (mechanical steering antenna), - Option 1: Define inter-satellite RRC re-establishment requirement. - <Exact requirement has to be filled in here> - Inter-satellite RRC re-establishment requirement is applicable only when the cause for the RRC re-establishment is an inter-satellite HO failure. Intra-satellite requirements are not applicable in this scenario. - For the other scenarios (i.e. the cause for the RRC re-establishment is not an inter-satellite HO failure), RRC re-establishment requirements are only applicable for intra-satellite re-establishments. - Option 2: Intra-satellite RRC re-establishment requirements are not applicable for the case RRC re-establishment is caused by an inter-satellite HO failure.

In last meeting, the RRC re-establishment were discussed although there were the agreements in RAN4#109 as:

- For type 1 UE (electronical steering antenna) - Inter-satellite RRC re-establishment: - No RRC re-establishment requirements - For type 2 UE (mechanical steering antenna) - Inter-satellite RRC re-establishment - No RRC re-establishment requirements

For Type 2 UE, RAN4 defined the blind handover requirements.

If a Type 2 UE turns to the direction of target satellite, then perform blind handover, the handover failed by some reasons such as the incorrect HO parameters or the target satellite has a worse condition. We check with RAN2 spec, there are not additional specific procedures which are the same as legacy RRC Re-establishment procedure. In this scenario, because the Type 2 UE has already turned to the direction of target satellite, the intra-satellite RRC re-establishment are not applicable for such case.

According to RAN2 procedure, UE initiation procedure, UE shall stop timer T304 and start timer T311.

The UE tried to selects a suitable NR cell. But currently the source cell has already been out of the direction of UE. UE has to search a suitable cell in target satellite. If T311 expiry (cannot find a suitable cell in target satellite), UE will go to RRC_IDLE and perform cell selection procedure.

Proposal 2: When Type 2 UEs perform inter-satellite HO and encounter inter-satellite HO failure, intra-satellite RRC re-establishment requirements are not applicable. UEs try to search a suitable NR cell of target satellite, if T311 expiry, UEs will go to RRC_IDLE and perform cell selection procedure.

- Conclusion

In this contribution, we provide our proposals for NTN RRM requirements in above 10GHz band:

Proposal 1: Support to keep the current side condition for case 2.

Proposal 2: When Type 2 UEs perform inter-satellite HO and encounter inter-satellite HO failure, intra-satellite RRC re-establishment requirements are not applicable. UEs try to search a suitable NR cell of target satellite, if T311 expiry, UEs will go to RRC_IDLE and perform cell selection procedure.

- Reference

[1] R4-2406430, WF on RRM requirements for NR_NTN_enh, Qualcomm

[2] R4-2403492, WF on RRM requirements for NR_NTN_enh, Qualcomm