3GPP TSG-RAN4 Meeting #110-bis	R4-2408438

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues SSBless SCell operation

Agenda item:	7.24.2.1

Document for:	Discussion

# Introduction

In this paper, we discuss about the remaining issues on SSBless SCell operation.

# Discussion

- Agreement on Test configurations for SSB-less - Single P-TRS is configured in SSB-less SCell in the SSB-less SCell activation TCs - Don’t configure the parameter SCell measurement cycle (measCycleSCell), SSB configuration and SMTC configuration for SSB-less SCell - Set EPRE difference as [9 dB] + [ΔPL] in the test cases. - the [9] can be updated to the same number as in core requirement if any updates on the core requirements. - Pcell has [higher or lower] EPRE than Scell. - Further discuss whether and how [ΔPL] can be reflected in the TC. - ΔPL is the power difference caused by BW difference and/or frequency difference between two Cells. - Encourage concrete proposals on how ΔPL due to BW difference and PL difference can be reflected in the test. - The TCI.State.0 could be used for the TRS of the SSB-less SCell with an additional note that the reference signal is SSB0 from spCell.

During the last discussion, there was some discussion about pathloss in test case design. Since it is not clearly defined in RRM requirement the definition of “after the compensation of AGC”, the test case should not discuss adding pathloss in conducted test. Different UE has different algorithm for AGC compensation which may not related to PL. It is not valid assumption that PL compensation is required for this test.

Observation: “after the compensation of AGC” is not clear definition. Different UE has different algorithm for AGC compensation which may not related to PL. It is not valid assumption that PL compensation is required for this test. Therefore, test case should not consider arbitrary pathloss for conducted test.

Proposal : RAN4 do not introduce pathloss margin [ΔPL].

We don’t have strong view as long as SNR for both reference Cell and SSBless SCell are reasonable but prefer to configure transmit power for TRS of the SSBless SCell is higher than SSB of the reference cell.

Proposal: Prefer to configure higher transmit power for TRS of the SSBless Scell than SSB transmit power of the reference cell

# Conclusions

Observation: “after the compensation of AGC” is not clear definition. Different UE has different algorithm for AGC compensation which may not related to PL. It is not valid assumption that PL compensation is required for this test. Therefore, test case should not consider arbitrary pathloss for conducted test.

Proposal : RAN4 do not introduce pathloss margin [ΔPL].

Proposal: Prefer to configure higher transmit power for TRS of the SSBless Scell than SSB transmit power of the reference cell

# References

[1] R4-2403526, WF on RRM requirements for NR network energy saving, Huawei, RAN4 #110

[2] R4-2406301, WF on RRM requirements for NR network energy saving, Huawei, RAN4 #110bis