3GPP TSG-RAN WG4 Meeting #111	R4-2408609

Fukuoka, Japan, 20th – 24th May, 2024

Source:	Ericsson

Title:	Discussion on reply LS to RAN2 on RRM enhancements for NR FR2 HST

Agenda item:	7.7.1

Document for:	Discussion

# 1	Introduction

In RAN2#125bis meeting, a LS was sent to RAN4 [1]:

… 1. Overall Description: In RAN2 #125, RAN2 agreed a TS38.331 CR (R2-2401565) to support RRM enhancement for Rel-18 NR FR2 HST. For the intra-frequency measurement enhancement on SCC, RAN2 agreed to use the existing RRC configuration parameter highSpeedMeasFlagFR2 in serving cell configuration of each SCell. During RAN2 discussion, it was questioned whether gNB should enable RRM enhancements for all serving cells in the same frequency band (i.e. intra-band CA) in consistent manner. That is, it is whether the presence of highSpeedMeasFlagFR2 should be the same for all serving cells in the same frequency band. For example, if highSpeedMeasFlagFR2 is present for PCell, highSpeedMeasFlagFR2 should be present to all SCells in the same frequency band as PCell. Some companies think that given that HST is a dedicated deployment, it is likely that all carriers in the same frequency band in CA are deployed for HST. In this case, if the same HST configuration is applied for all serving cells in the same frequency band, unnecessary complexity in UE implementation could be avoided. 2. Actions: To RAN4 group: ACTION: RAN2 respectfully ask RAN4 whether gNB should enable RRM enhancements for all serving cells in the same frequency band (i.e. intra-band CA) in consistent manner. …

In this paper we analyze the issue brought up by the LS and propose an LS reply.

# 2	Discussion

In TS 38.133 V18.5.0, we use highSpeedMeasFlagFR2-r17 to indicate the HST requirements for SpCell and SCell. For SCell, if UE support [measurementEnhancementCAInterFreqFR2-r18] and highSpeedMeasFlagFR2-r17 is configured, then the HST requirements shall be applied to SCell. One of examples in TS 38.133 V18.5.0 is copied in the below table.

- T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement - For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TSSB_measurement_period_intra is given in Table 9.2.5.2-7; otherwise, T SSB_measurement_period_intra is given in Table 9.2.5.2-2. - For UE indicating no-gap-no-interruption, TSSB_measurement_period_intra is given in Table 9.2.5.2-1 for FR1 and Table 9.2.5.2-2 for FR2. For UE indicating no-gap-with-interruption, TSSB_measurement_period_intra is given in Table 9.2.5.2-10 for FR1 and Table Table 9.2.5.2-11 for FR2. - For power class 6 UE supporting [measurementEnhancementCAInterFreqFR2-r18] when [highSpeedMeasFlagFR2] is configured, the T SSB_measurement_period_intra given in Table 9.2.5.2-7 (if SMTC <= 40ms) and Table 9.2.5.2-2 (if SMTC > 40ms) shall apply for SCC.

In current TS38.331 V18.1.0, as copied as follows, ‘highSpeedMeasFlagFR2-r17’ has not limited the usage in SpCell and SCell, on the contrary, which has clear differentiation in HST FR1 by different signallings.

In FR2, highSpeedMeasFlagFR2-r17:

In FR1, highSpeedMeasCA-SCell-r17:

We understand that currently the signaling ‘highSpeedMeasFlagFR2-r17’ can be configured to a UE on SpCell and SCell respectively, which follow the same rule for FR1, e.g. separate configurations on SpCell and SCell. But different from FR1, since ‘highSpeedMeasFlagFR2-r17’ always shall be configured on SpCell firstly before CA is configured and activated, the case that SpCell doesn’t configure ‘highSpeedMeasFlagFR2-r17’ but SCell configures ‘highSpeedMeasFlagFR2-r17’ shall be precluded firstly. Secondly, upon ‘highSpeedMeasFlagFR2-r17’ being configured on SpCell, ‘highSpeedMeasFlagFR2-r17’ is configured or not configured on SCell, the UE may fulfill the same HST requirements on SCell, since it may be assumed that SCell and SpCell are co-located, it doesn’t introduce UE’s complexity additionally.

Given that, we suppose there are below options to align RAN4’s understanding and RRC message IE definition in RAN2.

Option 1: Define ‘highSpeedMeasFlagFR2-r17’ only for SpCell.

Option 2: Define ‘highSpeedMeasFlagFR2-r17’ at the least for SpCell.

Option 3: Define ‘highSpeedMeasFlagFR2-r17’ for SpCell and SCell respectively.

Observation 1: There are the below options to align RAN4’s understanding and IE definition in RRC message in RAN2.

Option 1: Define ‘highSpeedMeasFlagFR2-r17’ only for SpCell.

Option 2: Define ‘highSpeedMeasFlagFR2-r17’ at the least for SpCell.

Option 3: Define ‘highSpeedMeasFlagFR2-r17’ for SpCell and SCell respectively.

To our understanding, Option 1 is enough, but we’re fine with Option 3, which is in line with FR1 method in which signalings are separately for each cell.

# 3 	Summary

Observation 1: There are the below options to align RAN4’s understanding and IE definition in RRC message in RAN2.

Option 1: Define ‘highSpeedMeasFlagFR2-r17’ only for SpCell.

Option 2: Define ‘highSpeedMeasFlagFR2-r17’ at the least for SpCell.

Option 3: Define ‘highSpeedMeasFlagFR2-r17’ for SpCell and SCell.

To our understanding, Option 1 is enough, but we’re fine with Option 3, which is in line with FR1 method in which signalings are separately for each cell.

# References

[1] R2-2403963, LS on RRM enhancements for NR FR2 HST, Samsung

# Appendix draft LS reply

3GPP TSG-RAN WG4 Meeting #111		R2-xxxx

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title:	Draft Reply LS to RAN2 on RRM enhancements for NR FR2 HST

Response to:	R2-2403963

Release:	Release 18

Work Item:	NR_HST_FR2_Enh

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:

Name:	Ming Li

E-mail Address:	ming.l.li@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1 Overall description

RAN4 would like to thank RAN2 for their LS R2-2403963.

RAN4 has discussed whether gNB should enable RRM enhancements for all serving cells in the same frequency band (i.e. intra-band CA) in consistent manner when operating NR FR2 HST. Based on the common understanding in RAN4, once the signalling ‘highSpeedMeasFlagFR2’ is configured to a UE on ﻿SpCell, then the UE shall fulfill the same HST requirements on SCell as well. Even we haven’t precluded ‘highSpeedMeasFlagFR2’ configuration on SCell, since ‘highSpeedMeasFlagFR2’ always shall be configured on ﻿SpCell firstly, either ‘highSpeedMeasFlagFR2’ is configured or not configured on SCell, the UE shall fulfill the same HST requirements on SCell.

# 2 Actions

To RAN2:

ACTION: 	RAN4 respectfully requests RAN2 to take the above feedback into consideration.

# 3 Dates of next TSG-RAN WG2 meetings

RAN4#112	from 2024-08-19 to 2024-08-23		Maastricht, Netherlands

RAN4#112bis	from 2024-10-14 to 2024-10-18		China (tbc)