3GPP TSG-RAN WG4 Meeting #111	R4-2408613

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	7.17.1.2

Source:	Intel Corporation

Title:	Power boosting features applicability to DL/UL CA scenarios

Document for:	Discussion

# Introduction

The updated UE feature list for Rel-18 NR coverage enhancements work item was agreed in RAN4 #110 [1]. The majority of open issues were resolved, however, there are still open issues on applicable scenarios for power boosting (PAR/MPR enhancements) features and, especially, with respect to CA scenarios applicability as described in WF [2]. Additional discussion took place in RAN4 #110bis with agreements captured in WF [3] and in chair notes.

RAN4#110: UE feature list [1] RAN4#110: WF R4-2403885 [2] - RAN4 is still discussing the applicable scenarios. - The common understanding in RAN4 is that power boosting is only supported for single CC UL at least in Rel-18. - UL CA is not supported for this feature in Rel-18. RAN4#110bis: WF R4-2406713 [3] - RAN4 further discuss the applicability of the power boosting feature related to CA configuration below: - Case A: FR1 CA with DL CA combination configured and with a single uplink CC used for transmission. - Issue 1: which applicable power class should be based on for CA configuration - Applicable power class type, configured output power requirements and criteria for the band activated with power boosting for the case - The power class indication for the single uplink CC, which is the basic to enable power boosting, is subject to the discussion outcome of NR_power_class thread. - Issue 2: MSD impact due the enabling of the power boosting feature for case A - Option 1: No new MSD requirements due to the Rel-18 power boosting feature will be specified in RAN4 specification. - UE will be verified with MSDs defined for the reported power class and corresponding power configurations in current spec - how to capture this in RAN4 specification is FFS - Option 2: MSD can be specified after justification of new test cases - enable combinations which shall be subject to power boost case by case. Basket approach is used similar to the basket approach used for introducing new band combinations. Requirements can be checked. RAN4#110bis: Main session chair notes - RAN4 further discuss the applicability of the power boosting feature related to CA configuration below: - Case B: FR1 inter-band UL CA, at least one indicated band supports the power boosting, where a single CC is used for transmission in each power boosted uplink band. - Case C: FR1+FR2 UL CA, FR1+FR2 DC, FR1+FR1 DC, power boosting feature is supported in an FR1 NR band, where a single CC is configured in this uplink band.

This contribution provides additional views on the applicable scenarios for Rel-18 power boosting features powerBoosting-pi2BPSK-QPSK-r18 and powerBoosting-pi2BPSK-QPSK-r18 and specifically on DL/UL CA applicability.

# Discussion

## General

During the WI stage the focus of technical analysis was primarily on enhancing power for UL single carrier scenarios. However, the comprehensive discussion on CA aspects was somewhat lacking and an initial discussions took place in maintenance stage only on RAN4 #110 and #110bis meetings. We believe it is crucial to address this gap, particularly in clarifying and finalizing UE behavior concerning DL and UL CA support/configuration.

It is essential to recognize that the continuation of work on power enhancements in future releases appears improbable (e.g. no relevant objectives in Rel-19 work package). Therefore, it is helpful to seize the opportunity to define proper UE behavior concerning DL and UL CA support at this point to maximize opportunities for the commercial feature adoption rather than leaving this undefined.

The key motivation behind supporting power boosting feature is to improve coverage performance, particularly for UEs operating at cell edges and utilizing lower-order modulations such as pi/2 BPSK and QPSK. However, it is crucial to note that being configured with coverage enhancements features not necessarily means that UE operates in single-carrier mode. In dynamic propagation conditions, networks may need to configure UEs to operate in CA mode consistently, while dynamically scheduling single-carrier or multi-carrier transmissions based on instantaneous propagation and traffic conditions. Limiting power boosting exclusively to single carrier case might impede network flexibility, potentially resulting in frequent UE CA (re)configurations or not enabling the feature in practical deployments.

Observation #1:	Limiting power boosting exclusively to single carrier case might impede network flexibility, potentially resulting in frequent UE CA (re)configurations or not enabling the feature in practical deployments.

Therefore, we see a value in enabling the power boosting feature in both DL and UL CA scenarios if it does not require substantial extra RAN4 work on the associated requirements definition.

In RAN4 #110bis meeting several candidate scenarios were identified:

- Case A: FR1 CA with DL CA combination configured and with a single UL CC used for transmission.

- Case B: FR1 inter-band UL CA, at least one indicated band supports the power boosting, where a single CC is used for transmission in each power boosted uplink band.

- Case C: FR1+FR2 UL CA, FR1+FR2 DC, FR1+FR1 DC, power boosting feature is supported in an FR1 NR band, where a single CC is configured in this uplink band.

Overall, Cases A/B/C are not fully mutually exclusive.  Case A considers scenarios where FR1 CA is configured for DL with a single UL CC used for transmission. This case emphasizes the need for power boosting in DL CA combinations but does not exclude the possibility of UL CA. The ambiguity in Case A arises from the "single UL CC used for transmission," which could mean either 1) a single UL CC is configured or that 2) UL CA is configured but only one UL CC is used at a time. Meantime, Cases B and C are specifically focused on UL CA scenarios and partially overlap with Case A. Case B focuses on inter-band UL CA within FR1, a single CC is used for UL transmission in each power-boosted uplink band. Case C extends the discussion to configurations involving both FR1 and FR2, including dual connectivity (DC) scenarios.

## FR1 CA/DC scenarios with single UL CC

Per discussion in RAN4 #110bis the applicability of power boosting features for the case UE is configured with DL CA/DC is not clearly defined. Same time we note that DL CA is the basic scenario and excluding such use case from the consideration makes all the work on the Rel-18 power boosting feature absolutely meaningless. The support of power boosting on a single UL CC in FR1 is not expected to have impact on UL TX requirements and the implementation is identical to the case of single DL/UL CC. From the DL requirements perspective, a certain impact on MSD performance was mentioned by companies in RAN4 #110bis. However, the existing MSD requirements already have a certain margin, which can accommodate a limited power boosting introduced in Rel-18 work item scope. Additionally, the agreed UE capabilities framework provides sufficient flexibility for UE to avoid declaring the support for specific band combination if any performance issues are expected. Therefore, it is possible to enable the support of power boosting operation without introducing any new requirements and the UEs declaring the support will be expected to meet the existing MSD requirements.

Proposal #1:	Enable power boosting enhancements/features for FR1 CA/DC scenarios with single configured UL CC including FR1 intra-band / inter-band CA and DC scenarios.

## FR1 CA/DC scenarios with inter-band UL CA

In RAN4 #110bis the following scenario was identified “Case B: FR1 inter-band UL CA, at least one indicated band supports the power boosting, where a single CC is used for transmission in each power boosted uplink band.” As mentioned above limiting power boosting exclusively to single UL carrier case might impede network flexibility, potentially resulting in frequent UE CA (re)configurations or not allowing efficient use of the feature in practical deployments. Therefore, it is important to enable UL CA scenarios as well.

Two general approaches to enable inter-band UL CA operation can be considered when N > 1 uplink CCs are configured:

- Case B-1:  A single UL CC is used for transmission in each power boosted uplink band (i.e., N > 1 simultaneous UL transmission)

The respective scenario provides the best flexibility in terms for practical deployments as it allows simultaneous transmissions of N > 1 UL CCs in different bands. From the requirements perspective it is expected that the existing requirements can apply and similar to scenarios with single UL CC we do not see a strong need to define new MSD requirements. The support of respective scenarios can be left up to UE implementation/capability and there is no need to artificially preclude it.

- Case B-2:  A single UL CC is used for transmission across all UL CCs (i.e., no simultaneous UL transmissions)

Acknowledging the complexities related to the support of simultaneous UL transmissions with enabled power boosting, as an alternative it is possible to consider certain scheduling restrictions when only a single uplink CC out of N > 1 configured CCs is used for power boosted transmissions. One possible way to enable such restriction is to consider using power boosting in case UL transmissions are scheduled on a single carrier within a specific slot (e.g., PUSCH/PUCCH is scheduled on one carrier, while other carriers do not have active UL transmission). This approach ensures that network can seamlessly transition UE operation between single-carrier and multi-carrier transmissions as dictated by dynamic propagation conditions, without the need for CA reconfigurations, while the implementation complexity of power boosting remains approximately same as for the single carrier case. A simplified example of such approach is provided in Figure 1, when UL CA has 2 component carriers, and the power boosting is applied depending on whether UL transmissions are scheduled in one or both carriers:

Figure 1. Power boosting for UL CA scenarios.

Taking into account potential benefits of simultaneous UL CC transmissions, we recommend considering Case B-1 for Rel-18 timeframe. In case potential issues with UE implementations are identified, additional UE capability to support case B-2 can be considered.

Proposal #2:	Enable power boosting enhancements/features for FR1 CA/DC scenarios with inter-band UL CA with at least one indicated band supporting the power boosting, where a single CC is used for transmission in each power boosted uplink band.

## FR1 CA/DC scenarios with intra-band UL CA

The scenarios with intra-band UL CA may indeed be problematic and the support of simultaneous UL transmissions within a single band may require adjustment of RF requirements. To enable such use case scheduling restrictions as described in section 2.3 can be considered. As an alternative, the scenario can be considered in the scope of Rel-19 UE RF work item as a part of objective to “Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration” under an assumption that a single UL CC is activated.

Proposal #3:	Further discuss applicability power boosting enhancements/features for FR1 CA/DC scenarios with intra-band UL CA under assumption of a single UL CC used for transmissions (based on scheduling restrictions or for single active CC case).

## FR1-FR2 CA/DC scenarios

The “Case C: FR1+FR2 UL CA, FR1+FR2 DC, FR1+FR1 DC, power boosting feature is supported in an FR1 NR band, where a single CC is configured in this uplink band” is relatively straightforward as no impact is expected for FR1/FR2 operation.

Proposal #4:	Enable power boosting enhancements/features for FR1-FR2 CA/DC scenarios with at least one indicated FR1 band supporting the power boosting, where a single FR1 UL CC is used for transmission in uplink FR1 band.

## UE capabilities signalling

Based on current assumptions UE reports the power boosting capabilities with “per FS” (per band per band combination) granularity, which already provides sufficient flexibility for UE to report the respective capabilities and no changes are needed to support power boosting for UL CA. The proposed UE feature list is provided below:

| Features | Index | Feature group | Components | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 41.NR_cov_enh2 | 41-2 | Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions without modified spectrum flatness requirement | 1. Support of UE power boosting for DFT-s-OFDM pi/2 BPSK and QPSK without modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1.The power boosting is only enabled when signalled via RCC powerBoostPi2BPSKRel18 for BPSK and powerBoostQPSKRel18 for QPSK | Per FS | FFS – RAN4 is still discussing the applicable scenarios The feature is applicable for the following scenarios: 1) single carrier DL/UL in FR1 bands 2) FR1 CA/DC with single UL carrier 3) FR1 CA/DC with inter-band UL CA with at least one indicated band supporting the power boosting, where a single CC is used for transmission in each power boosted uplink band. 4) FR1-FR2 CA/DC with at least one indicated FR1 band supporting the power boosting, where a single FR1 UL CC is used for transmission in uplink FR1 band. FFS for FR1 CA/DC with intra-band UL CA | Optional with capability signalling |
| 41.NR_cov_enh2 | 42-3 | Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions with modified spectrum flatness requirement shaping | 1. Support of UE power boosting for DFT-s-OFDM pi/2 BPSK and QPSK with modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1. The power boosting is only enabled when signalled via RCC powerBoostPi2BPSKRel18 for BPSK and powerBoostQPSKRel18 for QPSK | Per FS | FFS – RAN4 is still discussing the applicable scenarios The feature is applicable for the following scenarios: 1) single carrier DL/UL in FR1 bands 2) FR1 CA/DC with single UL carrier 3) FR1 CA/DC with inter-band UL CA with at least one indicated band supporting the power boosting, where a single CC is used for transmission in each power boosted uplink band. 4) FR1-FR2 CA/DC with at least one indicated FR1 band supporting the power boosting, where a single FR1 UL CC is used for transmission in uplink FR1 band. FFS for FR1 CA/DC with intra-band UL CA | Optional with capability signalling |

# Conclusion

In this contribution we provided views on provides views on the applicability of Rel-18 power boosting features powerBoostRel18 and powerBoostTSRel18 and to UL CA scenarios and in summary make the following proposals:

Proposal #1:	Enable power boosting enhancements/features for FR1 CA/DC scenarios with single configured UL CC including FR1 intra-band / inter-band CA and DC scenarios.

Proposal #2:	Enable power boosting enhancements/features for FR1 CA/DC scenarios with inter-band UL CA with at least one indicated band supporting the power boosting, where a single CC is used for transmission in each power boosted uplink band.

Proposal #3:	Further discuss applicability power boosting enhancements/features for FR1 CA/DC scenarios with intra-band UL CA under assumption of a single UL CC used for transmissions (based on scheduling restrictions or for single active CC case).

Proposal #4:	Enable power boosting enhancements/features for FR1-FR2 CA/DC scenarios with at least one indicated FR1 band supporting the power boosting, where a single FR1 UL CC is used for transmission in uplink FR1 band.

# References

[1] R4-2403885, “WF on coverage enhancement for part 2”, Nokia, RAN4 #110, February 2024

[2] R4-2403885, “WF on coverage enhancement for part 2”, Nokia, RAN4 #110, February 2024

[3] R4-2406713, “WF on power boosting feature in Rel-18”, Ericsson, RAN4 #110bis, April 2024