3GPP TSG-RAN WG4 Meeting # 111	R4-2408640

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4519 | rev | - | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RRM_enh2-Core) CR on SRS AS interruption requirements R17 (Cat F) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In exising requirments, the interruption requirements for SRS AS for NR SA contains sync and async case, which was mistakenly added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the interruption requirements for SRS AS for NR SA into following two cases: - Interruption length in symbols of victim CC when 1 SRS symbol is configured - Interruption length in slots of victim CC for rest of the SRS configurations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

##### 8.2.2.2.16	Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot. For interruption caused by SRS antenna port switching, the victim cell is based on the entry number of the band indicated by txSwitchImpactToRx and/or txSwitchWithAnotherBand regardless of per-FR MG capability. An UL interruption is allowed on any of the serving cells as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells as indicated in txSwitchImpactToRx.

The UE shall perform SRS antenna port switching only if the below conditions are met.

-	 the SRS switching is not colliding with any other UL transmission with higher priority defined in TS 38.214 [26] if the serving cell on which the higher priority transmission is performed is a victim cell based on txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted.

-	the SRS switching is not colliding with any NR measurements (i.e. SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD if the serving cell on which the NR measurements and the measurements for RLM/BFD/CBD is performed is a victim cell based on txSwitchImpactToRx or is the same carrier on which SRS is transmitted.

No requirements are defined for SRS antenna port switching if aperiodic SRS switching is colliding with aperiodic L1-RSRP/L1-SINR measurements and the serving cell on which the aperiodic L1-RSRP/L1-SINR measurement is configured is indicated in txSwitchImpactToRx or is the same carrier on which aperiodic SRS is scheduled/configured.

When 1 SRS symbol is configured in a slot for SRS antenna switching, the interruption requirement in Table 8.2.2.2.16-1 applies. For the rest of SRS configurations, the interruption requirement in Table 8.2.2.2.16-2 applies.

Table 8.2.2.2.16-1: Interruption length in symbols of victim CC when 1 SRS symbol is configured

| Victim cell SCS(kHz) | Aggressor cell SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 3 | 2 | 2 |
| 30 | 4 | 3 | 3 |
| 60 | 8 | 6 | 5 |
| 120 | 14 | 10 | 8 |

Table 8.2.2.2.16-2: Interruption length in slots of victim CC for rest of the SRS configurations

| Victim cell SCS(kHz) | Aggressor cell SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

### <End of Change 1>