3GPP TSG-RAN WG4 Meeting #111                                                           R4-2408743

Fukuoka City, Fukuoka, Japan, 20th – 24th May 2024

Agenda Item:	7.11.2.1

Source:	Ericsson

Title:	On UE capability aspects

Document for:	Discussion

# 1	Introduction

In this contribution, we provided our view on the proposals for UE capability aspects related.

# 2	Discussion

Updates to UE capability definition

There is proposal on updating the UE capability definition as specified in the following:

- For 36-2a: - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and 4RX] when co-scheduled UE(s)’ modulation order is not signalled DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled. - For 36-2b: - R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX] when co-scheduled UE(s)’ modulation order is not signalled DCI index 6 in Table 7.3.1.2.2-12 of TS38.212 is signalled. - Proposals on 36-2a and 36-2b: - Proposal 1: Combine 36-2a and 36-2b and remove number of layer descriptions if RAN4 agrees to not define 2+2 test under DCI 6 - Proposal 2: Remove FG 36-2b and keep 36-2a from UE feature list

Now it’s undetermined whether to remove the test cases of rank 2+2 with MOBD. If in the end there is no test cases of rank 2+2 with MOBD, we support to remove the FG 36-2b from the list as it is indicating the scenario beyond 2 layers that without a definition of test. Thus, we propose the following:

Proposal 1: Remove FG 36-2b and keep 36-2a only from the UE feature list if RAN4 agrees to not define 2+2 test under DCI 6

Regarding the modified texts in 36-2a, it is mentioned ‘DCI index 6 or 7 in Table 7.3.1.2.2-12…’. According to TS38.212 V18.2.0, the title of Table 7.3.1.2.2-12 is ‘Co-scheduled UE information’. We think ‘DCI index 6’ is confused, and therefore we propose to revise to ‘The co-scheduled UE information index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signaled.’

Proposal 2: Revise the texts in FG 36-2a as follows:

From: “… DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled.”

To: “… The co-scheduled UE information index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled.”

# 3	Summary

In this contribution, we provided our view on the issue of UE capability aspects. To summarize, we proposed the following:

Proposal 1: Remove FG 36-2b and keep 36-2a only from the UE feature list if RAN4 agrees to not define 2+2 test under DCI 6

Proposal 2: Revise the texts in FG 36-2a as follows:

From: “… DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled.”

To: “… The co-scheduled UE information index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 is signalled.”

# References

[1]	R4-2406114: WF on the advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #110bis, Changsha, China