3GPP TSG-RAN WG4 Meeting #111                                                            R4-2408746

Fukuoka City, Fukuoka, Japan, 20th – 24th May 2024

Agenda Item:	7.16.8.2

Source:	Ericsson

Title:	On UE demodulation requirement for NTN enhancement

Document for:	Information

# 1	Introduction

Good progress has been made during the last RAN4 #110bis meeting in Changsha. For the Channel model, it’s agreed to consider 1200Hz Doppler spread for both UL and DL. [1]

Issue 1-1: Doppler Spread for both UL and DL - Agreement - 1200Hz

As for the UE demodulation requirements, test case for PDSCH and PDCCH have been agreed as follows [1]:

Issue 2-1: PDSCH demodulation requirements for above 10 GHz bands - Agreement: Prop. Channel; MCS; HARQ Config. NTN-TDLC5-1200; MCS4; Disabled HARQ (4/16). MCS4; 16 HARQ Proc. NTN-TDLC5-1200; MCS4; 32 HARQ Proc. MCS13; 16 HARQ Proc. Issue 2-3: PDCCH demodulation requirements for above 10 GHz bands - Agreement - AL8 and AL16, with the following test configurations Case; Bandwidth (MHz); SCS (kHz); CORESET RB; CORESET duration; Aggregation level; CCE to REG mapping type; REG bundle size; Interleaver size; Shift index; DCI format; Payload (without CRC). 1; 200; 120; 60; 1; 8; Interleaved; 2; 3; 0; [1-0]; [40]. 2; 200; 120; 60; 2; 16; Interleaved; 2; 3; 0; [1-1]; [56].

Corresponding parameters are decided with [8] for K_offset and 1T1R for antenna configuration.

Applicability rule for all NTN UE demodulation tests has been agreed as the following structure leaving clause number as TBD.

Issue 2-5: Applicability rule - Agreement - All demodulation performance requirements for NTN enhancements are applicable for Type 1 fixed and Type 2 mobile VSAT UEs UE feature/capability [TBD]; Test type; Test list; Applicability notes. NR NTN access (nonTerrestrialNetwork-r17); FR2-NTN; PDSCH; Clause x (Testxx). PDCCH; Clause y (Test 1, Test 2). Increasing the number of HARQ processes (max-HARQ-ProcessNumber-r17); FR2-NTN; PDSCH; Clause x (Testxx). Disabled HARQ feedback for downlink transmission (harq-FeedbackDisabled-r17); FR2-NTN; PDSCH; Clause x (Testxx).

As we reviewed and performed the PDCCH simulations, we found some parameters may need to be reconsidered, which is discussed in this contribution with our proposal at the end.

# 2	Discussion

As indicated in the agreed simulation assumption [2], we found that the CORESET RB and the payload size (without CRC) are set to 60 and 40.

According to the PRB allocation calculation and the reference in Annex of TS38.101-4, the CORESET RB should be 132 for specifically 200MHz/120 BW/SCS combination. Thus, we propose to configure 132 for the CORESET RB in the parameter assumption table. Thus, we proposed the following:

Proposal 1: Use 132 for the CORESET RB of PDCCH test cases

Regarding the payload for Format 1_0, we re-calculated it based on the formula listed in the 7.3.1.2.1 in TS38.212:

- Frequency domain resource assignment - bits where is given by Clause 7.3.1.0

, where we use 132 as the value of  . By summing the other fixed bits, the total payload bit (without CRC) should be 42 bits, which means that the original 40 bits should be revised. Same reason for the payload for Format 1_1. Thus, we proposed the following:

Proposal 2: Use 42 bits for the payload of PDCCH test cases

Based on the belief of above analysis, we performed our PDSCH and PDCCH simulation with proposed value and provided the results in our accompanied contribution [3].

# 3	Summary

In this contribution, we provided our analysis on some parameter assumptions for PDCCH test cases.

We summarized our proposals as follows:

Proposal 1: Use 132 for the CORESET RB of PDCCH test cases

Proposal 2: Use 42 bits for the payload of PDCCH test cases

# References

[1]	R4-2406024: WF on NR_NTN_enh_SAN_UE_demod, Huawei, HiSilicon, RAN4 #110bis, Changsha, China

[2]	R4-2405146: Simulation assumption on demodulation requirements for NR NTN enhancements, Huawei, HiSilicon, RAN4 #110bis, Changsha, China

[3]	R4-2408747: Simulation results for NR NTN enhancement UE demodulation, Ericsson, RAN4 #111, Fukuoka City, Fukuoka, Japan