3GPP TSG-RAN WG4 Meeting #111                                                            R4-2408747

Fukuoka City, Fukuoka, Japan, 20th – 24th May 2024

Agenda Item:	7.16.8.2

Source:	Ericsson

Title:	Simulation results for NR NTN enhancement UE demodulation requirement

Document for:	Information

# 1	Introduction

Several agreements related to the test parameter have been made for both PDSCH and PDCCH in RAN4 #110bis meeting in Changsha, China. In this contribution, we provided our simulation results for PDSCH and PDCCH based on newly agreed configurations and our understandings. For PDCCH, we evaluated the performance difference of configuring ‘interleaved’ and ‘non-interleaved’.  132 CORESET RB and 42 bits payload was used for PDCCH test 1 and 2, which are different from the agreed simulation assumption [2].

# 2	Discussion

## 2.1 PDSCH simulation

In this section, we provided our ideal and impairment results with updated parameter assumptions.

### 2.1.1 Parameter assumption

Table 2.1.1-1 Updated simulation parameters for NR NTN PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Center Frequency |  | GHz | 20 |
| Bandwidth |  | MHz | 200 |
| Subcarrier spacing |  | kHz | 120 |
| Duplex Mode |  |  | FDD |
| TDD Slot Configuration |  |  | N/A |
| Channel model |  |  | NTN-TDLC5-1200 1200 configured as Doppler spread |
| K_offset |  |  | [8] |
| MCS/Rank |  |  | Rank 1 MCS4 (QPSK 0.3) MCS13 (16QAM 0.48) |
| Antenna configuration |  |  | 1T1R |
| Transform precoding |  |  | CP-OFDM |
| HARQ | Maximum number of HARQ transmissions |  | 16/32 and disabled Disable HARQ: Set the number of HARQ Processes as: 4 with feedback disabled, 12 with feedback enabled in 16 HARQ processes with re-Tx disable for all HARQ processes and only transmit initial transmissions with NDI toggling. Throughput shall be measured on processes with HARQ enabled. Which 4 processes to disable are randomly select at test configuration. |
|  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type |  | 1 |
|  | DM-RS duration |  | single-symbol DM-RS |
|  | Additional DM-RS position |  | pos 1 |
|  | Number of DM-RS CDM group(s) without data |  | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  | DM-RS port |  | {0} |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment | PDSCH mapping type |  | A |
|  | Start symbol |  | 2 |
|  | Allocation length |  | 12 |
| Frequency domain resource assignment | RB assignment |  | 200MHz for 120kHz SCS |
| Test metric |  |  | 70% of max throughput at target SNR. |

### 2.1.2 Simulation results

Table 2.1.2-1 Simulation results for NR NTN PDSCH

| Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | k-factor(dB) | Antenna configuration / Rank |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | DM-RS Additional position | HARQ | MCS |  | Ideal SNR (dB) DShift = 1200 | Impairment SNR (dB) |
| 200MHz/120kHz | NTN-TDLC5- 1200 | 8.4 | 1x1, Rank 1 | 70 | Pos1 | Disabled | 4 |  | -0.10 | 1.9 |
|  |  |  |  |  |  | 16 HAQR | 4 |  | -1.17 | 0.83 |
|  |  |  |  |  |  | 32 HARQ | 4 |  | -1.17 | 0.83 |
|  |  |  |  |  |  | Disabled | 13 |  | 7.82 | 9.82 |
|  |  |  |  |  |  | 16 HAQR | 13 |  | 6.58 | 8.58 |
|  |  |  |  |  |  | 32 HARQ | 13 |  | 6.58 | 8.58 |

## 2.2 PDCCH simulation

In this section, we provided our simulation results based on newly agreed simulation assumptions [2] and our understanding.

### 2.2.1 Parameter assumption

Table 2.2.1-1 Parameter assumption for NR NTN PDCCH

| Parameter | Unit | Value |
| --- | --- | --- |
| Test metric |  | 1% BLER |
| Bandwidth |  | 200MHz |
| SCS |  | 120kHz |
| CORESET RB |  | 132 |
| Propagation condition |  | NTN-TDLC5-1200 |
| Antenna configuration |  | 1T1R, low |

### 2.2.2 Simulation results

Table 2.2.2-1 Simulation results for NR NTN PDCCH

| Test Num | CORESET duration | Aggregation level | CCE to REG mapping type | REG bundle size | Interleave size | Shift index | DCI format | Payload (without CRC) | SNR (dB) DShift = 1200 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1_1 | 1 | 8 | Interleaved | 2 | 3 | 0 | 1_0 | 42 | 1.24 |
| 1_2 | 1 | 8 | Interleaved | 6 | 2 | 0 | 1_0 | 42 | 1.21 |
| 2 | 1 | 8 | Non-Interleaved | / | / | 0 | 1_0 | 42 | 1.22 |
| 3_1 | 2 | 16 | Interleaved | 2 | 3 | 0 | 1_1 | 56 | -0.29 |
| 3_2 | 2 | 16 | Interleaved | 6 | 2 | 0 | 1_1 | 56 | -0.29 |
| 4 | 2 | 16 | Non-interleaved | / | / | 0 | 1_1 | 56 | -0.3 |

Observation 1: With the same Aggregation level and CORESET duration, Interleaved compared to Non-Interleaved CCE-to-REG mapping does not result in a remarkable performance improvement.

Observation 2: From Aggregation level 8 to 16, an SNR improvement of around 1dB is achieved.

# 3	Summary

In this contribution, we provided our simulation results for NR NTN PDSCH and PDCCH demodulation requirements.

# References

[1]	R4-2406024: WF on NR_NTN_enh_SAN_UE_demod, Huawei, HiSilicon, RAN4 #110bis, Changsha, China.

[2]	R4-2405146: Simulation assumption on demodulation requirements for NR NTN enhancements, Huawei, HiSilicon, RAN4 #110bis, Changsha, China.