3GPP TSG-RAN WG4 Meeting #111	R4-2408763
Fukuoka, Japan, 20th – 24th May, 2024

Title:	R19 power enhance for single carrier

Source:	OPPO

Agenda item:	10.1.1.3.1

Document for:	Approval

- Introduction

Power enhancements for single carrier has been discussed in last meeting, and some scenarios were brought out, WF is as below. This paper discuss these aspects.

1.1 Scenarios for power domain enhancements for single carrier Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator) Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators) Scenario 2: Narrower UE channel BW within wider BS bandwidth Way forward: - Prioritize scenario 1-1 and scenario 2 for initial study of power domain enhancements for single carrier in terms of relaxed requirements - FFS on sub-scenarios of scenario 2. - Scenario 1-2 will be studied after scenario 1-1 and scenario 2 1.2. relaxed requirements from co-existence/regulation perspective Way forward: - For regions where at least ACLR and SEM can be relaxed for the identified scenarios, FFS whether SE could be relaxed, or under which conditions can be relaxed. 1.3 General considerations for power domain enhancements Way forward: - Only requirements relaxation should be considered for power domain enhancements in Rel-19 - only consider general requirements for the further evaluation - No power domain enhancements based on BWP is considered 1.4 Evaluation of relaxed requirements Way forward: - [No relaxation of ACLR/SEM/SE outside of the BS CBW for one operator holding spectrum for scenario 2, i.e. Narrower UE channel BW within wider BS bandwidth] - FFS whether outer, edge or inner RB allocation is prioritized for further evaluation - FFS impact on MPR by relaxing ACLR w/ or w/o relaxing SEM/Spurious Emissions - FFS whether IBE is considered instead of the relaxed ACLR/SEM/SE for the region between UE CBW and BS CBW.

- Discussion

2.1 Single operator in a band

The power enhancement by relaxing requirements has been brought out in last meeting, and mainly focus on two scenarios, one is only one operator in a band, the other scenario is two operators but with large enough guard band between them.

For these two scenarios, still there are ambiguities on which requirement should or can be applied before the analysis of how much power the UE can be boosted by relaxing the requirements and common understanding need to be achieved first.

Observation 1:   For the case of only one operator holding spectrum in a band, how the requirements can be relaxed need to be clear considering the regulation restrictions and also operator demands before power enhancement evaluations.

Below figures are some understandings of how requirements could be applied in different cases.

Figure 1 gives an example of n79 where operator A holds part of the spectrum and no other operators there. In this case, the possible requirements could be like “Requirement case A” or “Requirement case B”.

- Requirement Case A means the ACLR/SEM/SE emission requirements are relaxed to no requirement in the operator holding spectrum. And out of the operator holding spectrum, the ACLR/SEM/SE requirements are still applied if the requirement frequency range is overlapped with that.

- Requirement Case B means the ACLR/SEM/SE out of band emissions are relaxed to no requirement in the whole band. And ACLR/SEM/SE requirements are applied only outside of the band if the requirement frequency range is overlapped with that.

Among these two cases, Case A might be more feasible comparing to Case B considering the emissions are always be sensitive in regulation and it is more controllable inside the operator own spectrum once there is interference caused by this emission requirement relaxation.

Figure 1 Only one operator in a band

Observation 2:   For the case of only one operator in a band, relaxing requirements only inside the operator holding spectrum seems more cautious from regulation restriction and potential interference handling perspective.

Proposal 1:         For the case of only one operator in a band, relaxing emission requirements only inside the operator holding spectrum. For the outside of operator spectrum, ACLR/SEM/SE still applies when the frequency ranges are overlapped.

2.2 Adjacent operators in a band

For the scenario of more than one operator in a band and there is guard band between them, the possible emission requirement relaxation case A/B/C are given in below figure 2.

- Requirement Case A means the ACLR/SEM/SE emission requirements are relaxed to no requirement only in the operator holding spectrum. And out of the operator holding spectrum, the ACLR/SEM/SE requirements are still applied if the requirement frequency range is overlapped with that.

- Requirement Case B means the ACLR/SEM/SE emission requirements are relaxed to no requirement inside and between the operator holding spectrums. And outside of that, the ACLR/SEM/SE requirements are still applied if the requirement frequency range is overlapped with that.

- Requirement Case C means the ACLR/SEM/SE emission requirements are relaxed to no requirement inside and outside the operator holding spectrums but within the band as long as it is not overlapped with other operator spectrums. And outside of the band, the ACLR/SEM/SE requirements are still applied if the requirement frequency range is overlapped with that.

Similar consideration as single operator in a band scenario, case A here might be more cautious in emission relaxation considering it is always sensitive in regulations.

Figure 2 Two operator in a band with guard band

Observation 3:   Adjacent operator scenario is more complex than the single operator case, and it is more cautious to only relax the emission requirements inside operator holding spectrum.

Proposal 2:         For the case of adjacent operators in a band, relaxing emission requirements only inside the operator holding spectrum. For the outside of operator spectrum, ACLR/SEM/SE still applies when the frequency ranges are overlapped.

2.2 Regulation impacts

It needs to be noticed that nowadays most of the regulations require UE to meet the Tx emission requirements outside of its transmission bandwidth. It means that no matter the single operator in a band or adjacent operators in a band cases, UE still needs to meet all the Tx emission requirements/tests.

The power enhancement with emission relaxations defined here will only be an optional feature. It is expected that some NW indication to allow UE relax the emission requirements will be defined, however, UE must have ways to make sure the relaxation of emissions will not violate the regulations.

Observation 4:   In most of the regulations, UE is required to meet all the Tx emission requirements outside of its transmission bandwidth without relaxation. How much benefit this emission relaxation can be derived is unclear.

Proposal 3:         The power enhancement by emission relaxations shall be an optional feature if introduced. And UE must have ways to make sure the relaxation of emissions will not violate the regulations.

- Conclusion

This paper analyzed the power enhancements by emission relaxation in the scenario of single operator in a band or adjacent operators in a band.

Observation 1:   For the case of only one operator holding spectrum in a band, how the requirements can be relaxed need to be clear considering the regulation restrictions and also operator demands before power enhancement evaluations.

Observation 2:   For the case of only one operator in a band, relaxing requirements only inside the operator holding spectrum seems more cautious from regulation restriction and potential interference handling perspective.

Proposal 1:         For the case of only one operator in a band, relaxing emission requirements only inside the operator holding spectrum. For the outside of operator spectrum, ACLR/SEM/SE still applies when the frequency ranges are overlapped.

Observation 3:   Adjacent operator scenario is more complex than the single operator case, and it is more cautious to only relax the emission requirements inside operator holding spectrum.

Proposal 2:         For the case of adjacent operators in a band, relaxing emission requirements only inside the operator holding spectrum. For the outside of operator spectrum, ACLR/SEM/SE still applies when the frequency ranges are overlapped.

Observation 4:   In most of the regulations, UE is required to meet all the Tx emission requirements outside of its transmission bandwidth without relaxation. How much benefit this emission relaxation can be derived is unclear.

Proposal 3:         The power enhancement by emission relaxations shall be an optional feature if introduced. And UE must have ways to make sure the relaxation of emissions will not violate the regulations.