3GPP TSG-RAN WG4 Meeting #111	R4-2408764
Fukuoka, Japan, 20th – 24th May, 2024

Title:	R19 MPR applicability for FR1 intra-band UL CA

Source:	OPPO

Agenda item:	10.1.1.3.2

Document for:	Approval

- Introduction

The MPR reduction for FR1 intra-band UL CA has been discussed in last meeting with WF approved as below.

This paper discusses this aspect.

MPR applicability for FR1 intra-band contiguous UL CA Way forward: - [For PC3, the single CC MPR requirements (Table 6.2.2-1) can be applied to FR1 intra-band contiguous UL CA with only one UL CC activated. - For PC2, the single CC MPR requirements can be applied to FR1 intra-band contiguous UL CA with only one UL CC activated. If TxD is indicated for this intra-band contiguous ULCA, single CC with TxD MPR (Table 6.2D.2-1) should apply; if TxD is not indicated for this intra-band contiguous ULCA, single CC without TxD MPR (Table 6.2.2-2) should apply.] MPR applicability for FR1 intra-band UL non-contiguous CA Way forward: FFS in the next meeting

- Discussion

The MPR applicability for FR1 intra-band contiguous UL CA was discussed in last meeting, and the tentative agreement was put in [] for further checking. The uncertainty at that time is whether UE has enough time to reconfigure itself to meet different MPR requirements when SCC is activated or deactivated which requires the adjustment of Tx chains like PA/RFFE, etc.

According to 38.133, the interruption duration of SCC activation/deactivation is slot based (at ms level) as can be seen in below figure. And UE adjust its RF configurations usually is less than 210us according to the discussion in UL Tx switching. From this perspective, there is no problem in UE reconfiguration to meet different MPR requirements.

Observation 1:   The interruption duration of SCC activation/deactivation is slot based (at ms level) and UE adjust its RF configurations usually is less than 210us. Time seems enough for UE to adjust itself during the SCC activation/deactivation interruptions.