3GPP TSG-RAN WG4 Meeting #111	R4-2408779

Fukuoka, Japan, 20 May – 24 May 2024

Agenda Item:	7.8.7.1

Source:	Ericsson

Title:	UE demodulation requirements for NR less than 5MHz

Document for:	Discussion

# 1	Introduction

RAN4#110bis agreed with the WF on the UE demodulation requirements for NR less than 5MHz [1], and endorsed the big draft CR on UE demodulation requirements for UE supporting NR less than 5MHz [2].

Since the requirements are TBD, we provide our simulation results to finalize the UE demodulation requirements for UE supporting NR less than 5MHz.

# 2	PDCCH demodulation performance

Way forward: - Companies are encouraged to clarify further the requirement value selection and simulation results depending on how punctured CCE is treated.

Table 1 shows the CORESET configurations and Table 2 shows test cases for PDCCH demodulation requirements [1][2].

We should point out the REG bundle size (L) endorsed in [2] was set to 2. However, according to TS38.211 V18.2.0 7.3.2.2, L should be set to 6 for the CORESET 0 configured for NR less than 5MHz (See Appendix). We therefore propose to change to L=6.

Proposal 1: Change REG bundles size to 6 for Table 5.3.2.1.7-1 and Table 5.3.3.1.6-1 in TS38.101-4, according to TS 38.211 7.3.2.2.

Table 1	CORESET configuration for PDCCH decoding requirements.

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| Frequency domain resource allocation for CORESET0 | Start | RB Index | 8 |
|  | Length | RBs | 7 (Note 1) |
|  | Allocation |  | Contiguous |
| CCE to REG mapping type |  |  | nonInterleaved |
| REG bundle size |  |  | 2 => 6 |
| Shift Index |  |  | NA |
| [Note 1: The last RB in CORESET0 (RB#15) is punctured and not transmitted.] |  |  |  |

Table 2	Simulation assumptions for PDCCH with puncturing.

| Test number | Number of Tx | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | DCI payload size (w/o CRC) | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 15 | 3 | 4 | 35 bits | TDLA30-10 | 1x2 Low |
| 2 | 2 | 3 | 15 | 3 | 4 | 35 bits | TDLC300-100 | 2x4 Low |

Table 3 summarizes our simulation results for the punctured PDCCH decoding requirements. Note our simulation result assumes UE demodulates 7 RBs to decode the punctured PDCCH.

Table 3	Summary of PDCCH simulation results

| Test number | SNR (dB) to achieve 1% of Pm-dsg (alignments) | SNR (dB) to achieve 1% of Pm-dsg (with impairments) |
| --- | --- | --- |
| 1 | 3.5 | 5.5 |
| 2 | -3.8 | -1.8 |

# 3	PBCH demodulation performance

RAN4 has already agreed to define the punctured PBCH demodulation requirements with unknown SSB index for 2Rx/4Rx. Table 4 summarize our alignment simulation results and impairment simulation results to define PBCH demodulation requirements.

Table 4	Simulation results for PBCH demodulation with unknown SSB index.

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low | -5.8 | -3.8 |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 4 Low | -8.5 | -6.5 |

# 4	Summary

Proposal 1: Change REG bundles size to 6 for Table 5.3.2.1.7-1 and Table 5.3.3.1.6-1 in TS38.101-4, according to TS 38.211 7.3.2.2.

# References

- R4-2406044, “Way Forward for [110bis][323] NR_FR1_lessthan_5MHz_BW_demod”, Nokia, Nokia Shanghai Bell.

- R4-2405252, “Big draftCR for 38.101-4 on Lessthan5MHz UE demod perf and CSI requirements”, Nokia.

# Appendix

TS38.211 V18.2.0 7.3.2.2

For CORESET 0 on a carrier where the SS/PBCH block is detected at sync raster points defined in Tables 5.4.3.1-2 or 5.4.3.1-3 of [14, TS 38.101-1] and configured by the ControlResourceSetZero IE: - NRBCORESET and NsymbCORESET are defined by Table 13-0 in clause 13 of [5, TS 38.213]; - if NRBCORESET=12 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2; - if NRBCORESET=24 on a carrier with a channel bandwidth of 3 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2 or non-interleaved mapping as defined by clause 13 of [5, TS 38.213], followed by puncturing the 9 highest-numbered resource blocks to obtain the 15 resource blocks forming CORESET 0; - if NRBCORESET=24 on a carrier with a channel bandwidth of 5 MHz, the CORESET is obtained by applying the description above assuming interleaved mapping with R=2, followed by puncturing the 4 highest-numbered resource blocks to obtain the 20 resource blocks forming CORESET 0; - L=6; - nshift=NIDcell; - the UE may assume normal cyclic prefix when CORESET 0 is configured by MIB or SIB1; - the UE may assume the same precoding being used within a REG bundle.

Figure 1	CCE to PDCCH mapping.