# 3GPP TSG- RAN WG4 Meeting # 111	R4-2408799

# Fukuoka, Japan, 20th–24th May, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN phase3

# Agenda item:	10.15.2.1

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on Non-Terrestrial Networks (NTN) for NR Phase 3 with the following RAN4 objective. In this contribution, we want to share some further views from RedCap UE RF perspective.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

# 2 Discussion

  - 2.1 HD-FDD (e)RedCap

- Sub-topic 1-3: HD-FDD (e)RedCap Agreement: Start the discussions on the RF requirements for NTN RedCap with HD-FDD in the next meeting (RAN4#111).

The above is way forward in last meeting [2], we need to start the discussions on the RF requirements for NTN RedCap with HD-FDD in this meeting. However, RAN1 is focusing on HD collision rules to access whether any essential changes are required to support of HD-FDD RedCap UEs and eRedCap UEs in FR1-NTN bands. So, it is better to wait for RAN1 outcome before defining RF requirements for NTN RedCap with HD-FDD.

Proposal 1: It is better to wait for RAN1 outcome before defining RF requirements for NTN RedCap with HD-FDD.

For NTN bands n256 and n255, REFSENS aligns with NR bands n65 and n24 respectively. For NTN band n254, REFSENS is based on band n53 with 0.5dB relaxation due to n254 being an FDD band distinct from TDD band n53, which may suffer from potential performance degradation caused by simultaneous Tx and Rx operations.

Regarding NR HD-FDD RedCap UEs, it was agreed to tighten the 5 MHz REFSENS from FD-FDD as the following table for 5MHz CBW, which is based on the principle of a band with higher duplexer insertion loss having bigger tightening value. The 0.5 dB tightening value is derived from the insertion loss difference between HD-FDD and FD-FDD front-end filter architectures.

In addition, NR HD-FDD REFSENS for channel BW wider than 5 MHz also was defined to be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS, n=2 for 30kHz SCS, and n=4 for 60kHz SCS [3].

Table 2.1: NR HD-FDD 5MHz REFSENS tightening for different NR bands

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

Based on the above, if there is no RAN4 impact, we can use the following tightening value as starting point:

Table 2.2: NTN HD-FDD 5MHz REFSENS tightening for different NR bands

| Bands | HD-FDD REFSENS Tightening |
| --- | --- |
| n256 | [0.5] dB |
| n255 | [0] dB |
| n254 | [0.5] dB |

Proposal 2: After feasibility assessment from RAN1, if there is no RAN4 impact, we can use the tightening value in Table 2.2 as starting point.

  - 2.2 Frequency error

In current specification, e.g. TS 38.101-5, TS 38.101-1, it states that:

- TS 38.101-5 - 6.4.1 Frequency error The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

- TS 38.101-1 - 6.4.1 Frequency error The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B. - 6.4I (Reserved)

For NTN, the small Doppler shift used as a reference for demodulation is only achievable if there is pre-compensation at the transmitter end, since the total doppler variation may be significantly higher. However, no frequency error requirement is defined for (e)RedCap UE. In the objective, it notes that GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE. Hence, it is necessary to define frequency error requirement for (e)RedCap UEs with NR NTN operating in FR1-NTN bands.

Proposal 3: It is necessary to define frequency error requirement for (e)RedCap UEs with NR NTN operating in FR1-NTN bands.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NR NTN phase3 and the proposals are made as follows:

Proposal 1: It is better to wait for RAN1 outcome before defining RF requirements for NTN RedCap with HD-FDD.

Proposal 2: After feasibility assessment from RAN1, if there is no RAN4 impact, we can use the tightening value in Table 2.2 as starting point.

Proposal 3: It is necessary to define frequency error requirement for (e)RedCap UEs with NR NTN operating in FR1-NTN bands.

# 4 Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- R4-2406610, WF on NR_NTN_Ph3_UERF, Qualcomm Inc.

- R4-2206544, WF on the RedCap RF, Ericsson