# 3GPP TSG-RAN WG4 Meeting # 111				R4-2408838

Fukuoka, Japan, 20th  – 24th May, 2024

Agenda item:	9

Source: 	NTT DOCOMO, INC.

Title: 	Rel-18 RAN4 UE feature list for NR_MC_enh

Document for:	Approval

- Introduction

This section provides the RAN4 UE feature list for Rel-18 NR_MC_enh.. The same contribution with additional explanations is submitted in AI for Multi-carrier enhancement according to Chairman’s guidance.

- Proposal

Proposal: At latest, RAN4 will check the progress in RAN2 in Friday morning and update the UE feature list according to the agreement in last meeting.

Agreement: for FG 38-4 and FG 38-5 - RAN4 did not reach consensus on merging FG 38-4 and FG38-5 and there is no corresponding update of the feature list. - Companies are encouraged to further discuss the capability issue in RAN2 - Check RAN2 progress during May meeting. If RAN2 keep the agreement, RAN4 will agree on Option 2 and update the feature list in May meeting.

- NR_MC_enh

Option 1:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-5 | preferredBandPairs for four-band switching case | 1. Support the indication of the switching period can be improved to min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} assuming UE’s preferred (switched-from, switched-to) band pairs for parallel UL transmission switching for a band combination consisting of four different bands. | 38-1 | Yes | N/A | [Network can only assume the maximum switch period] | Per BC | No | FR1 only | Support mixture of FDD/TDD | [Note: Detailed information can refer to the LS to RAN2 in R4-2317609] | Optional with capability signalling |

Option 2:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-4 | Additional switching Period for switching case across three or four bands for Dual UL | 1. Indicate additionally the supported Tx switching period for switching case across three or four bands, when Rel-18 UL Tx switching is configured by uplinkTxSwitchingMoreBands-r18. If the capability is not reported, the switching period reported in switchingPeriodFor2T-r18 or switchingPeriodFor1T-r18 applies, as specified in TS 38.214 and TS 38.101-1. | 38-1 | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination. | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 1 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-5 | Void |  |  |  |  |  |  |  |  |  |  |  |