3GPP TSG-RAN WG4#111	R4-2408840

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	10.1.1.4.1

Source: 	Qualcomm France

Title: 	6Rx REFSENS

Document for:	Approval

- Introduction

Analysis and considerations 6Rx requirements are provided in this contribution.

- WID description and WF in RAN4#110bis

WID [1] includes the following content for 6Rx:

6Rx for handheld and FWA UE

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

General for all the areas

- Specify release independence requirements for the core part in TS 38.307/36.307, if needed and feasible.

WF [2] agreed in RAN4#110bis says the following for REFSENS:

# Topic 1:	REFSENS (delta RIB,6R)

## Sub-topic 1-1:	General considerations for specifying ΔRIB,6R value

Way forward: The following proposals can be endorsed.

-	Proposal 1: Define one delta value for all CBWs for each band.

-	Proposal 2: Consider different requirements for different bands.

Further discuss the following proposals:

-	Proposal 3: Study whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case.

-	Proposal 4: RF isolation at HB and UHB frequencies and antenna placement should be well studied and understood because of their impact on REFSENS for 6Rx handheld UEs.

-	Proposal 5: Existing ΔRIB requirements for 4Rx and 8Rx can be used as a starting point to study the performance of ΔRIB for 6Rx considering the form factor of handheld and FWA UE and other impact factors.

## Sub-topic 1-2:	ΔRIB,6R value for handheld UE and FWA

Way Forward: Further discuss if the same value is used for handheld UE and FWA.

## Sub-topic 1-3:	ΔRIB,6R values for handheld UE and FWA

Way forward: Further discuss the ΔRIB,6R requirements.

- Discussion

Sub-topic 1: REFSENS for 6Rx

4RX and 8RX ΔRIB is specified as follows.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n7 | -4.5 |
| n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

We did average the allowances and kept same delta between 4Rx and 6Rx for both n41, and n77/n79/n104 with the following values which we propose for 6Rx. 8Rx is not currently specified for n104, so said averaging is not straightforward but for now it was kept it in same bucket with n77/n78/n79

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.5 |
| n77, n78, n79, n104 | -3.0 |

In our view, there is no need to have separate numbers for handheld and FWA if the numbers are at reasonable level.

Proposal 1: Use the following ΔRIB,6R for both handheld and FWA

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.5 |
| n77, n78, n79, n104 | -3.0 |

- Conclusion

# REFSENS for 6Rx was discussed, with the following proposal

Proposal 1: Use the following ΔRIB,6R for both handheld and FWA

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.5 |
| n77, n78, n79, n104 | -3.0 |

- Reference

[1] RP-240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei

[2] R4-2406585, “WF on requirements for 6Rx”, AT&T