3GPP TSG-RAN WG4#111	R4-2408859

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	6.18.2

Source: 	Qualcomm France

Title: 	MSD for PC2 CA_n71B BCS4/5

Document for:	Approval

# Introduction

MSD test case for PC2 CA_n71B BCS4/5 is discussed in this contribution.

- Background

PC2 CA_n71B was added into HPUE basket, and revised in RAN#103 to include only one UL CC (n71A):

| NR CA configuration | Uplink NR CA configuration | contact name, company | Contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- |
| CA_n71B | n71A | Bill Shvodian, T-Mobile USA | bill.shvodian@t-mobile.com | Deutsche Telekom, Ericsson, Nokia, Nokia Shainghai Bell | new |

- Discussion

We use the same MSD test point as proposed in our contribution for PC3:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | TBD | FDD |

The analysis results are as follows:

MSD for CA_n71B PC2 not supporting TX Diversity:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | 6.6 | FDD |

MSD for CA_n71B PC2 supporting TX Diversity:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | 9.7 | FDD |

- Conclusion

MSD for PC2 CA_n71B BCS4/5 was discussed with the following proposals:

Proposal 1: MSD for CA_n71B PC2 not supporting TX Diversity:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | 6.6 | FDD |

Proposal 2: MSD for CA_n71B PC2 supporting TX Diversity:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | 9.7 | FDD |

- Reference