3GPP TSG-RAN WG4 Meeting #111	R4-2408957

# Fukuoka, Japan, 20 - 24 May, 2024

Title: 	Simulation results on BS demodulation requirements for further coverage enhancements

Source: 	Huawei, HiSilicon

Agenda item:	7.17.2

Document for:	Discussion

Background

During RAN4#110b meeting, WF [1] on NR_cov_enh2_demod was approved. In this contribution, we provide our initial simulation results about BS demodulation requirements for further coverage enhancements.

Simulation

Table 2-1 Ideal simulation results for PRACH repetition number 2 with 15kHz SCS

| Propagation conditions | Frequency offset | Duplex | SNR (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Burst format A2 |  |  | Burst format B4 |  |  | Burst format C2 |  |  |
|  |  |  | 4 symbols | 5 slots | 10 slots | 1 slot | 5 slots | 10 slots | 6 symbols | 5 slots | 10 slots |
| AWGN | 0 | FDD | -17.3 | - | - | -20.2 | - | - | -17.3 | - | - |
|  |  | TDD | -17.3 | -17.3 | -17.3 | - | -20.3 | -20.2 | -17.3 | -17.3 | -17.4 |
| TDLC300-100 Low | 400 Hz | FDD | -9.2 | - | - | -13.7 | - | - | -9.1 | - | - |
|  |  | TDD | -9.1 | -10.6 | -11.0 | - | -13.3 | -13.8 | -9.2 | -10.6 | -11.0 |

Table 2-2 Ideal simulation results for PRACH repetition number 2 with 30kHz SCS

| Propagation conditions | Frequency offset | Duplex | SNR (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Burst format A2 |  |  | Burst format B4 |  |  | Burst format C2 |  |  |
|  |  |  | 4 symbols | 10 slots | 20 slots | 1 slot | 10 slots | 20 slots | 6 symbols | 10 slots | 20 slots |
| AWGN | 0 | FDD | -17.7 | - | - | -20.4 | - | - | -17.7 | - | - |
|  |  | TDD | -17.7 | -17.6 | -17.6 | -20.4 | -20.5 | -20.5 | -17.7 | -17.8 | -17.7 |
| TDLC300-100 Low | 400 Hz | FDD | -9.5 | - | - | -11.9 | - | - | -9.4 | - | - |
|  |  | TDD | -9.4 | -10.7 | -10.4 | -11.8 | -13.4 | -13.2 | -9.4 | -10.8 | -10.5 |

Table 2-3 Ideal simulation results for PRACH repetition number 2 with 120kHz SCS

| Propagation conditions | Frequency offset | Duplex | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Burst format A2 | Burst format B4 | Burst format C2 |  |
|  |  |  | 4 symbols | 10 slots | 6 symbols | 10 slots |
| AWGN | 0 | TDD | -17.3 | -19.5 | -17.4 | -16.8 |
| TDLA30-300 Low | 4000 Hz | TDD | -9.2 | -11.2 | -9.2 | -8.6 |

Conclusion

In this contribution, we provide our initial simulation results about BS demodulation requirements for further coverage enhancements.

Reference

R4-2406137, WF on NR_cov_enh2_demod, RAN4#110b, China Telecom