3GPP TSG-RAN WG4 Meeting #111	R4-2408974

# Fukuoka, Japan, 20 - 24 May, 2024

Title: 	Simulation results on SAN demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	7.16.8.1

Document for:	Discussion

Background

During RAN4#110b meeting, WF [1] on NR_NTN_enh_SAN_UE_demod was approved. In this contribution, we provide our update simulation results on SAN demodulation requirements for NR NTN enhancements.

Simulation

Above 10GHz bands

PUSCH

Normal PUSCH with CP-OFDM

| Case number | MCS | Antenna configuration | SNR@70% of maximum throughput |
| --- | --- | --- | --- |
| 1 | 2 | 1x1 | -2.0 |
| 2 | 2 | 1x2 | -5.5 |
| 3 | 12 | 1x1 | 6.7 |
| 4 | 12 | 1x2 | 3.4 |

Normal PUSCH with DFT-s-OFDM

| Case number | Antenna configuration | SNR@70% of maximum throughput |
| --- | --- | --- |
| 1 | 1x1 | -1.8 |
| 2 | 1x2 | -5.3 |

PUSCH repetition Type A

| Case number | Antenna configuration | SNR@70% of maximum throughput |
| --- | --- | --- |
| 1 | 1x1 | -7.4 |
| 2 | 1x2 | -10.7 |

PUCCH

PUCCH format 0

| Case number | Number of OFDM symbols | Antenna configuration | SNR@1% of ACK missed detection |
| --- | --- | --- | --- |
| 1 | 1 | 1x1 | 7.4 |
| 2 | 1 | 1x2 | 1.7 |
| 3 | 2 | 1x1 | 4.8 |
| 4 | 2 | 1x2 | -0.8 |

PUCCH format 1

| Case number | Antenna configuration | SNR |  |
| --- | --- | --- | --- |
|  |  | 0.1% of NACK to ACK | 1% of ACK missed detection |
| 1 | 1x1 | -1.9 | -3.6 |
| 2 | 1x2 | -7.3 | -8.7 |

PUCCH format 2 (ACK missed detection)

| Case number | Antenna configuration | SNR@1% of ACK missed detection |
| --- | --- | --- |
| 1 | 1x1 | 2.2 |
| 2 | 1x2 | -2.4 |

PUCCH format 2 (UCI BLER)

| Case number | Antenna configuration | SNR@1% of UCI BLER |
| --- | --- | --- |
| 1 | 1x1 | 2.6 |
| 2 | 1x2 | -3.5 |

PUCCH format 3

| Case number | Antenna configuration | SNR@1% of UCI BLER |  |
| --- | --- | --- | --- |
|  |  | Additional DMRS | No additional DM-RS |
| 1 | 1x1 | 1.5 | 1.6 |
| 2 | 1x2 | -4.3 | -3.9 |

PUCCH format 4

| Case number | Antenna configuration | SNR@1% of UCI BLER |  |
| --- | --- | --- | --- |
|  |  | Additional DMRS | No additional DM-RS |
| 1 | 1x1 | 4.0 | 4.1 |
| 2 | 1x2 | -2.2 | -2.0 |

PRACH

| Case number | Burst format | Antenna configuration | SNR@1% of ACK missed detection |
| --- | --- | --- | --- |
| 1 | B4 | 1x1 | -8.1 |
| 2 | B4 | 1x2 | -13.5 |
| 3 | C2 | 1x1 | -4.8 |
| 4 | C2 | 1x2 | -10.3 |

PUSCH with DMRS bundling

Normal PUSCH with CP-OFDM

| Case number | SCS | Channel bandwidth | PUSCH mapping type | PUSCH aggregation factor | Antenna configuration | SNR@70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | 5MHz | A | n4 | 1x1 | -5.5 |
| 2 | 15kHz | 5MHz | A | n4 | 1x2 | -8.9 |
| 3 | 15kHz | 5MHz | B | n4 | 1x1 | -5.5 |
| 4 | 15kHz | 5MHz | B | n4 | 1x2 | -8.9 |
| 5 | 30kHz | 10MHz | A | n8 | 1x1 | -8.8 |
| 6 | 30kHz | 10MHz | A | n8 | 1x2 | -12.0 |
| 7 | 30kHz | 10MHz | B | n8 | 1x1 | -8.8 |
| 8 | 30kHz | 10MHz | B | n8 | 1x2 | -12.0 |

Conclusion

In this contribution, we provide our updated simulation results on SAN demodulation requirements for NR NTN enhancements.

Reference

R4-2406024, WF on NR_NTN_enh_SAN_UE_demod, RAN4#110b, Huawei, HiSilicon