3GPP TSG-RAN WG4 Meeting # 111	R4-2409004

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title: 	Discussions on for advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:

Document for:	Discussion

Background

In last meeting, a WF [1] on the receiver for MU-MIMO scenario was approved. This contribution provides our views related to open issues.

- Discussions

- Test setup for case with modulation order detection for Rank 1+1:

- There are three candidate options for case with modulation order detection:

- Option 1: Orthogonal precoding, TDLC300-100, ULA low, MCS 13 (Table 1) for Target UE, QPSK for co-UE, full FDRA for the co-UE - Option 2: Case26: Orthogonal precoding, TDLC300-100, ULA medium, MCS 17 (Table 1) for Target UE, 16QAM for co-UE, full FDRA for the co-UE - Option 3: Case 20: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE (as priority for requirement definition)

Based on our simulation results in Table 3-2, option 1 has smallest performance gain among three options.  Both Option 2 and Option 3 have significant performance gain, which are feasible for requirement definition. We slightly prefer Option 3, which has been considered as priority for requirement definition.

- Test setup for case with modulation order detection for Rank 2+2:

- For case with Rank 2+2, we update the simulation results in Table 3-2. We see that the performance gain is quite limited and we don’t think such limited gain is enough to define requirements. Therefore, we propose to not define requirements for Rank 2+2.

- Proposal 1: For case with modulation order detection:

For Rank 1+1: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

Don’t introduce requirements for Rank 2+2

RRC assistant information configuration on the MCS table

We have following options for RRC assistant information configuration on the MCS table

- For UEs not supporting modulation order blind detection: - Option 1: No need for the network to inform such information to the UE - Option 2: Signalled regardless of whether the UE supports MO BD - Option 2A: 256QAM MCS Table - Option 2B: 64QAM MCS table - For UEs supporting modulation order blind detection: - Agreement: - 256QAM MCS Table

Considering that each entry of the RRC assistant signaling is optional, we don’t think MCS table information is needed when DCI modulation order of co-scheduled UE is explicitly signaled.

- Proposal 2: Don’t configure RRC assistant information on the MCS table for case without modulation order detection.

For UE supporting MO BD, whether to introduce applicability rule to skip test(s) with modulation order indicated

We have following candidate options for applicability rules:

- Candidate options: - Option 1: Introduce applicability rule to skip tests with modulation order indicated for UEs capable of BD MO - Option 2: Do not introduce applicable rule skip tests with modulation order indicated

Based on our understanding, R-ML with modulation order detection contains all algorithm procedure of R-ML without modulation detection and additional step with modulation order detection. So all algorithms for R-ML on MU-MIMO has been verified when UE passes the case with modulation order detected and it’s not needed to test case without modulation order detection repeatedly.

Proposal 3: Introduce applicability rule that if UE pass the cases of Rank 1+1 with modulation order detection, cases of Rank 1+1 without modulation order detection is skipped.

- Simulation results

Table 3-1: Simulation results for cases without modulation order detection

| Duplex Mode | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Ideal results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | Rank1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 13.0 | 14.5 |
|  |  | 2T4R |  |  |  |  |  | 12.1 | 13.6 |
|  | Rank2+2 | 4T4R |  | TDLA30-10 | ULA Low | 17 | 16QAM | 15.1 | 16.6 |
|  | Rank1+1 | 2T2R |  | TDLC300-100 | ULA medium | 13 | QPSK | 13.4 | 14.9 |
| TDD |  | 2T4R |  |  |  |  |  | 12.5 | 14.0 |
|  | Rank2+2 | 4T4R |  | TDLA30-10 | ULA Low | 17 | 16QAM | 15.4 | 16.9 |

Table 3-2: Simulation results for cases with modulation order detection

| Duplex Mode | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) | MMSE-IRC SNR@70% maxTP(dB) | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | Rank1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA Low | 13 | QPSK | 10.1 | 12.5 | 2.4 |
|  |  | 2T2R |  |  | ULA medium | 17 | 16QAM | 20.9 | 26.1 | 5.2 |
|  |  | 2T2R |  |  | ULA medium | 13 | QPSK | 13.6 | 19.4 | 5.8 |
|  |  | 2T4R |  |  | ULA Low | 13 | QPSK | 5.8 | 6.5 | 0.7 |
|  |  | 2T4R |  |  | ULA medium | 17 | 16QAM | 20.9 | 27.9 | 7.0 |
|  |  | 2T4R |  |  | ULA medium | 13 | QPSK | 13.5 | 20.5 | 7.0 |
| TDD |  | 2T2R |  |  | ULA Low | 13 | QPSK | 10.6 | 13.0 | 2.4 |
|  |  | 2T2R |  |  | ULA medium | 17 | 16QAM | 22.1 | 28.8 | 6.7 |
|  |  | 2T2R |  |  | ULA medium | 13 | QPSK | 14.1 | 20.4 | 6.3 |
|  |  | 2T4R |  |  | ULA Low | 13 | QPSK | 6.4 | 7.1 | 0.7 |
|  |  | 2T4R |  |  | ULA medium | 17 | 16QAM | 22.5 | 32.4 | 9.9 |
|  |  | 2T4R |  |  | ULA medium | 13 | QPSK | 14.3 | 22.3 | 8.0 |
| FDD | Rank2+2 | 4T4R |  | TDLA30-10 | ULA Low | 13 | QPSK | 10.6 | 12.1 | 1.5 |
|  |  | 4T4R |  |  | XP medium | 13 | QPSK | 12.6 | 13.9 | 1.3 |
| TDD |  | 4T4R |  |  | ULA Low | 13 | QPSK | 10.6 | 12.3 | 1.7 |
|  |  | 4T4R |  |  | XP medium | 13 | QPSK | 12.7 | 14.1 | 1.4 |

- Conclusion

- In this contribution we provide our views on advanced receiver for MU-MIMO. The proposals are:

- Proposal 1: For case with modulation order detection:

For Rank 1+1: Orthogonal precoding, TDLC300-100, ULA medium, MCS 13 (Table 1) for Target UE, QPSK for co-UE. full FDRA for the co-UE

Don’t introduce requirements for Rank 2+2

- Proposal 2: Don’t configure RRC assistant information on the MCS table for case without modulation order detection.

Proposal 3: Introduce applicability rule that if UE pass the cases of Rank 1+1 with modulation order detection, cases of Rank 1+1 without modulation order detection is skipped.

- Reference

[1]    R4-2406114 WF on the advanced receiver for MU-MIMO scenario. China Telcom.