3GPP TSG-RAN WG4 Meeting # 110bis	R4-2409005

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title: 	Discussions on for advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	7.11.2.2

Document for:	Discussion

Background

In last meeting, a WF [1] on the receiver for MU-MIMO scenario was approved. This contribution provides our views related to simulation results

- Simulation results

The simulation results are captured in Table 2-2 and 2-3 with simulation assumptions in Table 2-1

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD, TDD |  |
| SCS |  | kHz | FDD:15 TDD:30 |  |
| Bandwidth |  | MHz | FDD:10 TDD:40 |  |
| TDD pattern |  |  | 7D1S2U, S=6D4G4U |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | Rank1+1: 1000 Rank2+2:1000, 1001 | Rank1+1:1001 Rank 2+2:1000,1001 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | Rank1+1: 1 Rank2+2: 2 | Rank1+1: 1 Rank2+2: 2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Orthogonal PMI selection: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH |  |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 in 38.101-4 |  |
| Receiver |  |  | R-ML with modulation order detection R-ML without modulation order detection MMSE-IRC |  |
| Modulation order estimation assumption |  |  | Per PRG detection with range from QPSK to 256QAM |  |
| Number of HARQ Processes |  |  | FDD:4 TDD:8 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

Table 2-2: Simulation results for cases without modulation order detection

| Duplex Mode | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Ideal results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | Rank1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 13.0 | 14.5 |
|  |  | 2T4R |  |  |  |  |  | 12.1 | 13.6 |
|  | Rank2+2 | 4T4R |  | TDLA30-10 | ULA Low | 17 | 16QAM | 15.1 | 16.6 |
|  | Rank1+1 | 2T2R |  | TDLC300-100 | ULA medium | 13 | QPSK | 13.4 | 14.9 |
| TDD |  | 2T4R |  |  |  |  |  | 12.5 | 14.0 |
|  | Rank2+2 | 4T4R |  | TDLA30-10 | ULA Low | 17 | 16QAM | 15.4 | 16.9 |

Table 2-3: Simulation results for cases with modulation order detection

| Duplex Mode | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) | MMSE-IRC SNR@70% maxTP(dB) | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | Rank1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA Low | 13 | QPSK | 10.1 | 12.5 | 2.4 |
|  |  | 2T2R |  |  | ULA medium | 17 | 16QAM | 20.9 | 26.1 | 5.2 |
|  |  | 2T2R |  |  | ULA medium | 13 | QPSK | 13.6 | 19.4 | 5.8 |
|  |  | 2T4R |  |  | ULA Low | 13 | QPSK | 5.8 | 6.5 | 0.7 |
|  |  | 2T4R |  |  | ULA medium | 17 | 16QAM | 20.9 | 27.9 | 7.0 |
|  |  | 2T4R |  |  | ULA medium | 13 | QPSK | 13.5 | 20.5 | 7.0 |
| TDD |  | 2T2R |  |  | ULA Low | 13 | QPSK | 10.6 | 13.0 | 2.4 |
|  |  | 2T2R |  |  | ULA medium | 17 | 16QAM | 22.1 | 28.8 | 6.7 |
|  |  | 2T2R |  |  | ULA medium | 13 | QPSK | 14.1 | 20.4 | 6.3 |
|  |  | 2T4R |  |  | ULA Low | 13 | QPSK | 6.4 | 7.1 | 0.7 |
|  |  | 2T4R |  |  | ULA medium | 17 | 16QAM | 22.5 | 32.4 | 9.9 |
|  |  | 2T4R |  |  | ULA medium | 13 | QPSK | 14.3 | 22.3 | 8.0 |
| FDD | Rank2+2 | 4T4R |  | TDLA30-10 | ULA Low | 13 | QPSK | 10.6 | 12.1 | 1.5 |
|  |  | 4T4R |  |  | XP medium | 13 | QPSK | 12.6 | 13.9 | 1.3 |
| TDD |  | 4T4R |  |  | ULA Low | 13 | QPSK | 10.6 | 12.3 | 1.7 |
|  |  | 4T4R |  |  | XP medium | 13 | QPSK | 12.7 | 14.1 | 1.4 |

- Conclusion

- In this contribution we provide our simulation results on advanced receiver for MU-MIMO.

- Reference

[1]    R4-2406114 WF on the advanced receiver for MU-MIMO scenario. China Telcom.