3GPP TSG-RAN WG4 Meeting # 111	R4-2409013

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title: 	Discussions on Rel-18 eDSS performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.25.1

Document for:	Discussion

Background

In last meeting, a WF on enhancement of DSS demodulation requirements was approved. There is no open issues left and companies are welcome to update results if any, in the contribution we provide our simulation and views on test applicability rules.

- Simulation results

The simulation results are captured in Table 2-1 with simulation assumptions captured in Appendix.

Table 2-1: PDCCH simulation results for eDSS

| Test number | Duplex | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | IdealSNR (dB) | Impairment SNR (dB) |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2X4 Low | 1 | -3.0 | -1.5 |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4X2 Low | 1 | -3.0 | -1.5 |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 4x4 Low | 1 | -3.5 | -2.0 |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 2x2 Low | 1 | -1.3 | 0.2 |

Based on our simulation results, the target SNR for all cases operates in reasonable range. We propose to confirm test cases in Table 2-1 for requirement definition.

Proposal 1: Confirm test cases in Table 2-1 for requirements definition.

- Conclusion

In this contribution we provide our simulation results for Rel-18 eDSS. The proposal is:

Proposal 1: Confirm test cases in Table 2-1 for requirements definition.

- Reference

[1]    R1-2406019 Way forward on [110][328] NR_DSS_enh. Ericsson

- Appendix

Table 5-1: Simulation assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |