3GPP TSG-RAN WG4 Meeting # 111                                R4-2409030

Fukuoka City, Fukuoka, Japan, 20th-24th May, 2024

Agenda item:	7.21.1

Source: 	ZTE Corporation, Sanechips

Title:           Discussion on impacts to RRM core requirements for Enhanced RedCap

Document for:	Approval

Introduction

In RAN4#109 meeting, RRM core part of this WI was closed. This contribution provides some analysis about how to define the measurements on higher priority carriers on maintenance in core part.

Discussion

In the last meeting, RAN4 has discussed enhanced support for reduced capability devices and has achieved some agreements in WF [1].

Measurements on higher priority carriers when configured with eDRX in IDLE mode When configured with eDRX_IDLE, the UE shall search every layer of higher priority at least every Thigher_priority_search = max(60, 15*eDRX_IDLE cycle length) * Nlayers seconds when eDRX_IDLE cycle <=10.24s, else every Thigher_priority_search = max(60, ceil (15/NDRX-PTW)*eDRX_IDLE cycle length) * Nlayers seconds if eDRX_IDLE is configured with PTW, where NDRX-PTW is the number of DRX cycles within a single PTW.

This issue is discussed last meeting and there is no consensus when configured with eDRX_IDLE in IDLE/INACTIVE mode.

It was agreed already in LTE during increased carrier monitoring discussions that a UE configured with eDRX measures on the higher priority carriers at least every 60* Nlayers. Later when extended DRX support was introduced for LTE, this requirement was updated to Thigher_priority_search = MAX(60 * Nlayers, one eDRX_IDLE cycle). In Clause 4.2.2 of TS 36.133, the requirements on higher priority search are specified.

4.2.2 Requirements The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds when the UE is not configured with eDRX_IDLE cycle, and at least every Thigher_priority_search = MAX(60 * Nlayers, one eDRX_IDLE cycle) when UE is configured with eDRX_IDLE cycle, where Nlayers is the total number of configured higher priority E-UTRA, UTRA FDD, UTRA TDD, CDMA2000 1x, HRPD and NR carrier frequencies and is additionally increased by one if one or more groups of GSM frequencies is configured as a higher priority.

In our understanding, the requirements on higher priority carriers between NR and LTE should be aligned. So, we perfer to define requirements that Nlayer is inside. However, due to the strong insistence of other companies, we can compromise, but the delay cannot be too long.

Thigher_priority_search is the time interval at which each high priority carrier is searched. The larger the value of Thigher_priority_search, the slower the search speed, and the more energy-efficient the UE is. Therefore, expanding the value of Thigher_priority_search is beneficial in terms of energy consumption. However, if the Thigher_priority_search value is set too high, it means that it takes longer to complete the search, which may have an impact on network performance.

For the discussion in last meeting, the tentative agreement divides the requirements into two cases based on the length of eDRX_cycle_length (PTW is configured or not). When eDRX_IDLE cycle <=10.24s, Thigher_priority_search = max(60, 15*eDRX_IDLE cycle length) * Nlayers seconds. Therefore, the max value of Thigher_priority_search is 153.6*Nlayer when eDRX_IDLE cycle length is 10.24s, which is acceptable to us.

However, when eDRX_IDLE cycle >10.24s, Thigher_priority_search = max(60, ceil (15/NDRX-PTW)*eDRX_IDLE cycle length) * Nlayers seconds. In this equation, the length of eDRX_IDLE_cycle is up to 10485.76 seconds, the delay is too long in some cases if increase eDRX_INACTIVE cycle by Nlayers times.

Observation 1: The delay in some cases is too long if Thigher_priority_search = max(60, ceil (15/NDRX-PTW)*eDRX_IDLE cycle length) * Nlayers seconds when eDRX_IDLE cycle >10.24s.

If we reach consensus on how to define the measurements on higher priority carriers in idle mode, the same conclusions can also be applied to INACTIVE mode as well by using INACTIVE mode eDRX cycle instead of the IDLE mode eDRX cycle.

Proposal 1: For INACTIVE mode, the same search rate as IDLE mode applies with eDRX_INACTIVE cycle length instead of eDRX_IDLE cycle length.

Conclusions

In this contribution, we provide our views on the RRM impacts for enhanced Redcap UE and the proposals are:

Observation 1: The delay in some cases is too long if Thigher_priority_search = max(60, ceil (15/NDRX-PTW)*eDRX_IDLE cycle length) * Nlayers seconds when eDRX_IDLE cycle >10.24s.

Proposal 1: For INACTIVE mode, the same search rate as IDLE mode applies with eDRX_INACTIVE cycle length instead of eDRX_IDLE cycle length.

References

- R4-2406425, WF on RRM requirements for NR_redcap_enh, Ericsson.